3GPP TSG-RAN WG4 Meeting #97e	R4-2015850

Electronic Meeting, 2nd – 13th November, 2020

Source:	Ericsson

Title:	Simulation results for Rel-16 NR HST PUSCH demodulation performance under fading condition

Agenda item:	7.15.3.2.1

Document for:	Discussion

# Introduction

In this contribution, we presented our simulation results for NR HST PUSCH demodulation performance under fading condition.

# Simulation assumptions and simulation results

In this section, we presented our simulation assumptions based on Table 1 for HST fading scenarios. The corresponding ideal simulation results and results with impairments for HST scenarios are given in Table 2 to  Table 3.

| Parameters | Values |
| --- | --- |
|  | FR1 |
| Waveform | CP-OFDM |
| Number of Tx and Rx | 1Tx2Rx |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 DM-RS symbols with l0 = 2 |
| Number of symbols | 14 |
| Time domain resource allocation type | type A |
| Frequency domain resource | full RB allocation of the applicable BW |
| MCS index | 2 |
| Carrier frequency (fc) | SCS = 15 kHz: fc = 2.1 GHz SCS = 30 kHz: fc = 3.6 GHz |
| Propagation channel | SCS = 15 kHz: TDLC300-600, f0 = 0 SCS = 30 kHz: TDLC300-1200, f0 = 0 |
| SCS and BW | 15kHz: 5 MHz 30kHz: 10 MHz |
| Testing metric | SNR @70% of maximum throughput |

Table 1. Simulation assumptions for HST under fading condition [1].

Table 2. Ideal simulation results for NR HST PUSCH demodulation performance under fading condition.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Fading channel | 15 kHz / 5 MHz | 2 | -4.1 |
|  | 30 kHz / 10 MHz | 2 | -3.9 |

Table 3. Results with impairments for NR HST PUSCH demodulation performance under fading condition.

| Scenario | SCS / BW | MCS | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Fading channel | 15 kHz / 5 MHz | 2 | -2.0 |
|  | 30 kHz / 10 MHz | 2 | -1.6 |

# Conclusion

In this contribution. we summarized our simulation results for {SCS. BW} combinations = {15kHz. 5MHz} and {30kHz. 10 MHz} for NR HST PUSCH demodulation performance under fading condition.

# References

R4-2012676, “WF on Rel-16 NR HST PUSCH BS demodulation requirements”, RAN4#96-e, Nokia, Nokia Shanghai Bell, …