3GPP TSG-RAN WG4 Meeting #97-e	R4-2015931

Electronic Meeting, 02 November – 13 November 2020

Source:	Ericsson

Title:	TP for TR 37.717-21-11: Including band combinations DC_7-40_n78

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-21-11 to add DC_7A-40A_n78A and DC_7A-40C_n78A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_7-40_n78

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-40_n78 | CA_7-40 | n78 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-40A_n78A DC_7A-40C_n78A | DC_7A_n78A | CA_7A-40A CA_7A-40C | n78A |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_7-40_n78, the TIB,c and RIB,c values are reused from CA_n40-n41-n79 and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-40_n78 | 7 | 0.5 |
|  | 40 | 0.5 |
|  | n78 | 0.8 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-40_n78 | 7 | 0.5 |
|  | 40 | 0 |
|  | n78 | 0.5 |

### 5.1.x.4	REFSENS requirements

There is IMD4 impact from DC_7_n78 UL affecting band 40 DL. MSD derived from CA_n1A-n7A-n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-40A_n78A DC_7A-40C_n78A | 7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
|  | n40 | 2340 | 5 | 25 | 2360 | 8.7 | IMD4 |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-202060, “WID revision: Rel-17 Dual Connectivity of 2 bands LTE inter-band CA and 1 NR band” Huawei, HiSilicon