3GPP TSG-RAN4 Meeting #97-e	R4-2016896

E-meeting, 2  – 18 November, 2020

Source: 	Ligado Networks, Skyworks Solutions Inc.

Title: 	WF on work items LTE_B24_mod and NR_band_n24

Document for: 	Approval

# A-MPR Simulation Agreements

Agreement #1: Perform separate A-MPR evaluations for LTE and NR

Agreement #2: A-MPR Assumptions to be used:

0 dBi UE antenna gain

4 dB post-PA losses

PA calibrated at 1 dB MPR for -30 dBc EUTRA/NR ACLR with 20 MHz DFT-s-OFDM 100 RB0 waveform

Image/carrier leakage = -25 dBc for LTE and-28 dBc for NR

-60 dBc CIM3 and -70 dBc CIM5

Filter attenuation to be used:

Tx attenuation in 1541 – 1612 MHz = 41.0 dB

Tx attenuation in 1612 – 1617 MHz = dB linear interpolation from 41.0 dB at 1612 MHz to 12.5 dB at 1617 MHz

Tx attenuation in 1617 – 1622 MHz = dB linear interpolation from 12.5 dB at 1617 MHz to 0 dB at 1622 MHz

Tx attenuation in 1622 – 1627.5 MHz = 0 dB

Agreement #3: Following sceanrios will be considered for A-MPR simulations:

- 5 MHz carrier from 1627.5 – 1632.5 MHz

- 5 MHz carrier from 1632.5 – 1637.5 MHz

- 5 MHz carrier from 1646.5 – 1651.5 MHz

- 5 MHz carrier from 1651.5 – 1656.5 MHz

- 10 MHz carrier from 1627.5 – 1637.5 MHz

- 10 MHz carrier from 1646.5 – 1656.5 MHz

Agreement #4: Companies are encouraged to bring A-MPR simulations with the agreed upon A-MPR assumptions and scenarios to the RAN4#98e meeting.

# Additional emission requirements for Band 24 and n24 in TS 36.101 and TS 38.101-1 respectively

Agreement #5: Include the the table below in section 6.6.3.3 of TS 36.101 and section 6.5.3.3 of TS 38.101-1

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit in EIRP1 (dBm) | Measurement bandwidth | Note |
| --- | --- | --- | --- |
|  | 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | EIRP of discrete emissions of less than 700 Hz bandwidth, averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | EIRP and averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement is converted to conducted requirement depending on the supported antenna gain Gant declared by the UE manufacturer. If not declared by the UE manufacturer, a 0 dBi antenna gain shall be used. |  |  |  |

Table: Additional Emission Requirements

# UE REFSENS for Band 24 and n24

Agreement #6:

- No updates are necessary to the Band 24 UE REFSENS

- Existing Band 24 UE REFSENS can be re-used for NR with the proper TXBW scaling

# References

[1] R4-2016972, Email discussion summary for [97e][131] NR_LTE_band_n24

# Appendix – Average Filter Attenuation (R4-2014161)

| Freq. range (MHz) | ETC average value Filter Attenuation (dB) |
| --- | --- |
| 1541-1590 | -45.8 |
| 1590-1608 | -42.8 |
| 1608 | -43.3 |
| 1609 | -44.0 |
| 1610 | -44.2 |
| 1611 | -44.5 |
| 1612 | -43.1 |
| 1613 | -39.8 |
| 1614 | -32.6 |
| 1615 | -25.8 |
| 1616 | -19.8 |
| 1617 | -15.6 |
| 1618 | -13.2 |
| 1619 | -9.5 |
| 1620 | -6.7 |
| 1621 | -4.5 |
| 1622 | -2.9 |
| 1623 | -2.8 |
| 1624 | -2.5 |
| 1625 | -2.4 |
| 1626 | -2.3 |
| 1627 | -2.3 |
| Note: The attenuation levels are absolute and not in addition to the insertion loss value |  |