3GPP TSG-RAN WG4 Meeting # 97-e                                  R4-2017829

Electronic Meeting, 2nd – 13th November, 2020

Source: 	Huawei, HiSilicon

Title: 	TP for TR 37.875: adding some UE RF study for NR V2X band combinations

Agenda Item:	10.23.1

Document for:	Approval

Introduction

A general discussion paper [1] was provided in last meeting to discuss the band n47/47 filter performance and UE architecture. In this paper, we provide our proposals into the general part of this TR.

- References

[1]	R4-2010931, “General discussion about Rel-17 band combinations for Uu and V2X con-current operation”, Huawei, HiSilicon

- Text Proposal

<TP for TR 37.875>

- 5	Additional UE RF: General part

- 5.1	UE RF aspects

5.1.1	Basic UE RF architecture and assumed performance with ITS band 47/n47

Based on the investigation, the band 47/n47 filter performance are shown in table 5.1.1-1.

Table 5.1.1-1 Filter performance for band n47/47

| Filter | IL [dB] | Min Attenuation [dB] @ |  |  |
| --- | --- | --- | --- | --- |
|  | Worst Case | 410-2690 MHz | 3300-4200 MHz | 4400-5000 MHz |
| n47/47 (5855-5925 MHz) | [2] | > [35] | > [32] | > [30] |

It’s assumed that the antenna isolation between band n47 and Uu licensed bands which is below 5GHz is about [15]dB. An example RF architecture for NR V2X band combinations with ITS band 47/n47 is shown in figure 5.1.1-1. It’s assumed that separate antennas is used for NR V2X band combination with ITS band 47/n47. Since separate antennas are assumed, there is no need to specify ΔTIB,c and ΔRIB,c for band 47/n47.

Figure 5.1.1-1 NR V2X band combinations RF architecture with separate antennas

- 5.2	RRM aspects

Editor Note: It will be added in the future [FFS]

<End of TP >