3GPP TSG-RAN4 Meeting #98e	R4-2100214

# Electronic Meeting, 25 Jan - 5 Feb, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1459 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on UL spatial relation switch requirements (R16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Detailed description in R4-2100213 Spatial relation info for PUCCH also includes PL-RS. For PUCCH a spatial relation info switch would also change the PL-RS. The known definition and delay requirements for PUCCH spatial relation info switch should also include PL-RS. Agreements in RAN4#97e to define requirement for unknown case are not captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated known condition for PUCCH Spatial relation switch Updated MAC CE based switching delay for PUCCH Spatial relation switch and unknown case Updated RRC based switching delay for unknown case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Core requirements for UL spatial relation switch are incomplete and incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.2 8.12.3 8.12.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 8.12	Uplink spatial relation switch delay

### 8.12.1	Introduction

The requirements in this clause apply for a UE configured with one or more spatial relation configurations on serving cell in MR-DC or standalone NR. There is no requirement when the UE is requested to switch to a spatial relation with the higher layer parameter spatialRelationInfo associated to SRS. UE shall complete the switch of active spatial relation within the delay defined in this clause when the UE is requested to switch to a spatial relation with the higher layer parameter spatialRelationInfo associated to a DL RS.

### 8.12.2	Known conditions for spatial relation when associated with DL-RS

The spatial relation associated to DL RS is known if the following conditions are met:

-	During the period from the last transmission of the DL RS resource used for the L1-RSRP measurement reporting for the target spatial relation to the completion of active spatial relation, where the DL RS resource for L1-RSRP measurement is the DL RS in target spatial relation or QCLed to the target spatial relation with QCL type-D.

-	Spatial relation switch command is received within 1280 ms upon the last transmission of the DL RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target spatial relation before the spatial relation switch command

-	The DL RS configured in spatial relation remains detectable during the spatial relation switching period

-	SNR of the DL RS configured in spatial relation ≥ -3dB

-	The SSB associated with the spatial relation remain detectable during the spatial relation switching period

-	SNR of the SSB associated with the spatial relation ≥ -3dB

Otherwise, the spatial relation associated with DL-RS is unknown.

The spatial relation for SRS is known or unknown based on spatial relation of associated DL-RS.

If the pathloss reference RS and DL-RS for PUCCH spatial relation info (PUCCH-SpatialRelationInfo) are QCLed (with Type D), spatial relation for PUCCH is known or unknown based on spatial relation of associated DL-RS.

If the pathloss reference RS and DL-RS for PUCCH spatial relation info (PUCCH-SpatialRelationInfo) are not QCLed (with Type D) the spatial relation for PUCCH is known if the associated with DL-RS is known and the associated pathloss reference RS is known. The known condition for pathloss reference RS is defined in clause 8.14.2. If either the associated pathloss RS or DL-RS is unknown, the spatial relation for PUCCH is unknown.

### 8.12.3	MAC-CE based spatial relation switch delay

If the target spatial relation for SRS is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, for UL spatial relation switch for semi-persistent SRS transmission of serving cell with a target UL spatial relation, the UE shall be able to transmit semi-persistent SRS with the target UL spatial relation in the slot n+ THARQ + 3Nslotsubframe,µ+ 1 when beamCorrespondenceWithoutUL-BeamSweeping is set to 1 where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

If the target spatial relation for PUCCH is known, the associated DL-RS and pathloss RS are QCLed, upon receiving PDSCH carrying MAC-CE activation command in slot n, for UL spatial relation switch for PUCCH transmission of serving cell with a target UL spatial relation, the UE shall be able to transmit PUCCH with the target UL spatial relation in the slot n+ THARQ + 3Nslotsubframe,µ+NM*3ms + 5*Ttarget_PL-RS + 2msNR slot length+ 1 when beamCorrespondenceWithoutUL-BeamSweeping is set to 1.

Where

-   NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS.

If the target spatial relation for SRS is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, for UL spatial relation switch for semi-persistent SRS transmission of serving cell with a target UL spatial relation, the UE shall be able to transmit semi-persistent SRS with the target UL spatial relation in the slot n+ THARQ + 3Nslotsubframe,µ+ TL1-RSRP NR slot length + 1 when beamCorrespondenceWithoutUL-BeamSweeping is set to 1.

Where

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

If the target spatial relation for PUCCH is unknown, the associated DL-RS and pathloss RS are QCLed, upon receiving PDSCH carrying MAC-CE activation command in slot n, for UL spatial relation switch for PUCCH transmission of serving cell with a target UL spatial relation, the UE shall be able to transmit PUCCH with the target UL spatial relation in the slot n+ THARQ + 3Nslotsubframe,µ+ TL1-RSRP + 5*Ttarget_PL-RS + 2msNR slot length+ 1 when beamCorrespondenceWithoutUL-BeamSweeping is set to 1.

TBD for requirements for PUCCH spatial relation switch if associated DL-RS and pathloss RS are not QCLed.

### 8.12.4	DCI based spatial relation switch delay

If the target spatial relation associated to DL RS is known, when a UE receives the DCI triggering aperiodic SRS at slot n with the higher layer parameter spatialRelationInfo, UE shall be able to transmit aperiodic SRS with target spatial relation of the serving cell on which spatial relation switch occurs in the slot+1, where, k is configured via higher layer parameter slotOffset[2] for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively in TS 38.214 [26].

The known condition for spatial relation associated to DL RS defined in clause 8.12.2 is applied.

### 8.12.5	RRC based spatial relation switch delay

If the target spatial relation associated to DL RS is known, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to transmit target periodic SRS with spatial relation of the serving cell on which periodic SRS with spatial relation reconfigured in the slot n+ TRRC_processing /NR slot length +1 when beamCorrespondenceWithoutUL-BeamSweeping is set to 1 where TRRC_processing is the RRC processing delay defined in TS38.331 [2].

If the target spatial relation associated to DL RS is unknown, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to transmit target periodic SRS with spatial relation of the serving cell on which periodic SRS with spatial relation reconfigured in the slot n+  TL1-RSRP + 5*TRRC_processing NR slot length+1 when beamCorrespondenceWithoutUL-BeamSweeping is set to 1. Where T L1-RSRP is defined in clause 8.12.3.

End of Change 1