3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100216

# Electronic Meeting, 25 Jan – 5 Feb, 2021

Agenda item:	7.16.1.2

Source:	Apple Inc.

Title:	PMI reporting requirements with larger number of TX ports

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#97e the way forward [1] for PMI reporting requirements with Type II codebook was agreed. In this contribution we present simulation results for PMI reporting with Type II codebook.

# 2. Discussion

In RAN4#97e, the following agreements were made for PMI reporting with Type II codebook.

- Introduce Rel-15 Type II codebook requirements with SU-MIMO set-up under the condition that with proper test parameters, test metric/ test requirements and test procedure to ensure enough performance difference over than Type I i.e. UE which employ Type I reporting will fail in the test case - Test metric: - Following PMI (Type II)/Random PMI (Type I codebook) (gamma values) based on the assumption that there are ensure enough performance difference over than Type I i.e. UE which employ Type I reporting will fail in the test case - This test metric applied to UE which support Type II codebook feedback irrespective whether supporting Type I codebook feedback or not - FFS: Whether to check UE reported codebook not only within Type I codebook set - Further study and define proper performance requirements if needed under MU-MIMO scenario in Rel-17 performance enhancement WI.

The following simulation assumptions agreed in RAN4#97e.

- Number of CSI-RS ports:

  - 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4)

- L (numberOfBeams)

  - 2

- Npsk (phaseAlphabetSize)

  - 8

- SubbandAmplitude

  - True

- PMI-FormatIndicator

  - Subband

- Sub-band Size:

  - 4 for FDD with 15kHz SCS, 10MHz CBW

  - 8 for TDD with 30kHz SCS, 40MHz CBW

- Channel Model

  - TDLA30-5

- MIMO Correlation

  - XP Medium (as baseline)

- MCS and Rank

  - MCS 20 (64QAM Table), Rank 2

- Beam-Steering Model

  - How to specify beam steering model into specification.

    - Same as specified in B.2.3B.4A of TS 36.101

  - Enable two independent beam directions applying the beam for comparison both eType II and Type I cookbook simulation

For the simulations assumptions agreed in [2], the PMI reporting results are provided for Type II and Type I codebook are provided in Table 1. We provide performance results using 90% max TP as the test point.

Table 1: PMI Reporting Results for Type II and Type I codebook

| TDLA-30-5Hz XP-Medium |  | Type II |  | Type I |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR @90% Max TP | TP Gain | SNR @90% Max TP | TP Gain |
| FDD | 16x2 | 10.56 | 2.92 | 13.77 | 2.06 |
|  | 16x4 | 7.06 | 2.83 | 9.11 | 2.26 |
| TDD | 16x2 | 11.25 | 2.82 | 14.40 | 2.02 |
|  | 16x4 | 7.15 | 2.81 | 8.94 | 2.30 |

Comparing Type II and Type I the performance, at 90% max TP, we have 3 dB gain in performance and considerable difference in TP gain. Using a lower test point like 80% or 70% max TP would result in larger delta between Type II and Type I results. Based on the results in Table 1, we propose to use 90% max TP as the test point for PMI reporting with Type II codebook.

Proposal #1: Define PMI reporting requirements with Type II codebook at 90% Max TP.

# 3. Conclusion

In this paper, we simulation results for PMI reporting requirements for Type II codebook. Our proposals are summarized below:

Proposal #1: Define PMI reporting requirements with Type II codebook at 90% Max TP.

# Reference

- R4-2017681, “Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, Ericsson, Samsung.

- R4-2017563, “Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports”, Ericsson