3GPP TSG-WG RAN4 Meeting #98-e	R4-2100254

# Electronic meeting, 25th January – 05th February, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5714 | rev | - | Current version: | 10.28.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 36.101 related to Band 24 regulatory updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks, Nokia, Skyworks Solutions Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-Core |  |  |  |  |  | Date: |  |  | 2021-1-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Regulatory requirements for Band 24 were updated in April, 2020 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant clauses are updated for Band 24 regulatory updates |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 24 requirements in the specification will not comply with the updated regulations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 5.7.4, 6.2.4, 6.6.3.3, Annex G2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: since there is no Band 24 product, proposed changes are from Rel’10. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.5	Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1.

Table 5.5-1 E-UTRA operating bands

| E-UTRA Operating Band |  | Uplink (UL) operating band BS receive UE transmit |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |  |
| 1 |  | 1920 MHz | – | 1980 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
| 2 |  | 1850 MHz | – | 1910 MHz |  | 1930 MHz | – | 1990 MHz |  | FDD |  |
| 3 |  | 1710 MHz | – | 1785 MHz |  | 1805 MHz | – | 1880 MHz |  | FDD |  |
| 4 |  | 1710 MHz | – | 1755 MHz |  | 2110 MHz | – | 2155 MHz |  | FDD |  |
| 5 |  | 824 MHz | – | 849 MHz |  | 869 MHz | – | 894MHz |  | FDD |  |
| 61 |  | 830 MHz | – | 840 MHz |  | 875 MHz | – | 885 MHz |  | FDD |  |
| 7 |  | 2500 MHz | – | 2570 MHz |  | 2620 MHz | – | 2690 MHz |  | FDD |  |
| 8 |  | 880 MHz | – | 915 MHz |  | 925 MHz | – | 960 MHz |  | FDD |  |
| 9 |  | 1749.9 MHz | – | 1784.9 MHz |  | 1844.9 MHz | – | 1879.9 MHz |  | FDD |  |
| 10 |  | 1710 MHz | – | 1770 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
| 11 |  | 1427.9 MHz | – | 1447.9 MHz |  | 1475.9 MHz | – | 1495.9 MHz |  | FDD |  |
| 12 |  | 699 MHz | – | 716 MHz |  | 729 MHz | – | 746 MHz |  | FDD |  |
| 13 |  | 777 MHz | – | 787 MHz |  | 746 MHz | – | 756 MHz |  | FDD |  |
| 14 |  | 788 MHz | – | 798 MHz |  | 758 MHz | – | 768 MHz |  | FDD |  |
| 15 |  | Reserved |  |  |  | Reserved |  |  |  | FDD |  |
| 16 |  | Reserved |  |  |  | Reserved |  |  |  | FDD |  |
| 17 |  | 704 MHz | – | 716 MHz |  | 734 MHz | – | 746 MHz |  | FDD |  |
| 18 |  | 815 MHz | – | 830 MHz |  | 860 MHz | – | 875 MHz |  | FDD |  |
| 19 |  | 830 MHz | – | 845 MHz |  | 875 MHz | – | 890 MHz |  | FDD |  |
| 20 |  | 832 MHz | – | 862 MHz |  | 791 MHz | – | 821 MHz |  | FDD |  |
| 21 |  | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
| 22 |  | 3410 MHz | – | 3490 MHz |  | 3510 MHz | – | 3590 MHz |  | FDD |  |
| 23 |  | 2000 MHz | – | 2020 MHz |  | 2180 MHz | – | 2200 MHz |  | FDD |  |
| 2417 |  | 1626.5 MHz | – | 1660.5 MHz |  | 1525 MHz | – | 1559 MHz |  | FDD |  |
| 25 |  | 1850 MHz | – | 1915 MHz |  | 1930 MHz | – | 1995 MHz |  | FDD |  |
| ... |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  | 1900 MHz | – | 1920 MHz |  | 1900 MHz | – | 1920 MHz |  | TDD |  |
| 34 |  | 2010 MHz | – | 2025 MHz |  | 2010 MHz | – | 2025 MHz |  | TDD |  |
| 35 |  | 1850 MHz | – | 1910 MHz |  | 1850 MHz | – | 1910 MHz |  | TDD |  |
| 36 |  | 1930 MHz | – | 1990 MHz |  | 1930 MHz | – | 1990 MHz |  | TDD |  |
| 37 |  | 1910 MHz | – | 1930 MHz |  | 1910 MHz | – | 1930 MHz |  | TDD |  |
| 38 |  | 2570 MHz | – | 2620 MHz |  | 2570 MHz | – | 2620 MHz |  | TDD |  |
| 39 |  | 1880 MHz | – | 1920 MHz |  | 1880 MHz | – | 1920 MHz |  | TDD |  |
| 40 |  | 2300 MHz | – | 2400 MHz |  | 2300 MHz | – | 2400 MHz |  | TDD |  |
| 41 |  | 2496 MHz |  | 2690 MHz |  | 2496 MHz |  | 2690 MHz |  | TDD |  |
| 42 |  | 3400 MHz | – | 3600 MHz |  | 3400 MHz | – | 3600 MHz |  | TDD |  |
| 43 |  | 3600 MHz | – | 3800 MHz |  | 3600 MHz | – | 3800 MHz |  | TDD |  |
| NOTE 1: Band 6 is not applicable NOTE 2: Void NOTE 3: Void NOTE 4: Void NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE 10: Void NOTE 11: Void NOTE 12: Void NOTE 13: Void NOTE 14: Void NOTE 15: Void NOTE 16: Void NOTE 17: DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz. |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 5.7.4	TX–RX frequency separation

a)	The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.7.4-1 for the TX and RX channel bandwidths defined in Table 5.6.1-1

Table 5.7.4-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX - RX carrier centre frequency separation |
| --- | --- |
| 1 | 190 MHz |
| 2 | 80 MHz. |
| 3 | 95 MHz. |
| 4 | 400 MHz |
| 5 | 45 MHz |
| 6 | 45 MHz |
| 7 | 120 MHz |
| 8 | 45 MHz |
| 9 | 95 MHz |
| 10 | 400 MHz |
| 11 | 48 MHz |
| 12 | 30 MHz |
| 13 | -31 MHz |
| 14 | -30 MHz |
| 17 | 30 MHz |
| 18 | 45 MHz |
| 19 | 45 MHz |
| 20 | -41 MHz |
| 21 | 48 MHz |
| 22 | 100 MHz |
| 23 | 180 MHz |
| 24 | -101.5, -120.5 MHz |
| 25 | 80 MHz |

b)	The use of other TX channel to RX channel carrier centre frequency separation is not precluded and is intended to form part of a later release.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.2.4	UE maximum output power with additional requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2.2-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 the specific requirements and identified subclauses are specified in Table 6.2.4-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4.-1 to 6.2.4-6 are in addition to the allowed MPR requirements specified in subclause 6.2.3.

Table 6.2.4-1: Additional Maximum Power Reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | 1.4, 3, 5, 10, 15, 20 | Table 5.6-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4,10, 23, 25, 35, 36 | 3 | >5 | ≤ 1 |
|  |  |  | 5 | >6 | ≤ 1 |
|  |  |  | 10 | >6 | ≤ 1 |
|  |  |  | 15 | >8 | ≤ 1 |
|  |  |  | 20 | >10 | ≤ 1 |
| NS_04 | 6.6.2.2.2 | 41 | 5 | >6 | ≤ 1 |
|  |  |  | 10, 15, 20 | Table 6.2.4-4 |  |
| NS_05 | 6.6.3.3.1 | 1 | 10,15,20 | ≥ 50 | ≤ 1 |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.6-1 | N/A |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | 10 | Table 6.2.4-2 | Table 6.2.4-2 |
| NS_08 | 6.6.3.3.3 | 19 | 10, 15 | > 44 | ≤ 3 |
| NS_09 | 6.6.3.3.4 | 21 | 10, 15 | > 40 | ≤ 1 |
|  |  |  |  | > 55 | ≤ 2 |
| NS_10 |  | 20 | 15, 20 | Table 6.2.4-3 | Table 6.2.4-3 |
| NS_11 | 6.6.2.2.1 6.6.3.3.13 | 23 | 1.4, 3, 5, 10, 15, 20 | Table 6.2.4-5 | Table 6.2.4-5 |
| .. |  |  |  |  |  |
| NS_20 | 6.2.2 6.6.2.2.1 6.6.3.3.14 | 23 | 5, 10, 15, 20 | Table 6.2.4-6 | Table 6.2.4-6 |
| NS_22 | 6.6.3.3.15 | 42, 43 | 5, 10, 15, 20 | Table 6.2.4-7 |  |
| NS_23 | 6.6.3.3.16 | 42, 43 | 5, 10, 15, 20 | N/A |  |
| .. |  |  |  |  |  |
| NS_32 | - | - | - | - | - |
| .. |  |  |  |  |  |
| NS_52 | 6.6.3.3.35 | 24 | 5, 10 | Table 6.2.4-34a |  |

Table 6.2.4-2: A-MPR for “NS_07”

| Parameters | Region A |  | Region B |  | Region C |
| --- | --- | --- | --- | --- | --- |
| RBstart | 0 - 12 |  | 13 – 18 | 19 – 42 | 43 – 49 |
| LCRB [RBs] | 6-8 | 1 to 5 and 9-50 | ≥8 | ≥18 | ≤2 |
| A-MPR [dB] | ≤ 8 | ≤ 12 | ≤ 12 | ≤ 6 | ≤ 3 |
| NOTE 1; RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2; LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. NOTE 4; For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. |  |  |  |  |  |

Table 6.2.4-3: A-MPR for “NS_10”

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart | 0 – 10 |
|  | LCRB [RBs] | 1 -20 |
|  | A-MPR [dB] | ≤ 2 |
| 20 | RBstart | 0 – 15 |
|  | LCRB [RBs] | 1 -20 |
|  | A-MPR [dB] | ≤ 5 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe |  |  |

Table 6.2.4-4: A-MPR requirements for "NS_04" with bandwidth >5MHz

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 10 | RBstart | 0 – 12 | 13 – 36 | 37 – 49 |
|  | RBstart + LCRB [RBs] | N/A | >37 | N/A 3 |
|  | A-MPR [dB] | ≤3dB | ≤2dB | ≤3dB |
| 15 | RBstart | 0 – 18 | 19 – 55 | 56 – 74 |
|  | RBstart + LCRB [RBs] | N/A | >56 | N/A 3 |
|  | A-MPR [dB] | ≤3dB | ≤2dB | ≤3dB |
| 20 | RBstart | 0 – 24 | 25 – 74 | 75 – 99 |
|  | RBstart + LCRB [RBs] | N/A | >75 | N/A 3 |
|  | A-MPR [dB] | ≤3dB | ≤2dB | ≤3dB |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: 3 refers to any RB allocation that starts in Region A or C is allowed the specified A-MPR NOTE 4: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 5: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |

Table 6.2.4-5: A-MPR for "NS_11"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Fc (MHz) | <2004 |  |  | ≥2004 |  |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | 1-15 |  |  | >5 |  |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤5 |  |  | ≤ 1 |  |  |  |  |  |  |  |  |  |  |
| 5 | Fc [MHz] | <2004 |  |  | 2004 ≤ Fc <2007 |  |  |  |  |  |  | ≥2007 |  |  |  |
|  | LCRB [RBs] | 1-25 |  |  | 1-6 & 15-25 |  |  |  | 8-12 |  |  | >6 |  |  |  |
|  | A-MPR [dB] | ≤7 |  |  | ≤ 4 |  |  |  | 0 |  |  | ≤ 1 |  |  |  |
| 10 | Fc [MHz] | 2005 ≤ Fc <2015 |  |  |  |  |  |  | 2015 |  |  |  |  |  |  |
|  | RBstart | 0-49 |  |  |  |  |  |  | 0-49 |  |  |  |  |  |  |
|  | LCRB [RBs] | 1-50 |  |  |  |  |  |  | 1-50 |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 12 |  |  |  |  |  |  | 0 |  |  |  |  |  |  |
| 15 | Fc [MHz] | <2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-4 | 5-21 |  |  |  |  |  |  |  | 22-56 |  |  |  | 57-74 |
|  | LCRB [RBs] | ≥1 | 7-50 |  |  | 0-6 & ≥50 |  |  |  |  | ≤25 |  | >25 |  | >0 |
|  | A-MPR [dB] | ≤15 | ≤7 |  |  | ≤10 |  |  |  |  | 0 |  | ≤6 |  | ≤15 |
|  | Fc [MHz] | 2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-12 |  | 13-39 |  |  |  |  |  | 40-65 |  |  |  | 66-74 |  |
|  | LCRB [RBs] | ≥1 |  | ≥30 |  |  |  | <30 |  | ≥ (69 – RBstart) |  |  |  | ≥1 |  |
|  | A-MPR [dB] | ≤10 |  | ≤6 |  |  |  | 0 |  | ≤2 |  |  |  | ≤6.5 |  |
| 20 | Fc [MHz] | 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-12 | 13-29 |  |  |  |  |  |  | 30-68 |  |  |  |  | 69-99 |
|  | LCRB [RBs] | ≥1 | 10-60 |  |  |  | 1-9 & >60 |  |  | 1-24 |  |  | ≥25 |  | ≥1 |
|  | A-MPR [dB] | ≤15 | ≤7 |  |  |  | ≤10 |  |  | 0 |  |  | ≤7 |  | ≤15 |

Table 6.2.4-6: A-MPR for "NS_20"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | < 2007.5 |  |  | 2007.5 ≤ Fc < 2012.5 |  |  |  |  |  |  |  |  |  |  |  | 2012.5 ≤ Fc ≤ 2017.5 |  |  |  |
|  | RBstart | ≤24 |  |  | 0-3 |  |  |  |  |  |  |  | 4-6 |  |  |  | ≤24 |  |  |  |
|  | LCRB [RBs] | >0 |  |  | 15-19 |  |  |  | ≥20 |  |  |  | ≥18 |  |  |  | 1-25 |  |  |  |
|  | A-MPR [dB] | ≤17 |  |  | ≤1 |  |  |  | ≤4 |  |  |  | ≤2 |  |  |  | ≤ 0 |  |  |  |
| 10 | Fc [MHz] | 2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-25 |  |  |  |  | 26-34 |  |  |  |  |  |  |  |  | 35-49 |  |  |  |  |
|  | LCRB [RBs] | >0 |  |  |  |  | 8-15 |  |  |  |  | >15 |  |  |  | >0 |  |  |  |  |
|  | A-MPR [dB] | ≤16 |  |  |  |  | ≤2 |  |  |  |  | ≤5 |  |  |  | ≤ 6 |  |  |  |  |
|  | Fc [MHz] | 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-5 |  |  |  |  |  |  |  |  | 6-10 |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | ≥32 |  |  |  |  |  |  |  |  | ≥40 |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤4 |  |  |  |  |  |  |  |  | ≤2 |  |  |  |  |  |  |  |  |  |
| 15 | Fc [MHz] | 2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-14 |  |  |  |  |  | 15-24 |  |  |  |  |  |  |  |  | 25-39 |  | 61-74 |  |
|  | LCRB [RBs] | 1-9 & 40-75 |  | 10-39 |  |  |  | 24-29 |  |  |  |  |  | ≥30 |  |  | ≥36 |  | ≤6 |  |
|  | A-MPR [dB] | ≤11 |  | ≤6 |  |  |  | ≤1 |  |  |  |  |  | ≤7 |  |  | ≤5 |  | ≤6 |  |
| 20 | Fc [MHz] | 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-21 | 22-31 |  |  |  |  |  |  | 32-38 |  |  |  |  | 39-49 |  |  | 50-68 |  | 69-99 |
|  | LCRB [RBs] | >0 | 1-9 & 31-75 |  |  | 10-30 |  |  |  | ≥15 |  |  |  |  | ≥24 |  |  | ≥25 |  | >0 |
|  | A-MPR [dB] | ≤17 | ≤12 |  |  | ≤6 |  |  |  | ≤9 |  |  |  |  | ≤7 |  |  | ≤5 |  | ≤16 |
| NOTE 1: When NS_20 is signaled the minimum requirements for the 10 MHz bandwidth are specified for E-UTRA UL carrier center frequencies of 2005 MHz or 2015 MHz. NOTE 2: When NS_20 is signaled the minimum requirements for the 15 MHz channel bandwidth are specified for E-UTRA UL carrier center frequency of 2012.5 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4-7: A-MPR for "NS_22"

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C | Region D |
| --- | --- | --- | --- | --- | --- |
| 5 | No A-MPR is needed for 5 MHz channel bandwidth |  |  |  |  |
| 10 | RBstart | 0-13 | 0-17 | ≤ 6 | ≥12 |
|  | LCRB [RBs] | > 36 | 33-36 | ≤ 32 | ≤ 32 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥44 |
|  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 3 | ≤ 3 |
| 15 | RBstart | 0-24 | 0-38 | ≤ 14 | ≥ 23 |
|  | LCRB [RBs] | > 50 | 37-50 | ≤ 36 | ≤ 36 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥59 |
|  | A-MPR [dB] | ≤ 5 | ≤ 4 | ≤ 3 | ≤ 3 |
| 20 | RBstart | 0-35 | 0-51 | ≤ 21 | ≥ 31 |
|  | LCRB [RBs] | > 64 | 49-64 | ≤ 48 | ≤ 48 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥79 |
|  | A-MPR [dB] | ≤ 5 | ≤ 4 | ≤ 3 | ≤ 3 |
| NOTE 1; RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2; LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. NOTE 4; For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. |  |  |  |  |  |

Table 6.2.4-8: Void

Table 6.2.4-9: Void

Table 6.2.4-10: Void

Table 6.2.4-11: Void

Table 6.2.4-12: Void

Table 6.2.4-13: Void

Table 6.2.4-14: Void

Table 6.2.4-15: Void

Table 6.2.4-16: Void

Table 6.2.4-17: Void

Table 6.2.4-18: Void

Table 6.2.4-18E: Void

Table 6.2.4-19: Void

Table 6.2.4-20: Void

Table 6.2.4-21: Void

Table 6.2.4-22: Void

Table 6.2.4-23: Void

Table 6.2.4-24: Void

Table 6.2.4-25: Void

Table 6.2.4-26: Void

Table 6.2.4-27: Void

Table 6.2.4-28: Void

Table 6.2.4-29: Void

Table 6.2.4-30a: Void

Table 6.2.4-30b: Void

Table 6.2.4-31: Void

Table 6.2.4-32: Void

Table 6.2.4-32a: Void

Table 6.2.4-32b: Void

Table 6.2.4-33: Void

Table 6.2.4-34: Void

Table 6.2.4-34a: A-MPR for "NS_52"

| Channel bandwidth confined to 1627.5- 1637.5MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | Carrier centre frequency (FC) (MHz) | Parameters | Region A | Region B |
| 5 MHz | 1630.0, 1630.3 | RBstart | ≤ 8 | ≤ 8 |
|  |  | LCRB [RBs] | ≤ 8 | > 8 |
|  |  | A-MPR [dB] | 3 | 2 |
|  | 1635.0 | No A-MPR needed |  |  |
|  | 1649.0 |  |  |  |
|  | 1654.0 |  |  |  |
| 10 MHz | 1632.5 | RBstart | ≤ 18 | ≤ 18 |
|  |  | LCRB [RBs] | ≤ 12 | > 12 |
|  |  | A-MPR [dB] | 5 | 7 |
|  | 1651.5 | No A-MPR needed |  |  |
| NOTE 1: The additional emission requirements in Table 6.6.3.3.35-1 are specified in EIRP. The A-MPR values to meet these additional emission requirements have been estimated using a 0 dBi antenna gain. The A-MPR may have to be adjusted if the supported antenna gain Gant declared by the UE manufacturer is different from 0 dBi. |  |  |  |  |

For PRACH, PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For each subframe, the A-MPR is evaluated per slot and given by the maximum value taken over the transmission(s) within the slot; the maximum A-MPR over the two slots is then applied for the entire subframe.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2.5 apply.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.6.3.3	Additional spurious emissions

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

<Unchanged sections omitted>

##### 6.6.3.3.17	Void

Table 6.6.3.3.17-1: Void

##### 6.6.3.3.18	Void

Table 6.6.3.3.18-1: Void

##### 6.6.3.3.19	Void

Table 6.6.3.3.19-1: Void

##### 6.6.3.3.20	Void

Table 6.6.3.3.20-1: Void

##### 6.6.3.3.21	Void

Table 6.6.3.3.21-1: Void

##### 6.6.3.3.22	Void

Table 6.6.3.3.22-1: Void

##### 6.6.3.3.23	Void

Table 6.6.3.3.23-1: Void

##### 6.6.3.3.24	Void

Table 6.6.3.3.24-1: Void

##### 6.6.3.3.25	Void

Table 6.6.3.3.25-1: Void

##### 6.6.3.3.26	Void

Table 6.6.3.3.26-1: Void

Table 6.6.3.3.26-2: Void

Table 6.6.3.3.26-3: Void

##### 6.6.3.3.27	Void

Table 6.6.3.3.27-1: Void

Table 6.6.3.3.27-2: Void

Table 6.6.3.3.27-3: Void

Table 6.6.3.3.27-4: Void

##### 6.6.3.3.28	Void

Table 6.6.3.3.28-1: Void

##### 6.6.3.3.29	Void

Table 6.6.3.3.29-1: Void

##### 6.6.3.3.30	Void

Table 6.6.3.3.30-1: Void

##### 6.6.3.3.31	Void

Table 6.6.3.3.31-1: Void

##### 6.6.3.3.32	Void

Table 6.6.3.3.32-1: Void

##### 6.6.3.3.33	Void

Table 6.6.3.3.33-1: Void

##### 6.6.3.3.34	Void

Table 6.6.3.3.34-1: Void

Table 6.6.3.3.34-2: Void

##### 6.6.3.3.35 	Minimum requirement (network signalled value “NS_52”)

When "NS_52" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.35-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.35-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit in EIRP1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | EIRP of discrete emissions of less than 700 Hz bandwidth, averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | EIRP and averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement is converted to conducted requirement depending on the supported antenna gain Gant declared by the UE manufacturer. If not declared by the UE manufacturer, a 0 dBi antenna gain shall be used. |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

# G.2		Typical receiver sensitivity performance (QPSK)

The residual BLER after HARQ shall be lower than 1% for the reference measurement channels as specified in Annexes G.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table G.2-1 and Table G.2-2

Table G.2-1: Reference sensitivity QPSK PSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  |  | [-102] |  |  | FDD |
| 2 |  |  |  | TBD |  |  | FDD |
| 3 |  |  |  | TBD |  |  | FDD |
| 4 |  |  |  | TBD |  |  | FDD |
| 5 |  |  |  | TBD |  |  | FDD |
| 6 |  |  |  | TBD |  |  | FDD |
| 7 |  |  |  | TBD |  |  | FDD |
| 8 |  |  |  | TBD |  |  | FDD |
| 9 |  |  |  | TBD |  |  | FDD |
| 10 |  |  |  | TBD |  |  | FDD |
| 11 |  |  |  | TBD |  |  | FDD |
| 12 |  |  |  | TBD |  |  | FDD |
| 13 |  |  |  | TBD |  |  | FDD |
| 14 |  |  |  | TBD |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 17 |  |  |  | TBD |  |  | FDD |
| 18 |  |  |  | TBD |  |  | FDD |
| 19 |  |  |  | TBD |  |  | FDD |
| 20 |  |  |  | TBD |  |  | FDD |
| 21 |  |  |  | TBD |  |  | FDD |
| 22 |  |  |  | TBD |  |  | TBD |
| 23 |  |  |  | TBD |  |  | FDD |
| 24 |  |  |  | [-102] |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 33 |  |  |  | [-102] |  |  | TDD |
| 34 |  |  |  | [-102] |  |  | TDD |
| 35 |  |  |  | [-102] |  |  | TDD |
| 36 |  |  |  | [-102] |  |  | TDD |
| 37 |  |  |  | [-102] |  |  | TDD |
| 38 |  |  |  | [-102] |  |  | TDD |
| 39 |  |  |  | [-102] |  |  | TDD |
| 40 |  |  |  | [-102] |  |  | TDD |
| 41 |  |  |  | [-102] |  |  | TDD |
| 42 |  |  |  | [-102] |  |  | TDD |
| Note 1: The transmitter shall be set to PUMAX as defined in clause 6.2.5 Note 2: Reference measurement channel is G.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. Note 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. |  |  |  |  |  |  |  |

Table G.2-2 specifies the minimum number of allocated uplink resource blocks for which the reference receive sensitivity requirement in lower SNR must be met.

Table G.2-2: Minimum uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 1 |  |  |  | [6]1 |  |  | FDD |
| 2 |  |  |  | [6]1 |  |  | FDD |
| 3 |  |  |  | [6]1 |  |  | FDD |
| 4 |  |  |  | [6]1 |  |  | FDD |
| 5 |  |  |  | [6]1 |  |  | FDD |
| 6 |  |  |  | [6]1 |  |  | FDD |
| 7 |  |  |  | [6]1 |  |  | FDD |
| 8 |  |  |  | [6]1 |  |  | FDD |
| 9 |  |  |  | [6]1 |  |  | FDD |
| 10 |  |  |  | [6]1 |  |  | FDD |
| 11 |  |  |  | [6]1 |  |  | FDD |
| 12 |  |  |  | [6]1 |  |  | FDD |
| 13 |  |  |  | [6]1 |  |  | FDD |
| 14 |  |  |  | [6]1 |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 17 |  |  |  | [6]1 |  |  | FDD |
| 18 |  |  |  | [6]1 |  |  | FDD |
| 19 |  |  |  | [6]1 |  |  | FDD |
| 20 |  |  |  | [6]1 |  |  | FDD |
| 21 |  |  |  | [6]1 |  |  | FDD |
| 22 |  |  |  | [6]1 |  |  | FDD |
| 23 |  |  |  | [6]1 |  |  | FDD |
| 24 |  |  |  | [6]1 |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 33 |  |  |  | 50 |  |  | TDD |
| 34 |  |  |  | 50 |  |  | TDD |
| 35 |  |  |  | 50 |  |  | TDD |
| 36 |  |  |  | 50 |  |  | TDD |
| 37 |  |  |  | 50 |  |  | TDD |
| 38 |  |  |  | 50 |  |  | TDD |
| 39 |  |  |  | 50 |  |  | TDD |
| 40 |  |  |  | 50 |  |  | TDD |
| 41 |  |  |  | 50 |  |  | TDD |
| 42 |  |  |  | 50 |  |  | TDD |
| Note 1. The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). 2. For the UE which supports both Band 11 and Band 21 the minimum uplink configuration for reference sensitivity is FFS. 4. For Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart _11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart _16 |  |  |  |  |  |  |  |

Unless given by Table G.2-3, the minimum requirements specified in Tables G.2-1 and G.2-2 shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

Table G.2-3: Network Signalling Value for reference sensitivity

| E-UTRA Band | Network Signalling value |
| --- | --- |
| 2 | NS_03 |
| 4 | NS_03 |
| 10 | NS_03 |
| 12 | NS_06 |
| 13 | NS_06 |
| 14 | NS_06 |
| 17 | NS_06 |
| 19 | NS_08 |
| 21 | NS_09 |
| 23 | NS_03 |
| 24 | NS_52 |
| 35 | NS_03 |
| 36 | NS_03 |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------