3GPP TSG-RAN WG4 Meeting #98-e                         　　　　　　　　　       R4-2100345

Online, 25th Jan-5th Feb, 2021

Source: 	KDDI.

Title:	TP for TR 37.717-21-11: DC_3-18_n41

Agenda item:	9.4.2

Document for:	Approval

- Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_3A-18A_n41A and according to the request in [1].

- References

[1]	RP-202060, Revised WID on Rel-17 Dual Connectivity (DC) of 2 bands LTE inter-band CA  (2DL/1UL) and 1 NR

band (1DL/1UL).

- Text Proposal

< Start of Text Proposal>

5.x	DC_3-18_n41

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-18A_n41A | DC_3A_n41A DC_18A_n41A |

5.x.2	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. And it can be get:

- IMD2 of band 18 UL and band n41 UL falling to band 3 DL.

- IMD3 of band 3 UL and band n41 UL falling to band 18 DL.

5.x.3	∆TIB and ∆RIB values

For DC_3-18_n41, the TIB,c and RIB,c values are given in the tables below.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41 | 3 | 0.3 |
|  | 18 | 0.5 |
|  | n41 | 0.5 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18_n41 | 3 | 0 |
|  | 18 | 0 |
|  | n41 | 0 |

5.x.4	Reference sensitivity exceptions

Table 5.x.4-1 shows the required MSD:

Table 5.x.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL Configuration | EUTRA and NR band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | IMD order |
| DC_3-18_n41 | 18 | 820 | 5 | 25 | 865 | 5 | N/A | N/A |
|  | n41 | 2670 | 5 | 25 | 2670 | 5 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 5 | 28.8 | IMD2 |
|  | 3 | 1725 | 10 | 50 | 1820 | 10 | N/A | N/A |
|  | n41 | 2565 | 10 | 50 | 2685 | 10 | N/A | N/A |
|  | 18 | 830 | 5 | 25 | 875 | 5 | 19.0 | IMD3 |

< End of Text Proposal>