3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100378

Electronic Meeting, Jan. 25-Feb. 5, 2021

Title: 	Discussion on NR repeater EVM and frequency stability requirements for FR1/FR2

Source: 	CATT

Agenda item:	11.11.2.3

Document for:	Approval

- Introduction

A new WI NR repeater was approved in RAN#90 meeting to define NR repeaters for both FR1 and FR2 bands including FDD and TDD mode. This contribution provides our analysis for RF repeater’s EVM and Frequency stability.

- Discussion

2.1 EVM

To our understanding, the EVM requirements for FR1 and FR2 repeater can be discussed as a whole because EVM requirement in theory is the SNR requirement for the Tx signals. The SNR requirement for a dedicate modulation doesn’t change because of frequency ranges. IAB EVM requirements are the same as BS from SNR perspective for both downlink and uplink. The system performance can be thought as the same as BS to UE or UE to BS. However, repeater’s EVM requirements need some discussion because extra noise are added to the signals after repeater amplifies and forwards the signal from donor BS. In order not to degrade the signal too much, keeping the noise level from repeater as low as possible is important. However, the contribution from RF hardware part especially is usually much larger than BB part in the BS design, it’s difficult to limit repeater’s EVM requirement to a very low level considering the PA efficiency and power consumption. According to the following equation,

EVMtotal=EVM12+EVM22

Table 1: Repeater EVM’s impact to total EVM

| EVM_received signal(%) | 10 | 10 | 8 | 8 | 6 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| EVM_repeater(%) | 8 | 6 | 8 | 6 | 8 | 6 |
| EVM_total(%) | 12.8 | 11.7 | 11.3 | 10.0 | 10.0 | 8.5 |

LTE repeater EVM requirement is 8%. As shown in Table 1, 8% EVM requirement impact to the final EVM is a little large. So we propose to define the requirement as 6%.

Proposal 1: EVM requirements for NR repeater downlink and uplink are defined as 6%. The proposal applies to both FR1 and FR2.

2.2 Frequency stability

Similar with EVM, RF repeater’s frequency error added on top of the frequency error of the received signal. NR RF repeater’s frequency stability requirement can follow LTE repeater’s requirements, i.e. ± 0.01 ppm.

Proposal 2: Frequency stability requirement for NR repeater reuse BS approach and define the requirement as ± 0.01 ppm. The proposal applies to both FR1 and FR2.

- Conclusion

This contribution provides the proposals for NR repeater EVM and frequency stability.

Proposal 1: EVM requirements for NR repeater downlink and uplink are defined as 6%. The proposal applies to both FR1 and FR2.

Proposal 2: Frequency stability requirement for NR repeater reuse BS approach and define the requirement as ± 0.01 ppm. The proposal applies to both FR1 and FR2.

Reference

[1] RP-202927, New WID on NR Repeaters