3GPP TSG-RAN WG4 Meeting #98-e	R4-2100397

Electronic Meeting, 25th Jan. – 5th Feb., 2021

Title: 	Discussion on power saving demodulation test

Source: 	CATT

Agenda item:	7.6.2

Document for:	Discussion

- Introduction

In RAN4#97-e meeting, PDCCH DCI format 2_6 demodulation test was discussed and it was agreed to use option 1c with Pm-dsg_total=1% to define joint PDCCH/PDCCH-WUS performance [1]. In addition, the following principles were also agreed.

- Test parameters for PDCCH

  - Reuse the test parameters for PDCCH in DRX-ON in 38.101-4

  - For FR1

    - For FDD: Test No. 3 in Table 5.3.2.1.1-1 and Test No. 3 in Table 5.3.3.1.1-1

    - For TDD: Test No. 2 in Table 5.3.2.2.1-1 and Test No. 2 in Table 5.3.3.2.1-1

  - For FR2

    - For TDD: Test No. 1-2 in Table 7.3.2.2.1-1

- Applicability rule

  - Apply the applicability rule that UE already fulfilling PDCCH-WUS performance requirement can skip the corresponding Rel-15 test(s)

- The number of search space for PDCCH-WUS

  - 1 search space for PDCCH-WUS

This document will provide the simulation results based on the simulation assumptions in the annex and further discuss the test parameter selection for the requirement.

- Discussion

2.1. Simulation results for FR1

1). Simulation results for PDCCH-WUS for FR1-FDD

2). Simulation results for PDCCH-WUS for FR1-TDD

From these simulation results, PDCCH-WUS performance can get much lower BLER than 0.1% at the same SNR as for the current normal PDCCH performance at 1% BLER. We propose to use option 1  (12 bits , AL=8) for PDCCH-WUS performance test for FR1 FDD and TDD

Proposal 1: It is proposed to use option 1 (12 bits, AL=8) for PDCCH-WUS performance test for FR1 FDD and TDD.

2.1. FR2 simulations

Firstly, it should be clarified that the simulation assumption for FR2 PDCCH-WUS in WF [1] has a problem, i.e. in Option 2, 1 CORESET symbol can’t accommodate 36bits with AL=16 when 60PRB CORESET bandwidth. In our simulations, we consider two methods to account for the payload of 36bits, i.e. 1 CORESET symbol with AL=8 or 2 CORESET symbol with AL=16. Then totally the following 3 test cases were considered,

Option 1:	12 bits + AL=8+1CORESET symbol

Option 2a: 	36 bits + AL=8+1CORESET symbol

Option 2b:	36 bits + AL=16+2 CORESET symbols

From these simulation results we can see that all three parameter selections for PDCCH-WUS performance can get <0.1% BLER at the same SNR setting for current normal PDCCH at 1% BLER. It is proposed to consider option 2a (36 bits , AL=8) for FR2 PDCCH-WUS performance test.

Proposal 2: It is proposed to consider option 2a (36 bits, AL=8) for FR2 PDCCH-WUS performance test.

- Conclusion

This contribution discussed PDCCH DCI formant 2_6 performance test and presented simulation results.

Proposal 1: It is proposed to use option 1 (12 bits, AL=8) for PDCCH-WUS performance test for FR1 FDD and TDD.

Proposal 2: It is proposed to consider option 2a (36 bits, AL=8) for FR2 PDCCH-WUS performance test.

Based on the proposal in this paper, a CR for FR2 PDCCH DCI formant 2_6 performance test is provided in [3].

- Reference

[1]	R4-2017677, WF on power saving demodulation, CMCC, CATT

[2]	R4-2017659, Work plan for power saving demodulation, CATT

[3]	R4-2100398, CR for TS38.101-4, test for FR2 PDCCH DCI format 2_6 demodulation, CATT

- Annex, simulation

1)  Simulation assumption

- Simulation assumptions for PDCCH-WUS for FR1-FDD

| Parameter | Value |  |
| --- | --- | --- |
|  | PDCCH-WUS | PDCCH |
| Duplex mode | FDD |  |
| DCI format | 2_6 | 1_1 |
| Bandwidth | 10MHz |  |
| SCS | 15kHz |  |
| Number of BS antennas | 1Tx |  |
| Number of UE antennas | 2Rx, 4Rx |  |
| DM-RS channel estimation | Realistic |  |
| Channel model | TDLA30-10 |  |
| DCI length (excluding 24bits CRS) + Aggregation level | Option1:12 bits , AL=8 Option2: 36 bits , AL=16 | 52 bits + AL=4 |
| CORESET symbol | 2 |  |
| CORESET bandwidth | 48RB |  |
| Mapping type | Non-Interleaved | Non-Interleaved |
| REG bundle size | 6 |  |

- Simulation assumptions for PDCCH-WUS for FR1-TDD

| Parameter | Value |  |
| --- | --- | --- |
|  | PDCCH-WUS | PDCCH |
| Duplex mode | TDD |  |
| DCI format | 2_6 | 1_1 |
| Bandwidth | 40MHz |  |
| SCS | 30kHz |  |
| Number of BS antennas | 1Tx |  |
| Number of UE antennas | 2Rx, 4Rx |  |
| DM-RS channel estimation | Realistic |  |
| Channel model | TDLC300-100 |  |
| DCI length (excluding 24bits CRS) + Aggregation level | Option1:12 bits + AL=8 Option2: 36 bits + AL=16 | 53 bits + AL=4 |
| CORESET symbol | 1 |  |
| CORESET bandwidth | 102RB |  |
| Mapping type | Interleaved(size 3) | Interleaved(size 3) |
| REG bundle size | 2 |  |

- Simulation assumptions for PDCCH-WUS for FR2

| Parameter | Value |  |
| --- | --- | --- |
|  | PDCCH-WUS | PDCCH |
| Duplex mode | TDD |  |
| DCI format | 2_6 | 1_1 |
| Bandwidth | 100MHz |  |
| SCS | 120kHz |  |
| Number of BS antennas | 1Tx |  |
| Number of UE antennas | 2Rx |  |
| DM-RS channel estimation | Realistic |  |
| Channel model | TDLA30-300 |  |
| DCI length (excluding 24bits CRS) + Aggregation level | Option1:12 bits + AL=8 Option2: 36 bits + AL=16 | 56 bits + AL=4 |
| CORESET symbol | 1 |  |
| CORESET bandwidth | 60RB |  |
| Mapping type | Interleaved(size 2) | Interleaved(size 2) |
| REG bundle size | 6 |  |