3GPP TSG-RAN4 Meeting #98-e	                                       R4-2100423

Electronic meeting, Jan. 25 – Feb. 5, 2021

Title:	Updated simulation results for CSI-RSRP measurement

Source:	CATT

Agenda Item:	7.14.2.1.1

Document for:	Discussion

- Introduction

In RAN4#92 meeting [1], it was agreed to conduct simulation studies to evaluate measurement bandwidth of CSI-RS for RRM measurement requirement. And the simulation assumption for CSI-RS based measurement is agreed in [2].

In RAN4 #96e meeting [3], it was agreed that the CSI-RS based L3 measurement requirements in R16 are defined for CSI-RS configuration {D=3 with PRB≥48}.

In last meeting, we have given our simulation results of CSI-RSRP measurement. Based on the latest conclusion [4], in this contribution, we provide the updated simulation results.

- Discussion

In this section, we provide the simulation results of delta RSRP under AWGN and EPA5 channel for 15 kHz and 30 kHz and provide the simulation results of delta RSRP under TDL-A and TDL-C channel for 60 kHz and 120 kHz. Table 1-8 provide the delta RSRP based on different samples, different transmission power which is indicated by SINR for 15 kHz, 30 kHz, 60 kHz and 120 kHz, respectively. The simulations are based on the CSI-RS configuration {D=3, PRB = 48}.

For FR1：

15 kHz case:

Table 1: Delta RSRP, SINR=-5.97 dB, td=CP, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -0.72 | 0.23 | 1.14 | 0.95 | 0.91 | 0.95 |
|  | 5 | -0.56 | 0.23 | 0.93 | 0.79 | 0.7 | 0.79 |
|  | 10 | -0.34 | 0.23 | 0.75 | 0.57 | 0.52 | 0.57 |
| EPA5 | 3 | -1.31 | 0.12 | 1.34 | 1.43 | 1.22 | 1.43 |
|  | 5 | -0.96 | 0.12 | 1.05 | 1.08 | 0.93 | 1.08 |
|  | 10 | -0.67 | 0.12 | 0.83 | 0.79 | 0.71 | 0.79 |

Table 2: Delta RSRP, SINR=-3.97 dB, td= CP, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -0.49 | 0.23 | 0.94 | 0.72 | 0.71 | 0.72 |
|  | 5 | -0.37 | 0.23 | 0.77 | 0.6 | 0.54 | 0.6 |
|  | 10 | -0.17 | 0.23 | 0.64 | 0.4 | 0.41 | 0.41 |
| EPA5 | 3 | -1.05 | 0.09 | 1.09 | 1.14 | 1 | 1.14 |
|  | 5 | -0.78 | 0.09 | 0.86 | 0.87 | 0.77 | 0.87 |
|  | 10 | -0.57 | 0.09 | 0.67 | 0.66 | 0.58 | 0.66 |

30 kHz case:

Table 3: Delta RSRP, SINR=-5.97 dB, td= CP, 30 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -0.68 | 0.25 | 1.12 | 0.93 | 0.87 | 0.93 |
|  | 5 | -0.39 | 0.25 | 0.96 | 0.64 | 0.71 | 0.71 |
|  | 10 | -0.22 | 0.25 | 0.72 | 0.47 | 0.47 | 0.47 |
| EPA5 | 3 | -1.36 | 0.21 | 1.58 | 1.57 | 1.37 | 1.57 |
|  | 5 | -1.02 | 0.21 | 1.31 | 1.23 | 1.1 | 1.23 |
|  | 10 | -0.64 | 0.21 | 0.94 | 0.85 | 0.73 | 0.85 |

Table 4: Delta RSRP, SINR=-3.97 dB, td= CP, 30 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -0.45 | 0.26 | 0.93 | 0.71 | 0.67 | 0.71 |
|  | 5 | -0.25 | 0.26 | 0.81 | 0.51 | 0.55 | 0.55 |
|  | 10 | -0.11 | 0.26 | 0.62 | 0.37 | 0.36 | 0.37 |
| EPA5 | 3 | -1.12 | 0.15 | 1.26 | 1.27 | 1.11 | 1.27 |
|  | 5 | -0.81 | 0.15 | 1.04 | 0.96 | 0.89 | 0.96 |
|  | 10 | -0.53 | 0.15 | 0.72 | 0.68 | 0.57 | 0.68 |

For FR2：

60 kHz case:

Table 5: Delta RSRP, SINR=-5.97 dB, td= CP, 60 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 3 | -1.79 | 0.15 | 1.94 | 1.94 | 1.79 | 1.94 |
|  | 5 | -1.35 | 0.15 | 1.62 | 1.5 | 1.47 | 1.5 |
|  | 10 | -0.94 | 0.15 | 1.18 | 1.09 | 1.03 | 1.09 |
| TDL-C | 3 | -1.32 | 0.09 | 1.34 | 1.41 | 1.25 | 1.41 |
|  | 5 | -0.99 | 0.09 | 1.08 | 1.08 | 0.99 | 1.08 |
|  | 10 | -0.70 | 0.09 | 0.82 | 0.79 | 0.73 | 0.79 |

Table 6: Delta RSRP, SINR=-3.97 dB, td= CP, 60 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 3 | -1.42 | 0.13 | 1.56 | 1.55 | 1.43 | 1.55 |
|  | 5 | -1.03 | 0.13 | 1.27 | 1.16 | 1.14 | 1.16 |
|  | 10 | -0.74 | 0.13 | 0.92 | 0.87 | 0.79 | 0.87 |
| TDL-C | 3 | -1.08 | 0.06 | 1.08 | 1.14 | 1.02 | 1.14 |
|  | 5 | -0.76 | 0.06 | 0.86 | 0.82 | 0.8 | 0.82 |
|  | 10 | -0.56 | 0.06 | 0.64 | 0.62 | 0.58 | 0.62 |

120 kHz case:

Table 7: Delta RSRP, SINR=-5.97 dB, td= CP, 120 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 3 | -1.85 | 0.48 | 2.71 | 2.33 | 2.23 | 2.33 |
|  | 5 | -1.37 | 0.48 | 2.24 | 1.85 | 1.76 | 1.85 |
|  | 10 | -0.78 | 0.48 | 1.67 | 1.26 | 1.19 | 1.26 |
| TDL-C | 3 | -1.73 | 0.42 | 2.56 | 2.15 | 2.14 | 2.15 |
|  | 5 | -1.20 | 0.42 | 2.13 | 1.62 | 1.71 | 1.71 |
|  | 10 | -0.78 | 0.42 | 1.52 | 1.2 | 1.1 | 1.2 |

Table 8: Delta RSRP, SINR=-3.97 dB, td= CP, 120 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 3 | -1.60 | 0.36 | 2.19 | 1.96 | 1.83 | 1.96 |
|  | 5 | -1.18 | 0.36 | 1.79 | 1.54 | 1.43 | 1.54 |
|  | 10 | -0.76 | 0.36 | 1.32 | 1.12 | 0.96 | 1.12 |
| TDL-C | 3 | -1.45 | 0.27 | 2.01 | 1.72 | 1.74 | 1.74 |
|  | 5 | -1.04 | 0.27 | 1.66 | 1.31 | 1.39 | 1.39 |
|  | 10 | -0.68 | 0.27 | 1.24 | 0.95 | 0.97 | 0.97 |

According to the above simulation results, the following observations are given:

For absolute accuracy:

Observation 1: When the sample number is 5, the absolute measurement error can be within ±1.5dB for the Es/Iot = -5.97dB and within ±1dB for Es/Iot = -3.97dB in FR1.

Observation 2: When the sample number is 5, the absolute measurement error can be within ±2.5dB for the Es/Iot = -5.97dB and within ±2dB for Es/Iot = -3.97dB in FR2.

For relative accuracy:

Observation 3: When the sample number is 5, the relative measurement error can be within ±1.5dB for the Es/Iot = -5.97dB and within ±1dB for Es/Iot = -3.97dB in FR1.

Observation 4: When the sample number is 5, the relative measurement error can be within ±2dB for the Es/Iot = -5.97dB and within ±1.5dB for Es/Iot = -3.97dB in FR2.

- Conclusion

In this contribution, we provide the updated simulation results for CSI-RSRP measurement, and provide the observations as follows:

Observation 1: When the sample number is 5, the absolute measurement error can be within ±1.5dB for the Es/Iot = -5.97dB and within ±1dB for Es/Iot = -3.97dB in FR1.

Observation 2: When the sample number is 5, the absolute measurement error can be within ±2.5dB for the Es/Iot = -5.97dB and within ±2dB for Es/Iot = -3.97dB in FR2.

Observation 3: When the sample number is 5, the relative measurement error can be within ±1.5dB for the Es/Iot = -5.97dB and within ±1dB for Es/Iot = -3.97dB in FR1.

Observation 4: When the sample number is 5, the relative measurement error can be within ±2dB for the Es/Iot = -5.97dB and within ±1.5dB for Es/Iot = -3.97dB in FR2.

- Reference

[1] R4-1910701, “RAN4 #92 Meeting report”, RAN4

[2] R4-1910949, “Link-level simulation assumptions for CSI-RS based measurements”, CATT

[3] RAN4_96e_RRM_chairman_report_12_EOM

[4] R4-2017230, Updated link-level simulation assumptions for CSI-RS based L3 measurements, CATT