3GPP TSG-RAN WG4 Meeting #98-e                                                R4-2100474

Electronic Meeting, 25 Jan. – 05 Feb., 2021

Title: 	Discussion on RLM/BFD relaxation factor

Source: 	CATT

Agenda item:	11.9.2

Document for:	Discussion

Introduction

In RAN4#97-e meeting, RLM/BFD relaxation for power saving was discussed. WF[1] was agreed. In this contribution, we provide analysis on RLM/BFD relaxation for NR UE power saving.

Discussion

For the RLM/BFD measurement relaxation schemes listed in WF[1]

- At least extending evaluation period of RLM/BFD measurement (Option 1) to be considered as the scheme of RLM/BFD measurements relaxation. FFS schemes as follows - Option 1a: RAN4 to further discuss use of a scaling factor for defining the relaxed RLM/BM evaluation period and indication intervals. - Option 2: Reducing the number of candidate beams when UE fulfilled relaxed criteria can be a feasible way to reduce power consuming. - Option 3: Reducing the number reducing the sample number.

We use the option 1a for simulation. Define K as the relaxation factor.

K = 1: every SSB sample is used for RLM measurement.

K = 2: every other SSB sample is used for RLM measurement

K = 4: every four SSB sample is used for RLM measurement

In Rel-15, the delta SINR is used for RLM evaluation. By increasing number of SSB with in evaluation period reduces the span of delta SINR.  Delta SINR = (estimated SINR – ideal SINR)

In WF[1]:

- Study the system impact of relaxed RLM/BFD measurements, taking in to account the following evaluation metrics: - increased latency in RLF triggering (for RLM) - increased latency in beam failure detection and the initiation of beam recovery procedure (for BFD) - Delta SINR as one of the performance statistic to evaluate the RLM/BFD performance impact

We use “Delta SINR = (estimated SINR_R17– estimated SINR_R15)” for simulation.

We evaluate Delta SINR in [2].  It shows delta SINR CDF curves by using different values of K (K = 2 and K=4). The 5%-ile and 95%-ile CDF values calculated from the CDF curves are summarized in Table.

Based on the result, delta SINR values of 5%-ile and 95%-ile CDF curves based on SSB are within a range of +1.89/- 1.96 dB with K = 2, +2.55/- 2.62 dB with K = 4, respectively for speed is 3km/h. delta SINR values of 5%-ile and 95%-ile CDF curves based on SSB are within a range of +4.6/- 4.46 dB with K = 2, +5.35/- 5.23 dB with K = 4, respectively for speed is 30km/h. Considering the accuracy of SINR, when speed is 30km/h, the system cannot meet the same performance as that in release 15.

Observation 1: for low speed, the relaxation factor can be 4 with no performance degradation for RLM compared to Rel-15.

Observation 2: When speed is 30km/h, the system cannot meet the same performance as that in release 15.

Proposal 1: Use of a scaling factor for defining the relaxed RLM/BM evaluation period. Use low mobility criteria to do RLM relaxation for power saving, and the relaxation factor can be 4 with no performance degradation for RLM compared to Rel-15.

Conclusion

In this contribution, we provide analysis on RLM/BFD relaxation for NR UE power saving.

Proposal 1: Use of a scaling factor for defining the relaxed RLM/BM evaluation period. Use low mobility criteria to do RLM relaxation for power saving, and the relaxation factor can be 4 with no performance degradation for RLM compared to Rel-15.

Reference

[1] R4-2017269, “WF on NR UE Power Saving Enhancements”, MediaTek

[2] R4-2100478, “Initial performance evaluation for RLM/BFD relaxation factor”