3GPP TSG-RAN WG4 Meeting #98-e                                                R4-2100475

Electronic Meeting, 25 Jan. – 05 Feb., 2021

Title: 	Discussion on CA for NR FR1 HST

Source: 	CATT

Agenda item:	11.6.2.1

Document for:	Discussion

Introduction

The work item was agreed for release 17 for further enhancement for HST in NR FR1 in [1].

- Carrier Aggregation (CA) scenario - Specify the UE RRM core requirements for CA scenario with the same target speed (up to 500km/h) and carrier frequency (up to 3.6 GHz) as Rel-16 NR HST

In this contribution, we summarize our views on RRM requirement for high speed train scenario for NR FR1 CA and give our proposals.

Discussion

In release 16, it supports HST for single SA carrier. When highSpeedMeasFlag-r16 is enabled, the enhancement is for both idle and RRC connected state. There is no need to consider idle mode for CA. But for RRC connected state, when highSpeedMeasFlag-r16 is enabled, there are enhanced requirements for the PCC cell identification and measurement period in [2].

Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps(FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2.5.2-3: Measurement period for intra-frequency measurements without gaps (deactivated SCell) (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | 5 x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | 5 x max(measCycleSCell, DRX cycle) x CSSFintra |

Table 9.2.5.2-5: T SSB_measurement_period_intra When highSpeedMeasFlag-r16 is configured (Frequency range FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX Note 2 | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 160ms | max(200ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| 160ms < DRX cycle≤ 320ms | ceil(4 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle) |
| DRX cycle>320ms | ceil( Y Note 3 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

Proposal 1: The current table 9.2.5.2-3 cannot work for high speed. Enhanced intrafrequency cell identification and measurement period for SCC should be defined for HST CA scenario.

For PSS/SSS detection, deactivated SCell, the current requirement is shown as below:

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | 5 x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | 5 x max(measCycleSCell, DRX cycle) x CSSFintra |

In 38.331,

measCycleSCell                      ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}  OPTIONAL    -- Need R

For long DRX cycle such as >320ms, it is more than 1.6s. If the speed is 500km/h, which means UE will move more than 222m during the time. If UE moves across several cells during the time which means UE cannot work well for CA operation.

Proposal 2: PSS/SSS detection for deactivated SCell may need to be enhanced depends on the deployment scenario.

Proposal 3: RAN4 need to clarify the deployment for CA. PCell and SCell have the same coverage or not?

Conclusion

In this contribution, we summarize our initial views on RRM requirement for high speed train scenario for NR FR1 CA.

Proposal 1: The current table 9.2.5.2-3 cannot work for high speed. Enhanced intrafrequency cell identification and measurement period for SCC should be defined for HST CA scenario.

Proposal 2: PSS/SSS detection for deactivated SCell may need to be enhanced depends on the deployment scenario.

Proposal 3: RAN4 need to clarify the deployment for CA. PCell and SCell have the same coverage or not?

Reference

[1] RP-202335, “Revised WID on enhanced NR support for high speed train scenario for frequency range 1 (FR1)” CMCC

[2] 38.133 16.6.0