3GPP RAN WG4 Meeting #98-e		R4-2100516

Online, 25th January – 05th February 2021

Agenda item:	9.22.3

Source:	Apple

Title:	A-MPR Proposal for n1 and 45MHz CBW

WI/SI:	NR_FR1_35MHz_45MHz_BW-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The latest revision of the WID introducing 35MHz and 45MHz CBW includes band n1 [1]. Simulations have been done for 45MHz channel bandwidth and NS_48 and NS_49. This paper proposes dedicated allocation regions for 45MHz CBW.

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions for the simulations were used:

- Power Class 3

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- Filter rejection at B3 Rx: 50dB

- Filter rejection at B34 Rx: 11dB

The simulations were done for 45MHz channel bandwidth with center frequency of 1942.5MHz and 1957.5MHz with NS_48 and NS_49. The spurious co-existence was according to table 1.

Table1: Co-existence for n1

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n1, n84 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |

###### 2.2 Simulation results

Table 2: Regions for NS_48

| Channel BW (MHz) | Carrier Frequency Fc (MHz) | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 45 | 1942.5 ≤ Fc ≤1957.5 | >=0, <5.76 | >0 | A2 |
|  |  | >=5.76, <19.44 | >=max(0,12*RB_end*SCS-3.6) | A4 |
|  |  | >=5.76, <19.44 | <max(0,12*RB_end*SCS-3.6) >=max(0,12*RB_end*SCS-5.76) | A3 |
|  |  | >=19.44, <38.16 | >=14.4 | A2 |
|  |  | >=30.24, <38.16 | <1.08 | A5 |
|  |  | >=38.16 | >0 | A1 |

Table 3: Regions for NS_49

| Channel BW (MHz) | Carrier Frequency Fc (MHz) | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 45 | 1942.5 ≤ Fc ≤1957.5 | >=7.74, <14.4 | <max(0,RB_end-7.74) | A5 |
|  |  | >=30.96, <35.28 | <1.08 | A5 |
|  |  | <35.28 | >=15.12 <max(0,RB_end-7.74) | A2 |
|  |  | <35.28 | <15.12 >=11.52 < max(0,RB_end-7.74) | A3 |
|  |  | <35.28 | >=max(0,RB_end-7.74) | A1 |
|  |  | >=35.28 | >0 | A1 |

Proposal: Use regions found in Table 2 for NS_48 and regions found in Table 3 for NS_49

# 3  Simulations

###### 3.1 NS_48

###### 3.2 NS_49

# 4	Conclusions

In this contribution we presented simulations results for band n1 with 45MHz CBW and NS_48 and NS_49. Based on the results allocation regions are proposed for A-MPR.

Proposal: Use regions found in Table 2 for NS_48 and regions found in Table 3 for NS_49

# 4	References

- RP-202607, “Introduction of channel bandwidths 35MHz and 45MHz for NR FR1”, Huawei, HiSilicon, 3GPP TSG RAN Meeting #90e