3GPP RAN WG4 Meeting #98-e		R4-2100518

Online, 25th January – 05th February 2021

Agenda item:	9.22.3

Source:	Apple

Title:	A-MPR Proposal for n25 and 45MHz CBW

WI/SI:	NR_FR1_35MHz_45MHz_BW-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The latest revision of the WID introduces 45MHz CBW to band n25 [1]. Simulations have been done for 45MHz channel bandwidth and NS_03.  This paper proposes dedicated A-MPR for NS_03 45MHz CBW.

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions for the simulations were used:

- Power Class 3

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- Coexistence requirements of n25

The simulations were done for 35MHz channel bandwidth with center frequency of 1867.5MHz and 1892.5MHz with NS_03.

Table 1

| Channel BW | Carrier Frequency Fc | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 45 MHz | 1872.5<=Fc <= 1892.5 MHz | >=38.16 | >0 | A1 |
|  |  | >=19.44, <38.16 | >=15.48 | A2 |
|  |  | <19.44 | >=max(0,RB_end_Hz-3.96e6) | A3 |

Table 2

| Waveform | Modulation | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
|  |  | Outer / Inner | Outer / Inner | Outer / Inner |
| DFT-s-OFDM | PI/2 BPSK | 3.5 | 2.5 | 2.0 |
|  | QPSK | 4.0 | 3.0 | 2.5 |
|  | 16QAM | 4.5 | 3.0 | 3.0 |
|  | 64QAM | 4.5 | 3.5 | 3.5 |
|  | 256QAM | 5.5 | 5.5 | 5.5 |
| CP-OFDM | QPSK | 5.5 | 4.5 | 5.0 |
|  | 16QAM | 5.5 | 4.5 | 5.0 |
|  | 64QAM | 5.5 | 4.5 | 5.0 |
|  | 256QAM | 7.5 | 7.5 | 7.5 |

Proposal: Use allocation regions of Table 1 and A-MPR Table 2 for 45MHz CBW NS_03.

# 3  Simulations

# Conclusions

This paper presented simulations results for n25 with 45MHz CBW and NS_03.

Proposal: Use allocation regions of Table 1 and A-MPR Table 2 for 45MHz CBW NS_03.

# 4	References

- RP-202607, “Introduction of channel bandwidths 35MHz and 45MHz for NR FR1”, Huawei, HiSilicon, 3GPP TSG RAN Meeting #90e