3GPP RAN WG4 Meeting #98-e		R4-2100530

Electronic meeting, February 25 – March 5, 2021

Agenda item:	12.1.8.2

Source:	Apple Inc.

Title:	TP to TR38.884 on structure updates related to band n262

WI/SI:	FS_FR2_enhTestMethods

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The TR38.884 [1] can be updated based on the revised SID in [2] and with a number of intermediate agreements reached as part of the discussion of the band n262 testability, as captured in [3].

Proposal 1:	It is proposed to approve the text proposal related to the structure updates related to band n262 testability.

# 2	References

- R4-1913071, “TR38.884 skeleton for FR2 test method enhancement,” Apple Inc., 3GPP RAN4 #92bis, October 2019

- RP-201862, “Revised SID: Study on enhanced test methods for FR2,” Apple Inc., vivo, 3GPP RAN #89, September 2020

- R4-2017599, “WF on testability aspects for the introduction of the new band n262,” Apple, 3GPP RAN4 #97, November 2020

# Text Proposal

<< begin change 1 >>

# 1	Scope

The objectives of this study are to enhance the FR2 RF testing methodology and to quantify the impact of the enhancements on the UE performance, as related to the polarization basis mismatch between the test equipment and UE and to add support for testing under extreme temperature conditions.

The development of testing methodology enhancements proceeds within the following scope:

-	In general

-	Target the testing and calibration aspects of the permitted methods for FR2 UE RF testing and the preliminary assessment of measurement uncertainty (Clause 5.2 and Annex B of TR38.810)

- 	The test methodologies and procedures shall be applicable for different device types and power classes with DUT size defined in the TR 38.810.  Prioritize the study to PC3 for aspects related to DUT size, and limit the study to free space conditions

-	The study item outcomes shall capture the efficacy of the enhancements

-	Objectives related to regulatory test cases shall be prioritized

The detailed objectives are:

1.	Define test methodology for high DL power and low UL power test cases

-	Considering path loss reduction, measurement antenna gain improvement, DUT positioning improvement, and MU improvement

-	Considering NFTF (defined in Clause 5.2 of TR38.810) and direct near field test methodologies as possible alternative methods

-	Other approaches are not precluded

-	Study preliminary assessment of measurement uncertainty of new alternative methods

2.	Define solutions to minimize the impact of polarization basis mismatch between the TE and DUT on the RF testing

-	Considering polarization basis mismatch between the test equipment and UE and UE implementations which may be impacted by this mismatch

-	Study EIS test metric which can apply to different UE RF implementations considering downlink polarization sweep enhancement

- 	Limit the study of this objective to the permitted UE RF methods defined in Clause 5.2 of TR38.810

-	Possible enhancements may be described as

-	Downlink polarization sweeping by the test equipment (i.e. introducing an additional degree of freedom for polarization alignment of the measurement antenna)

-	The use of circular polarization to perform measurements

-	Coherent combining and demodulation of orthogonally polarized received signals in the test equipment

-	Uplink polarization sweeping by the test equipment to search for the optimal polarization angle to receive and demodulate the signal transmitted by the UE

-	Considering NFTF (defined in Clause 5.2 of TR38.810) test methodology for EIS measurement

-	TPMI side condition method, where TPMI side conditions are applicable to Rel-16 (and higher) UEs

-	Test mode to trigger TX diversity

-	Other approaches are not precluded

3.	Study testability enhancements to support the verification of RF requirements for inter-band (FR2+FR2) CA

-	Work on inter-band DL CA is prioritized

-	Whether the test setup shall be restricted to emulating the signal from the same direction for the aggregated bands shall be aligned with the UE RF architecture assumption taken in the work item on NR RF Requirement Enhancements for FR2 [UID 830189]

4.	Support extreme temperature conditions for all applicable FR2 UE RF test cases

-	Considering beam peak search, spherical coverage, and total radiated power procedures

- 	Limit the study of this objective to the permitted UE RF methods defined in Clause 5.2 of TR38.810

-	Study preliminary impacts on system measurement uncertainty under extreme temperature conditions

5.	Study testability enhancements to support the verification of RF requirements for FR2 DL 256QAM

6.	Study testability enhancements to reduce test time

-	Including RF test method enhancement with reduced test time, and possible test time saving approach for UE Demodulation test and RRM test

7.	Study testability aspects for the introduction of the new band n262

-	Considering the extension of frequency applicability of the permitted methods in TR38.810 from 43.5 GHz up to at least 48.2 GHz

-	Considering the extension of frequency applicability of the test methodology enhancements in Objectives 1 through 6 above

<< end change 1 >>

<< begin change 2 >>

## 5.6	Extension of frequency applicability for band n262

Editor’s note: outcome of SI Objective 7 related to the extension of frequency applicability of the permitted methods is captured in this clause

<< end change 2 >>

<< begin change 3 >>

# 6	UE RRM testing methodology enhancements

Editor’s note: testing and calibration aspects of the permitted methods for FR2 UE RRM testing and the preliminary assessment of measurement uncertainty (Clause 6.2 and Annex B.2 of TR38.810) define the baseline UE RRM methodology for the purpose of this study.

## 6.1	Extension of frequency applicability for band n262

Editor’s note: outcome of SI Objective 7 related to the extension of frequency applicability of the permitted methods is captured in this clause

# 7	UE demodulation testing methodology enhancements

Editor’s note: testing and calibration aspects of the permitted methods for FR2 UE demodulation testing and the preliminary assessment of measurement uncertainty (Clause 7.2 and Annex B.2 of TR38.810) define the baseline UE demodulation methodology for the purpose of this study.

## 7.1	Extension of frequency applicability for band n262

Editor’s note: outcome of SI Objective 7 related to the extension of frequency applicability of the permitted methods is captured in this clause

<< end change 3 >>

<< begin change 4 >>

# 8	Test time reduction

Editor’s note: outcome of SI Objective 6 is captured in this clause. This objective may potentially impact RF, RRM, and demodulation test setups.

<< end change 4 >>