3GPP TSG RAN WG4 Meeting #98-e	R4-2100690

Electronic Meeting, 25 Jan. - 5 Feb., 2021

Agenda item:	9.13.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-31: EN-DC_1-8-11_n3-n28-n77

Document for:	Approval

1	Introduction

EN-DC of 3B LTE and 3B NR of DC_1-8-11_n3-n28-n77 was approved in RAN#89 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

8.X	DC_1-8-11_n3-n28-n77

8.X.1	Operating bands for DC

Table 8.x.1-1: EN-DC band combination (six bands)

| EN-DC Band | E-UTRA CA Band | NR CA Band |
| --- | --- | --- |
| DC_1-8-11_n3-n28-n77 | CA_1-8-11 | CA_n3-n28-n77 |

8.X.2	Inter-band DC Configurations

Table 8.x.2-1: Inter-band EN-DC configurations (six bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-8A-11A_n3A-n28A-n77A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |

8.X.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR3DL/1UL are already covered by the fallback configurations, DC_1-8-11_n3-n28, DC_1-8-11_n3-n77, DC_1-8-11_n28-n77, DC_1-8_n3-n28-n77, DC_1-11_n3_n28-n77 and DC_8-11_n3-n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.X.4	∆TIB and ∆RIB values

For DC_1-8-11_n3-n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

8.X.5	MSD

As mentioned in 8.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]

3.  Reference

[1]  RP-201571  Rel-17 Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL), ZTE