3GPP TSG-RAN WG4 Meeting #98-e		                               R4-2100703

Electronic Meeting, 25 Jan – 05 Feb, 2021

Source: 	Murata Manufacturing Co Ltd.

Title: 	REFSENS of n8 and n71 for 35MHz channel bandwidth

Agenda Item:	9.22.3	UE RF requirements	[NR_FR1_35MHz_45MHz_BW-Core]

Document for:	Approval

Introduction

In the RAN4#97e meeting, there is no consensus on UL configuration and channel location for MSD analysis for n8, n71 at CBW 35MHz. But there is the new WF [1] be approved that encourage participants in the discussion to study further scenario what UL BW is limited 20MHz.

In this contribution, we provide the Tx noise measurement results by UL/DL BW = 20/35MHz (UL/DL asymmetric operation), MSD analysis and REFSENS for n8 and n71.

Discussion

WF Review

In the WF [1], the following option1 is chosen for n8, n71 MSD analysis.

- Option1: UL BW is limited to 20 MHz

  - No need for DMPR, NS_43 and NS_35 A-MPR and associated SEM requirements

  - No issue with other band protection

  - Low MSD can be specified using current 20MHz UL configuration:

  - Option 1a: Worst case 20MHz UL closest to DL

  - Option 1b: Best case 20MHz UL furthest to DL

  - Option 1c: both 1b and 1c are specified

  - Limited impact to band combinations

Power Amplifier Calibration and Test Waveforms

PA conditions:

The power amplifier (PA) is calibrated to meet 1 dB MPR for Power Class 3 using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge.

Test conditions:

- QPSK DFT-s-OFDM SCS 15kHz with the UL RBs be located as close as possible to the DL operating band but confined within the transmission bandwidth configuration for the CBW;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Counter-IM3 (C-IM3) set to -60 dBc;

- Counter-IM5 (C-IM5) set to -70 dBc;

Measurement Results

Since reduced UL Resource block allocation does no help to improve the Rx band noise (RxBN) of PA [2], we still use 20RB that is the current specification for 20MHz in measurement.

The measurement results are shown in Table 2-1 and Table 2-2.

Table 2-1 n8 RxBN measurement results

|  | Tx freq. | Rx freq. | Tx/Rx BW | Tx/Rx Gap | RB alloca. | RxBN |
| --- | --- | --- | --- | --- | --- | --- |
|  | (MHz) | (MHz) | (MHz) | (MHz) | (-) | (dBm/BW) |
| Ref. | 897.5 | 942.5 | 35/35 | 10 | 20@168 | -12.2 |
| Option 1a | 905 | 942.5 | 20/35 | 10 | 20@86 | -14.0 |
| Option 1b | 890 | 942.5 | 20/35 | 25 | 20@86 | -37.9 |

Table 2-2 n71 RxBN measurement results

|  | Tx freq. | Rx freq. | Tx/Rx BW | Tx/Rx Gap | RB alloca. | RxBN |
| --- | --- | --- | --- | --- | --- | --- |
|  | (MHz) | (MHz) | (MHz) | (MHz) | (-) | (dBm/BW) |
| Ref. | 680.5 | 634.5 | 35/35 | 11 | 20@0 | -12.6 |
| Option 1a | 673 | 634.5 | 20/35 | 11 | 20@0 | -14.1 |
| Option 1b | 688 | 634.5 | 20/35 | 26 | 20@0 | -37.8 |

Observation 1:

Compared to the reference condition UL/DL BW = 35 / 35MHz, RxBN is improved less than 2dB at Option 1a, while more than 25dB at Option 1b.

In previous measurement of Ref., IM3 of Tx signal and LO Leakage and IM3 of Tx signal and Tx image falling into the Rx band are the main causes of RxBN degradation [3]. In this measurement, IM3 of Tx signal and LO leakage shifts to the lower frequency and falls into the Tx-Rx gap at Option 1a. On the other hand, in addition to IM3 of Tx signal and LO Leakage, IM3 of Tx signal and Tx image also falls into the Tx-Rx gap at Option 1b due to wider Tx-Rx gap. This is the reason why RxBN is not improved much even if UL BW was limited to 20MHz at Option 1a.

MSD Estimation

The link-budget calculation assumptions and MSD analysis are shown in table 3-1.

Table 3-1 MSD analysis for n8 and n71

| Parameter | Unit | n8 |  |  |  |  |  | n71 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ref. |  | Option 1a |  | Option 1b |  | Ref. |  | Option 1a |  | Option 1b |  |
| Tx/Rx BW | MHz | 35/35 |  | 20/35 |  | 20/35 |  | 35/35 |  | 20/35 |  | 20/35 |  |
| Antenna isolation | dB | 10.0 |  | 10.0 |  | 10.0 |  | 10.0 |  | 10.0 |  | 10.0 |  |
| Front-end loss | dB | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  |
| PA output power | dBm | 27.0 |  | 27.0 |  | 27.0 |  | 27.0 |  | 27.0 |  | 27.0 |  |
| PA RxBN | dBm | -12.2 |  | -14.0 |  | -37.9 |  | -12.6 |  | -14.1 |  | -37.8 |  |
| DPX_ISO_from Tx to Rx | dB | 50.0 |  | 50.0 |  | 50.0 |  | 55.0 |  | 55.0 |  | 55.0 |  |
| DPX ATT RX band | dB | 30.0 |  | 30.0 |  | 30.0 |  | 45.0 |  | 45.0 |  | 45.0 |  |
| Div_filter_Tx rejection | dB | 45.0 |  | 45.0 |  | 45.0 |  | 40.0 |  | 40.0 |  | 40.0 |  |
| Transceiver effective phase noise | dBc/Hz | -144.0 |  | -144.0 |  | -144.0 |  | -144.0 |  | -144.0 |  | -144.0 |  |
| Thermal noise at PRx/DRX ANT port | dBm/Hz | -164.0 |  | -164.0 |  | -164.0 |  | -164.0 |  | -164.0 |  | -164.0 |  |
| SNR requirement for QPSK | dB | -1.0 |  | -1.0 |  | -1.0 |  | -1.0 |  | -1.0 |  | -1.0 |  |
| TX noise power at PRX/DRx ANT port | dBm | -58.2 | -58.2 | -60.0 | -60.0 | -83.9 | -83.9 | -63.6 | -63.6 | -65.1 | -65.1 | -88.8 | -88.8 |
| TX IM2 due to wider CBW at PRx/DRx ANT port | dBm | -87.0 | -105.0 | -87.0 | -105.0 | -87.0 | -105.0 | -97.0 | -95.0 | -97.0 | -95.0 | -97.0 | -95.0 |
| Thermal noise at PRx/DRx ANT Port | dBm | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 |
| Total noise level at PRx/DRx ANT Port | dBm | -59.2 | -59.2 | -61.0 | -61.0 | -81.6 | -82.7 | -64.6 | -64.6 | -66.1 | -66.1 | -84.8 | -84.6 |
| De-sens. | dB | 26.5 |  | 24.7 |  | 3.5 |  | 21.1 |  | 19.7 |  | 1.0 |  |

Proposal 1:

REFSENS of Option 1a are as follows.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n8 | 15 | -97 | -93.8 | -91.4 | -85.8 |  |  | -64.0 |  |  |  | FDD |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  | -64.1 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 |  |  | -69.1 |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  | -69.2 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE x: UL BW is limited to 20MHz, and UL channel shall be closest to DL channel within operating band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n8 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2:

REFSENS of Option 1b are as follows.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n8 | 15 | -97 | -93.8 | -91.4 | -85.8 |  |  | -85.2 |  |  |  | FDD |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  | -85.3 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 |  |  | -87.7 |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  | -87.8 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE x: UL BW is limited to 20MHz, and UL channel shall be farthest from DL channel within operating band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n8 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

Conclusion

In this contribution, we present PA Rx band noise in NR band n8, n71 at UL/DL BW = 20/35MHz and propose the following reference sensitivity levels and UL configurations.

Observation 1:

Compared to the reference condition UL/DL BW = 35 / 35MHz, RxBN is improved less than 2dB at Option 1a, while more than 25dB at Option 1b.

Proposal 1:

REFSENS of Option 1a are as follows.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n8 | 15 | -97 | -93.8 | -91.4 | -85.8 |  |  | -64.0 |  |  |  | FDD |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  | -64.1 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 |  |  | -69.1 |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  | -69.2 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE x: UL BW is limited to 20MHz, and UL channel shall be closest to DL channel within operating band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n8 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2:

REFSENS of Option 1b are as follows.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n8 | 15 | -97 | -93.8 | -91.4 | -85.8 |  |  | -85.2 |  |  |  | FDD |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  | -85.3 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 |  |  | -87.7 |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  | -87.8 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE x: UL BW is limited to 20MHz, and UL channel shall be farthest from DL channel within operating band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n8 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |

References

[1]		R4-2016863, WF on general aspects for UE RF requirements, Skyworks Solutions Inc., Apple

[2]		R4-2016011, n8 35MHz A-MPR and MSD Measurements, Skyworks Solutions Inc.

[3] 	R4-2015432, REFSENS of n3, n8, n25 and n71 for new channel bandwidth, Murata Manufacturing Co Ltd.