3GPP TSG-RAN WG4 Meeting # 98-e	R4-2100839

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source:	ZTE Corporation

Title:	On test cases for cell reselection under NR-U

Agenda item:	7.1.6.3.2

Document for:	Discussion

# 1	Introduction

In RAN4 97-e, the first meeting to discuss performance part of NR-U, companies has some preliminary discussions on the scope of test cases, test configurations and the work plan. During the meeting, whether to define test cases for some of the scenarios of cell reselection was discussed. Companies had different views on the scope test cases while some consensus was captured in the WF [1] as shown below.

Issue 3-3-2: Cell Reselection Tests For Cell reselection, at least the following test cases are to be specified in NR-U - Cell reselection to FR1 intra-frequency NR when CCA is used on the serving and target cell - Cell reselection to FR1 inter-frequency NR when CCA is used on the serving and target cell - Cell reselection from E-UTRAN (FDD and TDD) to FR1 when CCA is used on the target cell Other cases discussed in RAN4 97e: - Cell reselection from NR-U cell to NR (NR-U -> NR(FR1) - Cell reselection from NR cell to NR-U (NR(FR1) -> NR-U) - Cell reselection from NR-U cell to E-UTRAN (NR-U - > E-UTRAN (FDD,TDD))

- This paper provides our view on this topic.

2	Discussion

For cell reselection, some companies suggested not to define test cases for case 4, 5 and 6. The proposal was backed by the argument that case 4, 5, 6 can be functionally verified by the cases 1-3 which already reflect the UE capability and essential functions and if the UE can pass the tests for cases 1-3 then the UE is assumed to be able to meet requirements for cases 4-6 as well, and thus, no need to specify test cases for cases 4-6.

We don’t agree with the above argument and think that test cases for cases 4-6 shall be specified.

If the UE can be assumed to meet requirements for cases 4-6 conditioned that the UE passes tests for cases 1-3, then it doesn’t mean much burden for the UE to also be tested for cases 4-6 since no extra implementation is required.

RAN4 shall define test cases for cases 1-6 under cell re-selection.

3	Conclusion

Proposal 1: RAN4 shall define test cases for cases 1-6 under cell re-selection.

References

- R4-2017089