3GPP TSG-RAN WG4 Meeting #98-e                                            	R4-2100897

Electronic Meeting, Jan.25-Feb.5, 2021

Source:	Samsung

Title:	Discussion and simulation results for Rel-15 Type II codebook test cases

Agenda item:		7.16.1.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, Rel-15 Type II codebook requirements with SU-MIMO was agreed to set up with proper test parameters, test metric or test requirements, and test procedure to ensure enough performance gain over than Type I. Meanwhile, MU-MIMO scenario should postpone to Rel-17 WI for further study if necessary. It is difficult to align with random PMI for Rel-15 Type II and Rel-16 eType II, as a result new test metric will be considered to define the requirements. The test metric redefined as:

- Following PMI (Type II)/Random PMI (Type I codebook) (gamma values)  based on the assumption that there are ensure enough performance difference over than Type I i.e. UE which employ Type I reporting will fail in the test case

  - This test metric applied to UE which support Type II codebook feedback irrespective whether supporting Type I codebook feedback or not

- FFS: Whether to check UE reported codebook not only within Type I codebook set

Besides introducing new test metric, “Custom low correlation” is also evaluated in the following meetings.

In this contribution, simulation results for different test metrics will be evaluated and views on test set-up will be also presented.

- Simulation results and discussion

  - Simulation results for R15 Type II codebook

In the following figures, we evaluated performance under following PMI and random PMI with Type I and Type II codebook constructions under XP Medium channel correlation and XP Custom Low correlation with 64QAM rank2.

- Simulation results for FDD

Fig.1 Throughput performance of FDD 16x2 XP Medium

Fig.2 Throughput performance of FDD 16x4 XP Medium

Fig.3 Throughput performance of FDD 16x2 XP Custom Low

Fig.4 Throughput performance of FDD 16x4 Custom Low

The SNR point at 70%,90%,and 95% of full throughput with different test metric are summarized in the following tables respectively, in which the TP ratio with marked yellow is newly added in WF [1] ：

Table 2.1: SNR point and TP ratio at 70% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 9.4dB/2.55 | 6.8dB/1.93 | 7.9dB/2.62 | 4.3dB/2.30 |
| Type II (following R15 Type II PMI / Random Type I PMI) | 6.8dB/4.18 | 5.2dB/2.51 | 6.6dB/3.72 | 4.0dB/2.41 |

Table 2.2: SNR point and TP ratio under 90% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 13.2dB/2.10 | 8.8dB/2.01 | 10.8dB/2.37 | 6.0dB/2.32 |
| Type II (following R15 Type II PMI / Random Type I PMI) | 9.5dB/3.18 | 7.6dB/2.24 | 9.0dB/2.87 | 5.8dB/2.38 |

Table 2.3: SNR point and TP ratio under 95% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 14.9dB/1.9 | 9.6dB/1.98 | 12.0dB/2.16 | 7.1dB/2.19 |
| Type II (following R15 Type II PMI / Random Type I PMI) | 10.8dB/2.88 | 8.6dB/2.01 | 9.9dB/2.65 | 6.9dB/2.27 |

- Simulation results for TDD

Fig.5 Throughput performance of TDD 16x2 XP Medium

Fig.6 Throughput performance of TDD 16x4 XP Medium

Fig.7 Throughput performance of TDD 16x2 Custom Low

Fig.8 Throughput performance of TDD 16x4 Custom Low

The SNR point at 70%,90%,and 95% of full throughput with different test metric are summarized in the following tables respectively, in which the TP ratio with marked yellow is newly added in WF [1] ：

Table 2.4: SNR point and TP ratio under 70% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 10.0dB/1.86 | 6.9dB/1.70 | 8.0dB/1.98 | 5.4dB/1.74 |
| Type II (following R15 Type II PMI / Random Type I PMI) | 7.8dB/2.43 | 5.7dB/1.96 | 6.4dB/2.51 | 4.8dB/1.90 |

Table 2.5: SNR point and TP ratio under 90% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 13.2dB/1.83 | 9.1dB/1.76 | 11.1dB/1.83 | 7.6dB/1.78 |
| Type II (following R15 Type II PMI / Random Type I PMI) | 11.3dB/2.12 | 8.2dB/1.91 | 9.3dB/2.23 | 7.8dB/1.76 |

Table 2.6: SNR point and TP ratio under 95% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 14.7dB/1.68 | 9.9dB/1.73 | 12.0dB/1.79 | 8.6dB/1.71 |
| Type II (following R15 Type II PMI / Random Type I PMI) | 13.3dB/1.87 | 9.6dB/1.81 | 10.8dB/2.03 | 9.3dB/1.64 |

  - Summary for results

Observation 1-MIMO Correlation: It is observed that the performance gain with Type II compared to Type I codebook in XP medium MIMO correlation is larger than in Custom Low correlation cases. Especially for 16x2 ‘Custom Low’, there is marginal gain for Type II codebook.

Observation 2-Test Metric: As shown in table 2.1~2.3, the TP ratios are reasonable and the SNR points are workable under the “following Type II PMI vs. random Type I PMI” test metrics.

Observation 3-Test point: The performance gap between following Type II and Type I is more obvious under 70% relative TP point than 90%, and 95% points.

Observation 4- Performance gap between Type II and Type (XP-Medium with 70% relative test point):

- 2.5dB around SNR gap for 2Rx cases, 1.5dB around SNR gap for 4Rx cases

Proposal 1: Introduce Type II codebook test cases

- SU-MIMO set-up

- MIMO correlation: XP Medium MIMO correlation

- Test metric: Relative TP between following Type-II/random PMI with Type I codebook

- Test point: relative 70%  TP with following PMI

For test metric, additional test metric can be considered to ensure UE following Type I codebook reporting fail the test cases i.e.

For UE which also support Type I codebook with 16Tx, the absolute throughput achieved at SNR test point with Type II codebook configuration need to be larger the absolute TP achieved with Type I codebook reporting under same test set-up.

- Conclusion

In this contribution, we provide simulation results for Type II and Type I codebook based on the agreements in RAN4 #97e meeting. We compare different test metrics under Type I and Rel-15 Type II cases, from the above results, we have following proposals:

Proposal 1: Introduce Type II codebook test cases

- SU-MIMO set-up

- MIMO correlation: XP Medium MIMO correlation

- Test metric: Relative TP between following Type-II/random PMI with Type I codebook

- Test point: relative 70% TP with following PMI

Reference

[1] R4-2017681,	“Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, Ericsson, Samsung, 3GPP RAN4 #97-e meeting

[2] R4-2017563,	“Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports”, Ericsson, 3GPP RAN4 #97-e meeting

[3] R4-2014746, “Views and simulation results for Rel-15 Type II PMI test case”, Samsung, 3GPP RAN4 #96-e meeting

Annex: Simulation configurations

The simulation configurations are proposed in [2].

Table A.1   FDD test configurations

| Parameter |  | Unit | Test 5.1 | Test 5.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | ‘Medium’ or ‘Custom Low’ XP 16 x 2 (N1,N2) = (4,2) | ‘Medium’ or ‘Custom Low’ XP 16 x 4 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.2.3B.4A of TS36.101 -Relative power ratio among two beams can be fixed as 1 (p=1) |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 16 |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Subband |  |
| Sub-band Size |  | RB | 8 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 5 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | TypeII |  |
|  | L (numberOfBeams) |  | [2] |  |
|  | NPSK (phaseAlphabetSize) |  | [8] |  |
|  | subbandAmplitude |  | True |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). |  |  |  |  |

Table A.2 TDD test configurations

| Parameter |  | Unit | Test 6.1 | Test 6.2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | ‘Medium’ or ‘Custom Low’ XP [16 x 2 (N1,N2) = (4,2)] | Medium’ or ‘Custom Low’ XP [16 x 4 (N1,N2) = (4,2)] |
| Beamforming Model |  |  | As specified in Annex B.2.3B.4A of TS36.101 -Relative power ratio among two beams can be fixed as 1 (p=1) |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | [16] |  |
|  | CDM Type |  | CDM4 (FD2, TD2) |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | [(Row 12, (2, 4, 6, 8)] |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | [(5,-)] |  |
|  | CSI-RS interval and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Subband |  |
| Sub-band Size |  | RB | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 8 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | Type II |  |
|  | L (numberOfBeams) |  | [2] |  |
|  | NPSK (phaseAlphabetSize) |  | [8] |  |
|  | subbandAmplitude |  | True |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | [(4,2)] |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF FFFF FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 6.5 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). |  |  |  |  |