3GPP TSG-RAN4 Meeting #98-e	R4-2100932

# Electronic Meeting, 25th Jan - 5th Feb, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0272 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on MsgA PUSCH radiated performance requirement for TS 38.141-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Perf |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MsgA PUSCH requirements have been introduced for Rel-16 NR 2-step RACH |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the note for different TDD pattern Correction the FRC and channel conditional for test requirement Minor modification with adding the reference Add the missing clause number Add the 2-step RA type in title of test requirement Update the CBW for AWGN level setting Update the requirement based on the latest simulation summary Remove the [] for requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-16 NR 2-step RACH requirement can not be verifed properly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

### 8.2.9	Performance requirements for MsgA PUSCH

#### 8.2.9.1	Definition and applicability

The performance requirement of MsgA PUSCH is determined by a maximum allowed block error rate of Msg A received by BS at given SNR for FRCs listed in annex A. The performance requirements assume that the precedent preamble of MsgA is correctly detection. The performance requirements of assume no HARQ retransmission.

These requirements are applicable for wide area and medium range BS that support 2-step RA type. The requirements are not applied for a local area BS that supports 2-step RA type.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in clause 8.1.2.1.

#### 8.2.9.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2] clause 11.2.1.9.

For BS type 2-O, the minimum requirement is in TS 38.104 [2] clause 11.2.1.4.

#### 8.2.9.3	Test Purpose

The test shall verify the receiver's ability to detect MsgA PUSCH under multipath fading propagation conditions for a given SNR.

#### 8.2.9.4	Method of test

##### 8.2.9.4.1	Initial Conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

##### 8.2.9.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.9.4.2-1.

Table 8.2.9.4.2-1: Test parameters for testing MsgA PUSCH for 2-step RA type

| Parameter |  |  | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- | --- |
| Preamble format |  |  | Free choice |  |
| Transform precoding |  |  | Disable |  |
| Power offset between preamble and MsgA(Note 1) |  |  | Free choice |  |
| Default TDD UL-DL pattern(Note 2) |  |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions |  | 1 |  |
|  | RV sequence |  | 0 |  |
| DM-RS | DM-RS configuration type |  | 1 |  |
|  | DM-RS duration |  | single-symbol DM-RS |  |
|  | Additional DM-RS position |  | Pos 1 or Pos 2 (Note 3) | Pos1 |
|  | Number of DM-RS CDM group(s) without data |  | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |  |
|  | DM-RS port(s) |  | 0 |  |
|  | DM-RS sequence generation |  | NID0=0, group hopping and sequence hopping are disabled |  |
| Time | PUSCH mapping type |  | A, B | B |
| domain | Start symbol |  | 0 | 0 |
| resource assignment | Allocation length |  | 14 | 10 |
| Frequency domain resource assignment | RB assignment |  | 2 | 2 |
|  | Starting PRB index |  | 0 | 0 |
|  | Frequency hopping |  | Disabled |  |
| Code block group based PUSCH transmission |  |  | Disabled |  |
| PT-RS |  |  | Not configured |  |
| Timing offset (TO) Cycling (us) |  |  | 15 kHz SCS: 0:0.2:3.8 30 kHz SCS: 0:0.1:2 | 60 kHz SCS: 0:0.1:0.6 120 kHz SCS: 0:0.1:0.5 |
| NOTE 1: The power ratio between preamble and msgA is set to be sufficient to achieve 100% preamble detection. The SNR for the requirement is defined on the msgA PUSCH NOTE 2: The same requirements are applicable to FDD and TDD with different UL-DL patterns NOTE 3: For FR1, either pos 1 or pos 2 may be used for the test FRC. A pass with either of these possibilities is sufficient to demonstrate compliance to the core requirement. |  |  |  |  |

6)	The channel emulators shall be configured according to the corresponding channel model defined in annex J. Unless stated otherwise, the MIMO correlation matrices for the gNB are defined in annex G for low correlation.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in clause 8.2.9.5.1 and 8.2.9.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.2.9.4.2-2.

Table 8.2.9.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  | 30 | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
| BS type 2-O | 60 | 50 | EISREFSENS_50M + ΔFR2_REFSENS + 15 dBm / 47.52MHz |
|  | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	The test signal generator sends a MsgA including a preamble and PUSCH signal and the receiver tries to detect the MsgA signal contents. The MsgA signal is sent with timing offset as described below. The following statistics are collected: the number of incorrectly decoded MsgA PUSCH transmitted during the MsgA signal transmission.

The timing offset base value for MsgA signal with 15 kHz SCS is set to 0. This offset is increased within the loop, by adding in each step a value of 0.2 us for BS type 1-O with 15 kHz SCS, until the end of the tested range, which is 3.8 us. At the end of the testing range, the offset is reset to zero. The timing offset scheme for MsgA transmission is presented in Figure 8.2.9.4.2-1.

Figure 8.2.9.4.2-1: Timing offset scheme for MsgA transmission for BS type 1-O with 15 kHz SCS

The timing offset base value for MsgA signal with 30 kHz is set to 0. This offset is increased within the loop, by adding in each step a value of 0.1 us for BS type 1-O with 30 kHz SCS, until the end of the tested range, which is 2 us. At the end of the testing range, the offset is reset to zero. The timing offset scheme for MsgA transmission is presented in Figure 8.2.9.4.2-2.

Figure 8.2.9.4.2-2: Timing offset scheme for MsgA transmission for BS type 1-O with 30 kHz SCS

The timing offset base value for MsgA signal with 60 kHz is set to 0. This offset is increased within the loop, by adding in each step a value of 0.1 us for BS type 2-O with 60 kHz SCS, until the end of the tested range, which is 0.6 us. At the end of the testing range, the offset is reset to zero. The timing offset scheme for MsgA transmission is presented in Figure 8.2.9.4.2-3.

Figure 8.2.9.4.2-3: Timing offset scheme for MsgA transmission for BS type 2-O with 60 kHz SCS

The timing offset base value for MsgA signal with120 kHz is set to 0. This offset is increased within the loop, by adding in each step a value of 0.1 us for BS type 2-O with 120 kHz SCS, until the end of the tested range, which is 0.5 us. At the end of the testing range, the offset is reset to zero. The timing offset scheme for MsgA transmission is presented in Figure 8.2.9.4.2-4.

Figure 8.2.9.4.2-4: Timing offset scheme for MsgA transmission for BS type 2-O with 120 kHz SCS

#### 8.2.9.5	Test Requirement

##### 8.2.9.5.1	Test Requirement for BS type 1-O

The block error rate of MsgA PUSCH for the reference measurement channel as specified in Annex A at the SNR given in table 8.2.9.5.1-1 to table 8.2.9.5.1-4 shall not exceed 1%.

Table 8.2.9.5.1-1: Test requirements for MsgA PUSCH for 2-step RA type, Type A, 15 kHz SCS

| Number of TX antennas | Number of RX demodulation branches | Cyclic prefix | Propagation conditions (Annex G) | BLER | FRC (Annex A) | Time offset (Note 1) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 | 1% | G-FR1-A8-1, or G-FR1-A8-3 | 0, 0.2, 3.8 | 7.9 |
| Note 1: The time offset values are described as X, Y, Z where X is the first TO value, Y is the step in which the TO should be incremented, and Z is the largest TO value in the range. |  |  |  |  |  |  |  |

Table 8.2.9.5.1-2: Test requirements for MsgA PUSCH for 2-step RA type, Type A, 30 kHz SCS

| Number of TX antennas | Number of RX demodulation branches | Cyclic prefix | Propagation conditions (Annex G) | BLER | FRC (Annex A) | Time offset (Note 1) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 | 1% | G-FR1-A8-2, or G-FR1-A8-4 | 0, 0.1, 2.0 | 7.7 |
| Note 1: The time offset values are described as X, Y, Z where X is the first TO value, Y is the step in which the TO should be incremented, and Z is the largest TO value in the range. |  |  |  |  |  |  |  |

Table 8.2.9.5.1-3: Test requirements for MsgA PUSCH for 2-step RA type, Type B, 15 kHz SCS

| Number of TX antennas | Number of RX demodulation branches | Cyclic prefix | Propagation conditions (Annex G) | BLER | FRC (Annex A) | Time offset (Note 1) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 | 1% | G-FR1-A8-1, or G-FR1-A8-3 | 0, 0.2, 3.8 | 7.6 |
| Note 1: The time offset values are described as X, Y, Z where X is the first TO value, Y is the step in which the TO should be incremented, and Z is the largest TO value in the range. |  |  |  |  |  |  |  |

Table 8.2.9.5.1-4: Test requirements for MsgA PUSCH for 2-step RA type, Type B, 30 kHz SCS

| Number of TX antennas | Number of RX demodulation branches | Cyclic prefix | Propagation conditions (Annex G) | BLER | FRC (Annex A) | Time offset (Note 1) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 | 1% | G-FR1-A8-2, or G-FR1-A8-4 | 0, 0.1, 2.0 | 8.2 |
| Note 1: The time offset values are described as X, Y, Z where X is the first TO value, Y is the step in which the TO should be incremented, and Z is the largest TO value in the range. |  |  |  |  |  |  |  |

##### 8.2.9.5.2	Test Requirement for BS type 2-O

The block error rate of MsgA PUSCH for the reference measurement channel as specified in Annex A at the SNR given in table 8.2.9.5.2-1 to table 8.2.9.5.2-2 shall not exceed 1%.

Table 8.2.9.5.2-1: Test requirements for MsgA PUSCH for 2-step RA type, Type B, 60 kHz SCS

| Number of TX antennas | Number of RX demodulation branches | Cyclic prefix | Propagation conditions (Annex G) | BLER | FRC (Annex A) | Time offset (Note 1) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 | 1% | G-FR2-A3-25 | 0, 0.1, 0.6 | 9.3 |
| Note 1: The time offset values are described as X, Y, Z where X is the first TO value, Y is the step in which the TO should be incremented, and Z is the largest TO value in the range. |  |  |  |  |  |  |  |

Table 8.2.9.5.2-2: Test requirements for MsgA PUSCH for 2-step RA type, Type B, 120 kHz SCS

| Number of TX antennas | Number of RX demodulation branches | Cyclic prefix | Propagation conditions (Annex G) | BLER | FRC (Annex A) | Time offset (Note 1) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 | 1% | G-FR2-A3-26 | 0, 0.1, 0.5 | 8.9 |
| Note 1: The time offset values are described as X, Y, Z where X is the first TO value, Y is the step in which the TO should be incremented, and Z is the largest TO value in the range. |  |  |  |  |  |  |  |

<End of Change>