3GPP TSG-RAN WG4 Meeting #98-e                                                           R4-2100979

Online, 25th Jan - 5th Feb, 2021

Source:	Samsung, TELUS, Bell mobility

Title:	TP for TR 38.717-03-01: CA_n13-n25-n66

Agenda item:			9.9.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-01 to include CA_n13-n25-n66 according to the request in [1].

2. Reference

- RP-202720, Revised WID: Rel-17 NR inter-band CA for 3 bands DL with 1 band UL.

# 3. Text Proposal

<Start of Text Proposal>

6.x	CA_n13-n25-n66

6.x.1	Operating bands for CA

Table 6.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n13-n25-n66 | n13 | 777 MHz | – | 787 MHz | 746 MHz | – | 757 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n13A-n25A-n66A | - | n13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

6.x.3	Co-existence studies

Co-existence studies of CA_n13-n25-n66 with 2UL are already covered in the constituent fall-back modes.

6.x.4	∆TIB,c and ∆RIB,c values

For CA_n13-n25-n66, the TIB,c and RIB,c values can reuse DC_2-13_n66 defined in TS 38.101-3.

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n13-n25-n66 | n13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 6.X.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n13-n25-n66 | n13 | 0 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |

6.x.5	REFSENS requirements

MSD requirements are captured in lower order combinations.

<End of Text Proposal>