3GPP TSG-RAN WG4 Meeting #98-e	R4-2100996

Electronic meeting, 25th January – 5th Feburary, 2021

Source:	Ericsson

Title:	Discussion on remain issue of NR-U PDSCH demodulation requirements

Agenda item:	7.1.7.2

Document for:	Discussion

# Introduction

In RAN4#97-e meeting, companies got some agreements on PDSCH demodulation assumptions and also many open issues are left [1].

- Agreements

  - Use 20 MHz LBT BW (for channel sensing and clear channel assessment).

  - Do not multiplex SSB and Data.

  - Define tests for TDD 30kHz only.

  - Define tests with low delay spread and low doppler speed propagation channel.

  - Q factor for SSB: NSSBQCL  = 8

- Open issues

  - Test design

    - Option 1: Use NR PDSCH Demod Performance Tests as a starting point (R16 NR CA for Scenario A, R15 PDSCH for Scenario C);

    - Option 2: Use LTE LAA Demod Performance Test setup as a starting point for scenario A

  - PDSCH Type

    - Option 1: Type A only;

    - Option 2: Type B only for UE with capability, Type A otherwise;

    - Option 3: Type A plus Rel-15 Type B for partial slots, dedicated test to verify typeB-PDSCH-length-r16 capability with corresponding applicability rule;

  - PDSCH Type B Start and Length for PDSCH Performance Tests (if agreed to use PDSCH mapping Type B)

    - Option 1: Subset of fixed [start, length] values: [2,4], [2,12];

    - Option 2: Random start in Symbol {2, 9} and length {12, 5} for first slot of burst. Start in Symbol 2 and length {4, 7, 10, 12} for last slot of burst.  Start in Symbol 2 and length 12 for other slots;

    - Option 3: Start in Symbol {7} and length {4} symbols

  - PDCCH Format to be used

    - Option 1: Format 2-0, using CO-DurationPerCell-r16 to indicate the COT duration;

    - Option 2: Do not use DCI Format 2-0

In this contribution, we will continue discussion on open issues and also concerns on some agreements.

# Discussion

Issue 1: Test design

As discussed in our contribution [4], we propose to define new PDSCH requirement for unlicensed carrier with burst transmission and the licensed carrier requirement can reuse Rel-16 CA requirement. The test setup for unlicensed carrier is the same and could setup a single carrier as Rel-15 NR with burst transmission.

Proposal 1: Use same setup as Rel-15 NR for unlicensed carrier in scenario A or C.

Issue 2: PDSCH Type

PDSCH mapping type B is optional for NR which means type A requirement should be prioritized. LTE defines frame structure 3 which is similar as NR PDSCH mapping type B, and eLAA define requirements based on it. But Rel-15 NR already define requirements for type B, and the extra type B symbol length {9,10} won’t affect DM-RS symbol number. Furthermore, burst transmission will estimate different transmission start time and it could cover the intention of using type B. In that case, it might be unnecessary to specially define type B requirements for NR-U.

Proposal 3: Only define requirement for PDSCH type A.

Issue 3: PDCCH Format to be used

The enhancement for GC-PDCCH includes adding COT information and availability information of carriers/RB sets into DCI format 2-0, but this feature is optional. The COT information can help UE to switch it’s PDCCH monitoring group for power saving by the end of COT. The fixed carriers/RB sets are used in demodulation tests. In that case, it won’t affect demodulation test and no need to be configured.

Proposal 3: Do not configure DCI format 2-0 for PDSCH demodulation requirement.

Issue 4: Detailed simulation parameters

For scenario A PCell, Rel-16 NR CA requirement could be reused.

For scenario A SCell and scenario C PCell, same assumptions could be configured.

- Bandwidth

As discussed in [4], 20MHz is necessary to cover typical LBT bandwidth. Current CA requirements applicability rule are defined for “the largest aggregated CA bandwidth combination”. There will be a huge effort if we follow this rule, so a new rule might be needed. Otherwise, all possible bandwidth should be included.

- Burst transmission model

Follow proposal in [4].

- Test metric: 70% as baseline

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 20, [40, 60, 80] |
| SCS |  | kHz | 30 |
| Active DL BWP index |  |  | 1 |
| Slot Pattern |  |  | DDSU, S=10D2G2U |
| LBT failure probability |  |  | 0.5 |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | The first SSB |
|  | SSB Q factor |  | 8 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | According to DL Transmission Model |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| Test metric |  |  | 70% throughput |

# Conclusion

Issue 1: Test design

Proposal 1: Use same setup as Rel-15 NR for unlicensed carrier in scenario A or C

Issue 2: PDSCH Type

Proposal 3: Only define requirement for PDSCH type A

Issue 3: PDCCH Format to be used

Proposal 3: Do not configure DCI format 2-0 for PDSCH demodulation requirement

Issue 4: Detailed simulation parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 20, [40, 60, 80] |
| SCS |  | kHz | 30 |
| Active DL BWP index |  |  | 1 |
| Slot Pattern |  |  | DDSU, S=10D2G2U |
| LBT failure probability |  |  | 0.5 |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | The first SSB |
|  | SSB Q factor |  | 8 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | According to DL Transmission Model |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| Test metric |  |  | 70% throughput |

# References

[1]	R4-2017685, “Way Forward on NR-U UE demodulation requirements”, Qualcomm, RAN4#97-e

[2]	R4-2017610, “Summary_315_NR_unlic_Demod_UE_and_Round.docx”, Qualcomm, RAN4#97-e

[3]	TS36.104 v16.6.0

[4]	R4-2100995, “Discussion on general issue of NR-U UE and CSI demodulation requirements”, RAN4#98-e, Ericsson