3GPP TSG-RAN WG4 Meeting #98-e	R4-2101000

Electronic meeting, 25th January – 5th Feburary, 2021

Source:	Ericsson

Title:	Simulation results for NR-U PUSCH demodulation requirement

Agenda item:	7.1.7.4.2

Document for:	others

# Introduction

In RAN4#97-e meeting, companies got agreements on PUSCH simulation assumptions [1]. This contribution delivers some simulation results based on our analysis and proposals [2].

# Simulation assumptions and results

Table 1. Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | {0, 2, 3, 1} |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | The first interlace |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Table 2. Simulation results of 20MHz bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | Additional DM-RS position | MCS | Subcarrier spacing (kHz) | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | typeA | typeB |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | pos1 | 20 | 15 | 9.5 | 9.5 |
|  |  |  |  |  |  |  | 30 | 9.4 | 9.4 |
|  |  |  |  |  |  | 16 | 15 | 6.8 | 6.8 |
|  |  |  |  |  |  |  | 30 | 6.7 | 6.7 |

Table 3. Simulation results of 80MHz bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | Additional DM-RS position | MCS | Subcarrier spacing (kHz) | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | typeA | typeB |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | pos1 | 20 | 30 | 9.8 | 9.8 |
|  |  |  |  |  |  | 16 | 30 | 6.8 | 6.8 |

# Conclusion

In this contribution, we deliver our simulation results based on the WF in the last meeting and our discussion paper [2].

# References

[1]	R4-2017688, “Way forward on NR-U BS demodulation requirements for general part and PUSCH”, RAN4#97-e, Huawei, HiSilicon

[2]	R4-2100999, “Discussion on NR-U PUSCH demodulation requirements”, RAN4#98-e, Ericsson

[3]