3GPP TSG-RAN WG4 Meeting #98-e	R4-2101041

Electronic Meeting, January 25th – February 5th 2021

Agenda item:	11.11.1

Source: 		NTT DOCOMO, INC.

Title: 	Views on candidate bands for NR repeater

Document for:	Approval

- Introduction

In RAN #90, new WI on NR repeater was approved [1].

In this contribution, we provide our views on candidate bands for NR repeater.

2.	Discussion

2.1	Candidate bands

At the previous RAN plenary meeting, new WI NR repeater was agreed and the objective is shown as below.

RAN#90e [1] Normative work phase objective [RAN4] - Specify RF and EMC requirements for NR repeaters - Consider FR1 (FDD and TDD) and FR2 (TDD) bands For the above objective, the leveraging of RF specifications for LTE repeater and IAB should be sought. It is assumed that the repeater does not perform adaptive beamforming towards the UE. It is assumed that the TDD repeater meets both BS and UE emission requirements (or the more stringent absolute level in dBm) in all slots. The following constraint may be considered for TDD repeaters, as needed, to contain the workload associated with this project: - TX EIRP/TRP/power of Repeater not exceeding any UE power class defined in the band Note: FR1 FDD repeaters testing is assumed to be conducted. This assumption can be revisited in RAN#91.

FR1 (FDD and TDD) and FR2 (TDD) bands will be considered for NR repeater and we discuss on the candidate bands for NR repeater in this paper.

All of NR bands defined in clause 5.2 in TS 38.104 are as below:

Table 5.2-1: NR operating bands in FR1 NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high; Duplex mode. n1; 1920 MHz – 1980 MHz; 2110 MHz – 2170 MHz; FDD. n2; 1850 MHz – 1910 MHz; 1930 MHz – 1990 MHz; FDD. n3; 1710 MHz – 1785 MHz; 1805 MHz – 1880 MHz; FDD. n5; 824 MHz – 849 MHz; 869 MHz – 894 MHz; FDD. n7; 2500 MHz – 2570 MHz; 2620 MHz – 2690 MHz; FDD. n8; 880 MHz – 915 MHz; 925 MHz – 960 MHz; FDD. n12; 699 MHz – 716 MHz; 729 MHz – 746 MHz; FDD. n13; 777 MHz – 787 MHz; 746 MHz – 756 MHz; FDD. n14; 788 MHz – 798 MHz; 758 MHz – 768 MHz; FDD. n18; 815 MHz – 830 MHz; 860 MHz – 875 MHz; FDD. n20; 832 MHz – 862 MHz; 791 MHz – 821 MHz; FDD. n25; 1850 MHz – 1915 MHz; 1930 MHz – 1995 MHz; FDD. n26; 814 MHz – 849 MHz; 859 MHz – 894 MHz; FDD. n28; 703 MHz – 748 MHz; 758 MHz – 803 MHz; FDD. n29; N/A; 717 MHz – 728 MHz; SDL. n30; 2305 MHz – 2315 MHz; 2350 MHz – 2360 MHz; FDD. n34; 2010 MHz – 2025 MHz; 2010 MHz – 2025 MHz; TDD. n38; 2570 MHz – 2620 MHz; 2570 MHz – 2620 MHz; TDD. n39; 1880 MHz – 1920 MHz; 1880 MHz – 1920 MHz; TDD. n40; 2300 MHz – 2400 MHz; 2300 MHz – 2400 MHz; TDD. n41; 2496 MHz – 2690 MHz; 2496 MHz – 2690 MHz; TDD. n46; 5150 MHz – 5925 MHz; 5150 MHz – 5925 MHz; TDD3. n48; 3550 MHz – 3700 MHz; 3550 MHz – 3700 MHz; TDD. n50; 1432 MHz – 1517 MHz; 1432 MHz – 1517 MHz; TDD. n51; 1427 MHz – 1432 MHz; 1427 MHz – 1432 MHz; TDD. n53; 2483.5 MHz – 2495 MHz; 2483.5 MHz – 2495 MHz; TDD. n65; 1920 MHz – 2010 MHz; 2110 MHz – 2200 MHz; FDD. n66; 1710 MHz – 1780 MHz; 2110 MHz – 2200 MHz; FDD. n70; 1695 MHz – 1710 MHz; 1995 MHz – 2020 MHz; FDD. n71; 663 MHz – 698 MHz; 617 MHz – 652 MHz; FDD. n74; 1427 MHz – 1470 MHz; 1475 MHz – 1518 MHz; FDD. n75; N/A; 1432 MHz – 1517 MHz; SDL. n76; N/A; 1427 MHz – 1432 MHz; SDL. n77; 3300 MHz – 4200 MHz; 3300 MHz – 4200 MHz; TDD. n78; 3300 MHz – 3800 MHz; 3300 MHz – 3800 MHz; TDD. n79; 4400 MHz – 5000 MHz; 4400 MHz – 5000 MHz; TDD. n80; 1710 MHz – 1785 MHz; N/A; SUL. n81; 880 MHz – 915 MHz; N/A; SUL. n82; 832 MHz – 862 MHz; N/A; SUL. n83; 703 MHz – 748 MHz; N/A; SUL. n84; 1920 MHz – 1980 MHz; N/A; SUL. n86; 1710 MHz – 1780 MHz; N/A; SUL. n89; 824 MHz – 849 MHz; N/A; SUL. n90; 2496 MHz – 2690 MHz; 2496 MHz – 2690 MHz; TDD. n91; 832 MHz – 862 MHz; 1427 MHz – 1432 MHz; FDD2. n92; 832 MHz – 862 MHz; 1432 MHz – 1517 MHz; FDD2. n93; 880 MHz – 915 MHz; 1427 MHz – 1432 MHz; FDD2. n94; 880 MHz – 915 MHz; 1432 MHz – 1517 MHz; FDD2. n951; 2010 MHz – 2025 MHz; N/A; SUL. n964; 5925 MHz – 7125 MHz; 5925 MHz – 7125 MHz; TDD3. n975; 2300 MHz – 2400 MHz; N/A; SUL. n985; 1880 MHz – 1920 MHz; N/A; SUL. Table 5.2-2: NR operating bands in FR2 NR operating band; Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high; Duplex mode. n257; 26500 MHz – 29500 MHz; TDD. n258; 24250 MHz – 27500 MHz; TDD. n259; 39500 MHz – 43500 MHz; TDD. n260; 37000 MHz – 40000 MHz; TDD. n261; 27500 MHz – 28350 MHz; TDD.

In addition, the operating bands for IAB are defined in clause 5.2 in TS 38.174 as below:

5.2 Operating bands NR IAB is designed to operate in the operating bands in FR1 defined in table 5.2-1 and operating bands in FR2 defined in 38.104 [2]. Table 5.2-1 NR IAB operating bands in FR1 NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high; Duplex Mode. n41; 2496 MHz – 2690 MHz; 2496 MHz – 2690 MHz; TDD. n77; 3300 MHz – 4200 MHz; 3300 MHz – 4200 MHz; TDD. n78; 3300 MHz – 3800 MHz; 3300 MHz – 3800 MHz; TDD. n79; 4400 MHz – 5000 MHz; 4400 MHz – 5000 MHz; TDD.

Regarding IAB requirements, the operating bands for FR1 are limited as n41, n77, n78 and n79 which is assumed to be used in the near future, and there are no limitation for FR2 bands. The operating band of IAB for FR1 may be added by some request since there is a NOTE “Other operating bands and IAB-DU or IAB-MT channel bandwidths may be considered in future releases.” in TS 38.174 clause 5.1.

Observation 1: The operating bands of IAB are limited compared with all of NR bands for Base Station.

We think that NR repeater is expected to be widely used for covering areas that are difficult to be covered, and filling the gap of areas. Therefore, NR repeater will be used in not only NR major bands (n41, n77, n78, n79, FR2 bands) but also other bands corresponding LTE bands (including FDD and TDD band). Taking into account of these NR repeater’s situation, we think that considering all of NR bands which is defined in TS 38.104 is necessary as NR repeater’s operating bands. Limiting of the number of NR bands which can be supported by NR repeater affects using it in other bands due to the time lag of introducing the requirements for the new bands in 3GPP specification. In order to allow for more rapid development, ultimately we would like to make band agnostic requirements of NR repeater for FR1 and FR2 respectively.

Observation 2: NR repeater is expected to be widely used for covering areas and filling the gap of areas in NR bands including corresponding LTE FDD and TDD bands.

Proposal 1: RAN4 considers all of NR bands defined in TS 38.104 as NR repeater’s operating bands.

3.	Conclusion

In this contribution, we provide views on candidate bands for NR repeater. The following proposals are obtained.

Observation 1: The operating bands of IAB are limited compared with all of NR bands for Base Station.

Observation 2: NR repeater is expected to be widely used for covering areas and filling the gap of areas in NR bands including corresponding LTE FDD and TDD bands.

Proposal 1: RAN4 considers all of NR bands defined in TS 38.104 as NR repeater’s operating bands.

References

- RP-202927, “New WID on NR Repeaters”, Qualcomm, RAN #90e, December 2020