3GPP TSG-RAN WG4 Meeting #98-e                                      R4-2101068

Electronic Meeting, 25 Jan. – 5 Feb., 2021

Agenda Item:	7.3.6.2.1

Source:	MediaTek Inc.

Title:	Simulation results for NR V2X PSSCH test case

Document for:		Discussion

- Introduction

In RAN4#96 meeting, the simulation assumptions of PSSCH were agreed [1]. In the contribution, we will provide initial simulation results of PSSCH based on simulation assumption.

- Discussion

Simulation assumptions

The simulation assumptions are showed as following:

Table 1 Simulation assumption for PSSCH demodulation performance

| Parameters | Unit | Value |  |
| --- | --- | --- | --- |
| Test |  | SCH_Test1 | SCH_Test3 |
| Synchronization source |  | GNSS | GNSS |
| Propagation condition |  | TDLA30-180 | TDLA30-2700 |
| Channel bandwidth | MHz | 20 | 20 |
| Allocated resource blocks | RB | 20 | 20 |
| PSCCH resource allocation |  | 10 RBs with 2 symbols | 10 RBs with 2 symbols |
| Subcarrier spacing | kHz | 30 | 30 |
| Timing offset |  | CP/2-12Ts | CP/2-12Ts |
| Frequency offset | Hz | 600 | 600 |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  | 9 | 9 |
| CP-OFDM symbols for slot without PSFCH |  | 12 | 12 |
| DMRS symbols for slot with PSFCH |  | 2 | 3 |
| DMRS symbols for slot without PSFCH |  | 2 | 4(Note 2) |
| Modulation order |  | 6 | 2 |
| MCS index |  | 17 | 4 |
| Transport block CRC | Bits | 24 | 24 |
| Maximum number of HARQ transmissions |  | 1 | 1 |
| PSFCH resource period | Slot | 1 | 1 |
| Antenna configuration |  | 1x2 | 1x2 |
| Performance metric |  | SNR@10% BLER of PSSCH |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: It is only applied to cases when PSFCH resource period =1 |  |  |  |

Some controversial simulation parameter are used for this simulation based on our discussion in [2].

Simulation results

- SCH_Test1

Based on the simulation assumptions, we provide simulation results about SCH_Test1 in Figure 1.

Figure 1 PSSCH with TDLA30ns-180Hz simulation result

According to the simulation results, we can see that the corresponding SNR targeted 1% BLER is about 19.2dB.

Proposal 1: The required SNR targeted 10% BLER for PSSCH test case with TDLA30ns-1400Hz is about 19.2dB.

- SCH_Test2

Based on the simulation assumptions, we provide simulation results about SCH_Test1 in Figure 2.

Figure 2 PSSCH with TDLA30ns-2700Hz simulation result

According to the simulation results, we can see that the corresponding SNR targeted 10% BLER is about 4.2dB.

Proposal 2: The required SNR targeted 10% BLER for PSSCH test case with TDLA30ns-2700Hz is about 4.2dB.

- Summary

In this contribution, we provide the PSSCH simulation result and make the following proposal:

Proposal 1: The required SNR targeted 10% BLER for PSSCH test case with TDLA30ns-1400Hz is about 19.2dB.

Proposal 2: The required SNR targeted 10% BLER for PSSCH test case with TDLA30ns-2700Hz is about 4.2dB.

- Reference

- R4-2017470, “Simulation assumptions for V2X single link test cases”,  Huawei, HiSilicon

- R4-2101065, “Discussion on NR V2X Demod test”,  MediaTek