3GPP TSG-RAN WG4 Meeting #98-e                	 R4-2101147

Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda Item:	7.11.2

Source:	MediaTek Inc.

Title:	Discussion on interruption test cases for Tx switching

Document for:		Discussion

# Introduction

In RAN4#97e meeting, a big CR [1] for the interruption test cases of Tx switching was approved in Email discussion after meeting. In this paper, we provide some further observations and suggestions.

# Discussion

EUTRAN configuration

Test case A.4.5.8 is conducted with one EUTRAN PCell and one NR PSCell, where the test parameters and applicability for E-UTRAN PCell are defined in A.3.7.2. There is no deterministic configuration for FDD or TDD in A.3.7.2.

However, in RAN4#96e meeting, the agreement in [2] was to define test case with EUTRAN in FDD. Therefore, we suggest to clear indicate in the test case that the EUTRAN cell is in a FDD band.

Proposal 1: Clarify in Test case A.4.5.8 that the EUTRAN cell is in FDD.

UE without interruption

In current test cases (A.4.5.8, A.6.5.7.1 and A.6.5.7.2), L1-RSRP is triggered to request UE to measure the CSI-RS. The location of the CSI-RS symbol depends on UE’s capability uplinkTxSwitchingPeriod. However, this capability is used to indicate the time UE needs to Tx switch, it does not mean UE always needs the interruption. There is another UE capability uplinkTxSwitching-DL-Interruption-r16 to indicate the need of interruption. If UE does not report uplinkTxSwitching-DL-Interruption-r16, UE should be able to receive DL signals even during the switching period.

Observation 1: If UE does not report uplinkTxSwitching-DL-Interruption-r16, UE should be able to receive DL signals even during the switching period.

For this kind of UEs, our suggestion is to move the CSI-RS to the last OFDM symbol in the special slot. For example, symbol #9 for S=’10DL:2GP:2UL’ on TDD carrier and symbol#13 on FDD carrier.

Proposal 2: If UE does not report uplinkTxSwitching-DL-Interruption-r16, aperiodic CSI-RS for L1-RSRP reporting is triggered on symbol #9 for S=’10DL:2GP:2UL’ on TDD carrier and symbol#13 on FDD carrier.

We also submit a CR [3] to implement above proposals.

# Summary

In this paper, we provide our view about the interruption test cases for Tx switching. We have the following observations and proposals.

Observation 1: If UE does not report uplinkTxSwitching-DL-Interruption-r16, UE should be able to receive DL signals even during the switching period.

Proposal 1: Clarify in Test case A.4.5.8 that the EUTRAN cell is in FDD.

Proposal 2: If UE does not report uplinkTxSwitching-DL-Interruption-r16, aperiodic CSI-RS for L1-RSRP reporting is triggered on symbol #9 for S=’10DL:2GP:2UL’ on TDD carrier and symbol#13 on FDD carrier.

# Reference

[1] R4-2017172	“Big CR: Introduction of Rel-16 NR FR1 RF WI RRM performance requirements”, Huawei, HiSilicon

[2] R4-2012150, “WF on test case for DL interruption due to Tx switching between two uplink carriers”, Huawei, HiSilicon

[3] R4-2101148, “Update on interruption test cases for Tx switching R16”, MediaTek inc.