3GPP TSG-RAN WG4 Meeting #98-e	R4-2101245

# E-meeting, 25th January – 5th February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0139 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on FRC for URLLC UE Higher BLER requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Definition of UE URLLC requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added FRC for FR1 pre-emption indication requirements (in case MCS16 will be used) - Added FRC for FR2 requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE URLLC requirements are not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.1.1, A.3.2.2.2, A.3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

<SKIP UNCHANGED PART>

Table A.3.2.1.1-2: PDSCH Reference Channel for FDD (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-2.1 FDD | R.PDSCH.1-2.2 FDD | R.PDSCH.1-2.3 FDD | R.PDSCH.1-2.4 FDD | [R.PDSCH.1-2.5 FDD] |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 | 52 | 52 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 16 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.64 |
| Number of MIMO layers |  | 1 | 2 | 3 | 4 | 1 |
| Number of DMRS REs |  | 12 | 12 | 24 | 24 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 13064 | 26120 | 35856 | 48168 | 17424 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 2 | 4 | 5 | 6 | 3 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 26208 | 52416 | 71136 | 94848 | 26208 |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 27456 | 54912 | 74880 | 99840 | 27456 |
| Max. Throughput averaged over 2 frames | Mbps | 12.411 | 24.814 | 34.063 | 45.760 | 16.553 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

#### A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

<SKIP UNCHANGED PART>

Table A.3.2.2.2-2: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-2.1 TDD | R.PDSCH.2-2.2 TDD | R.PDSCH.2-2.3 TDD | R.PDSCH.2-2.4 TDD | [R.PDSCH.2-2.5 TDD] |
| Channel bandwidth | MHz | 40 | 40 | 40 | 40 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 106 | 106 | 106 | 106 | 106 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 13 | 13 | 13 | 13 | 16 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  | 1 | 2 | 3 | 4 | 1 |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 12 | 12 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 | 24 | 24 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 8456 | 16896 | 22032 | 29192 | 11528 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 26632 | 53288 | 73776 | 98376 | 35856 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6}for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 2 | 3 | 3 | 4 | 2 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | 4 | 7 | 9 | 12 | 5 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20, 21 | Bits | 53424 | 106848 | 144008 | 193344 | 53424 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 17808 | 35616 | 45792 | 61056 | 17808 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 55968 | 111936 | 152640 | 203520 | 55968 |
| Max. Throughput averaged over 2 frames | Mbps | 37.644 | 75.318 | 104.004 | 138.646 | 50.711 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

#### A.3.2.2.5	Reference measurement channels for SCS 120 kHz FR2

Table A.3.2.2.5-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 and FR2.120-1A (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-1.1 TDD | R.PDSCH.5-1.2 TDD |  |  |  |
| Channel bandwidth | MHz | 100 | 100 |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |
| Allocated resource blocks | PRBs | 66 | 66 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 2 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 2 |  |  |  |
| Allocated slots per 2 frames |  | 127 | 127 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 4 |  |  |  |
| Modulation |  | QPSK | QPSK |  |  |  |
| Target Coding Rate |  | 0.30 | 0.30 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 6 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 12 | 6 |  |  |  |
| Overhead for TBS determination |  | 6 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 3624 | 736 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 5504 | 736 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 16 | 16 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 16 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 1 | 1 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 1 | 1 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slots i = 80, 81 | Bits | 17490 | 2310 |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 12210 | 2310 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79,82,…,159} | Bits | 18282 | 2310 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 31.942 | 4.673 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<SKIP UNCHANGED PART>

Table A.3.2.2.5-10: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-2

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-10.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {2,…,159} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 78 |  |  |  |  |
| MCS table |  | 64QAMLowSE |  |  |  |  |
| MCS index |  | [13/16/19] |  |  |  |  |
| Modulation |  | [QPSK/16QAM/16QAM] |  |  |  |  |
| Target Coding Rate |  | [0.51/0.37/054] |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {2,…,159} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0, 1 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {2,…,159} | Bits | [9224/13320/19464] |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0, 1 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {2,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0, 1 and Slot i, if mod(i, 4) = {2,3} for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {2,…,159} | CBs | [2/2/3] |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0,1 and Slot i, if mod(i, 4) = {2, 3} for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80, 81 | Bits | [17490/34980/34980] |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,1} for i from {2,…,159} | Bits | [18282/36564/36564] |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | [35.974/51.948/75.910] |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

END OF CHANGE