3GPP TSG-RAN WG4 Meeting # 98-e 	  													      R4-2101275

Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda Item:	7.7.3.2.1.2

Source:	Intel Corporation

Title:	Discussion on PRS RSRP accuracy requirements for NR Positioning

Document for:		Discussion

# Introduction

In the last RAN4 meeting, PRS RSRP requirements were discussed, and some agreements achieved [1]. In this contribution we will focus on the discussion of remaining issue below which are unique for PRS RSRP measurement.

- Measurement accuracy requirements

# Discussion

In the last meeting the following agreement for PRS RSRP measurement SINR side condition was achieved [1].

Define PRS RSRP measurement accuracy requirements for 2 SINR side conditions - SINR side condition #1: [-3 dB or -6 dB]. - SINR side condition #2: [-13 dB]. - No differentiation between serving and neighbor cell/TRPs - Same side condition apply for FR1 and FR2 - FFS whether the test case will cover both side condition

Therefore, for RSRP measurement the different accuracy requirements for these two SINR conditions shall be defined.

Proposal 1:  The different accuracy requirements for PRS RSRP measurement with two SINR conditions below shall be defined.

  - SINR side condition #1: [-3 dB or -6 dB].

  - SINR side condition #2: [-13 dB].

According to the simulation result in Figure 1 below, we can see:

Figure 1. Simulation results for different SINR

Observation 1: When SINR is higher enough (>= -6dB) the performance gap because of DL-PRS-NumSymbols ,DL-PRS_ResourceRepetitionFactor and DL-PRS-CombSizeN can be very little.

Proposal 2 : The accuracy requirements of RSRP when SINR is larger than [-6dB] can be independent with the following parameters:

- DL-PRS-NumSymbols

- DL-PRS_ResourceRepetitionFactor

- DL-PRS-CombSizeN

As agreed in the last meeting, how to define the accuracy requirements of PRS RSRP with the parameters combinations (e.g. PRS BW, and others) can follow the same principle for RSTD measurement.

Thus, base on our discussion for RSTD accuracy requirements [3] and corresponding simulation results aligned in RAN4 in Appendix, we can conclude that :

Proposal 3 : The accuracy requirements of PRS RSRP can be specified by the Table 1 below.

Table 1: PRS RSRP accuracy

| FR | PRS BW, MHz (or PRBs) | PRS_NormLenthPerSlot | Accuracy (dB) (SINR= -6dB) | Accuracy (dB) (SINR= -13dB) |
| --- | --- | --- | --- | --- |
| FR1 | 52 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
|  | 104 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
|  | 268 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
| FR2 | 32 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
|  | 64 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
|  | 128 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
| Note 1: PRS_NormLenthPerSlot = (DL-PRS-NumSymbols x DL-PRS_ResourceRepetitionFactor) /DL-PRS-CombSizeN |  |  |  |  |

# Conclusion

In this contribution, some considerations on PRS RSRP measurement accuracy requirements are provided and the following observations and proposals can be drawn:

Proposal 1:  The different accuracy requirements for PRS RSRP measurement with two SINR conditions below shall be defined.

  - SINR side condition #1: [-3 dB or -6 dB].

  - SINR side condition #2: [-13 dB].

Observation 1: When SINR is higher enough (>= -6dB) the performance gap because of DL-PRS-NumSymbols ,DL-PRS_ResourceRepetitionFactor and DL-PRS-CombSizeN can be very little.

Proposal 2 : The accuracy requirements of RSRP when SINR is larger than [-6dB] can be independent with the following parameters:

- DL-PRS-NumSymbols

- DL-PRS_ResourceRepetitionFactor

- DL-PRS-CombSizeN

Proposal 3 : The accuracy requirements of PRS RSRP can be specified by the Table 1 below.

Table 1: PRS RSRP accuracy requirements

| FR | PRS BW, MHz (or PRBs) | PRS_NormLenthPerSlot | Accuracy (dB) (SINR=[ -6dB]) | Accuracy (dB) (SINR= -13dB) |
| --- | --- | --- | --- | --- |
| FR1 | 52 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
|  | 104 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
|  | 268 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
| FR2 | 32 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
|  | 64 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
|  | 128 | 1 | TBD | TBD |
|  |  | ≥2 | TBD |  |
|  |  | ≥4 | TBD |  |
|  |  | ≥6 | TBD |  |
| Note 1: PRS_NormLenthPerSlot = (DL-PRS-NumSymbols x DL-PRS_ResourceRepetitionFactor) /DL-PRS-CombSizeN |  |  |  |  |

# References

- R4-2017371, “WF on requirements on NR Pos performance requirements”

- 3GPP TS 38.133 v16.5.0: "NR: Requirements for support of radio resource management "

- R4-2101274, “Discussion on NR PRS RSTD measurement accuracy requirements”

# Appendix: Link level simulation results

Table A1: RSRP accuracy simulation results in AWGN (ns)

| FR | PRS BW (PRBs) | PRS_NormLenthPerSlot | SINR= -6dB |  |  |  |  | SINR= -13dB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intel | Company1 |  |  |  | Intel | Company1 |  |  |  |
| FR1 | 52 | 1 | ±1.4 |  |  |  |  | ±11.1 |  |  |  |  |
|  |  | ≥2 | ±1.1 |  |  |  |  | ±9.4 |  |  |  |  |
|  |  | ≥4 | ±0.75 |  |  |  |  | ±8.1 |  |  |  |  |
|  |  | ≥6 | ±0.61 |  |  |  |  | ±7.2 |  |  |  |  |
|  | 104 | 1 | ±0.98 |  |  |  |  | ±8.7 |  |  |  |  |
|  |  | ≥2 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥4 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥6 | ±0.41 |  |  |  |  | ±5.8 |  |  |  |  |
|  | 268 | 1 | ±0.56 |  |  |  |  | ±5.3 |  |  |  |  |
|  |  | ≥2 | ±0.39 |  |  |  |  | ±5.0 |  |  |  |  |
|  |  | ≥4 | ±0.26 |  |  |  |  |  |  |  |  |  |
|  |  | ≥6 | ±0.20 |  |  |  |  | ±4.3 |  |  |  |  |
| FR2 | 32 | 1 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥2 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥4 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥6 |  |  |  |  |  |  |  |  |  |  |
|  | 64 | 1 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥2 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥4 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥6 |  |  |  |  |  |  |  |  |  |  |
|  | 128 | 1 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥2 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥4 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥6 |  |  |  |  |  |  |  |  |  |  |
| Note 1: PRS_NormLenthPerSlot = (DL-PRS-NumSymbols x DL-PRS_ResourceRepetitionFactor) /DL-PRS-CombSizeN |  |  |  |  |  |  |  |  |  |  |  |  |

Table A2: RSRP accuracy simulation results in the fading channel (ns)

| FR | PRS BW (PRBs) | PRS_NormLenthPerSlot | SINR= -6dB |  |  |  |  | SINR= -13dB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intel | Company1 |  |  |  | Intel | Company1 |  |  |  |
| FR1 | 52 | 1 | ±4.5 |  |  |  |  | ±9.9 |  |  |  |  |
|  |  | ≥2 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥4 | ±3.0 |  |  |  |  | ±8.0 |  |  |  |  |
|  |  | ≥6 | ±2.9 |  |  |  |  | ±7.0 |  |  |  |  |
|  | 104 | 1 | ±3.2 |  |  |  |  | ±8.5 |  |  |  |  |
|  |  | ≥2 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥4 | ±2.7 |  |  |  |  | ±7.5 |  |  |  |  |
|  |  | ≥6 |  |  |  |  |  |  |  |  |  |  |
|  | 268 | 1 | ±2.9 |  |  |  |  | ±7.0 |  |  |  |  |
|  |  | ≥2 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥4 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥6 |  |  |  |  |  |  |  |  |  |  |
| FR2 | 32 | 1 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥2 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥4 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥6 |  |  |  |  |  |  |  |  |  |  |
|  | 64 | 1 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥2 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥4 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥6 |  |  |  |  |  |  |  |  |  |  |
|  | 128 | 1 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥2 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥4 |  |  |  |  |  |  |  |  |  |  |
|  |  | ≥6 |  |  |  |  |  |  |  |  |  |  |
| Note 1: PRS_NormLenthPerSlot = (DL-PRS-NumSymbols x DL-PRS_ResourceRepetitionFactor) /DL-PRS-CombSizeN |  |  |  |  |  |  |  |  |  |  |  |  |