3GPP TSG-RAN WG4 Meeting #98-e	R4-2101296

# Electronic Meeting, 25th Jan - 5th Feb, 2020

Title: 	Simulation results on PDSCH requirements for NR DL 256QAM for FR2

Source: 	Huawei, HiSilicon

Agenda item:	7.10.1.1

Document for:	Information

Background

During RAN4#97-e meeting, way forward [1] for NR Rel-16 UE demodulation and CSI reporting requirements for FR2 DL 256QAM was approved. In this contribution, we provide our simulation results on PDSCH requirements for alignment. The simulation assumption is derived from WF [1] as Table 1-1 below.

Table 1-1 Simulation assumption for NR DL 256QAM for FR2

| Parameter | Value |
| --- | --- |
| Tx EVM | 3% |
| Rx impairment modelling and band agnostic requirements | Not explicitly model Rx impairment |
| SCS | 120 kHz |
| Channel bandwidth and PRB allocation | 50MHz CBW with full PRB allocation |
| Rank | Rank 1 only |
| MCS | MCS 20 |
| TDD Configuration | DDDSU, S=10D:2G:2U |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |
| PDCCH duration | Symbol #0 in each slot |
| PDSCH configuration | Full DL slots: Start symbol 1, Duration 13 Special slots: Start symbol 1, Duration 9 |
| PDSCH mapping type | Type A |
| PRB bundling size and Precoding model | PRB bundling size: 2 Precoding model: Random Precoding, per slot , WB granularity |
| DM-RS configuration | Type 1 single symbol front loaded, 1 additional DMRS |
| PT-RS configuration | per 2PRB in frequency domain per symbol in time domain |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |
| Overhead for TBS determination | 6 |
| HARQ RV sequence | {0, 2, 3, 1} |
| HARQ process number | 8 |
| Receiver assumption | MMSE-IRC |
| MIMO configuration | 2Tx 2Rx ULA low |
| Test metric | 70% of max TP |

Simulations

The simulation results is shown in the table 2-1 and Figure 2-1 below.

Table 2-1: Simulation results for NR DL 256QAM for FR2

| Case number | CBW/MHz | MIMO | Rank | Propagation condition | MCS | SNR@70%max throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| 1 | 50 | 2x2 | 1 | TDLD30-75 | 20 | 17.11 | 19.61 |

Figure 2-1: Ideal simulation results for NR DL 256QAM for FR2

Conclusion

In this paper we provide our simulation results on UE demodulation and CSI reporting requirements for FR2 DL 256QAM for alignment.

Reference

R4-2017536, WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM, RAN4#97-e, China Telecom