3GPP TSG-RAN WG4 Meeting #98-e	R4-2101331

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Simulation results on URLLC UE CQI reporting test

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.1.2

Document for:	Discussion

Background

In this paper, the simulation results for URLLC UE demodulation requirements are presented based on the agreed parameters on previous e-meetings.

Simulation

Simulation assumption

The simulation parameters were suggested in WF [1], which is presented in table below:

Table 2-1: Simulation assumption for UE CQI reporting test

| Parameter |  |  | Unit | FDD | TDD |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 40 |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 106 |
| Subcarrier spacing |  |  | kHz | 15 | 30 |
| MCS table |  |  |  | Table 3 |  |
| PDSCH starting symbol/length |  |  |  | 2/12 |  |
| Number of PDSCH MIMO layers |  |  |  | 1 |  |
| PDSCH mapping type |  |  |  | Type A |  |
| DMRS type |  |  |  | Type 1 |  |
| DMRS duration |  |  |  | single-symbol DMRS |  |
| Number of additional DMRS |  |  |  | 1 |  |
| Slot pattern |  |  |  | N/A | 7D1S2U, S=6D: 4G: 4U |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1x2, ULA low​ 1x4, ULA low​ |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 2,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 | 16 |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 10/9 |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |
|  |  | RI Restriction |  | 00000001 |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 8 | 9.5 |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Target BLER |  |  |  | 10^-5 |  |
| PBCH |  |  |  | slot#0 per 20ms periodicity |  |
| PT-RS |  |  |  | Disabled |  |
| PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL |  |  |  |  |  |

Simulation results:

The simulation results of FDD and TDD for 15 kHz and 30 kHz are shown in below tables:

- FDD

Table 2-2: Simulation results for UE CQI reporting test (FDD, 1x2)

| SNR | Median CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} | BLER for Median CQI - 1 | BLER for Median CQI | BLER for Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| -12 | 0 | 100 | 9.54e-2 | 9.54e-2 | 9.54e-2 |
| -11 | 0 | 100 | 1.2e-4 | 1.2e-4 | 1.2e-4 |
| -10 | 1 | 100 | 0 | 0 | 2.5e-3 |
| -9 | 2 | 100 | 0 | 2.13e-6 | 1e-1 |
| -8 | 3 | 100 | 0 | 8.65e-6 | 1.5e-1 |
| -7 | 4 | 100 | 0 | 8.54e-6 | 8.58e-1 |
| -6 | 4 | 100 | 0 | 0 | 8.32e-2 |
| -5 | 5 | 100 | 0 | 1.52e-6 | 9.87e-1 |
| -4 | 5 | 100 | 0 | 0 | 2.1e-1 |
| -3 | 6 | 100 | 0 | 2.12e-6 | 1e0 |
| -2 | 6 | 100 | 0 | 0 | 9.66e-5 |
| -1 | 7 | 100 | 0 | 0 | 9.86e-1 |
| 0 | 8 | 100 | 0 | 5.74e-6 | 1e0 |
| 1 | 8 | 100 | 0 | 0 | 9.78e-1 |
| 2 | 9 | 100 | 0 | 8e-6 | 1e0 |
| 3 | 9 | 100 | 0 | 0 | 8.98e-1 |
| 4 | 10 | 100 | 0 | 7.54e-6 | 1e0 |
| 5 | 10 | 100 | 0 | 0 | 7.36e-1 |
| 6 | 11 | 100 | 0 | 1.22e-5 | 1e0 |
| 7 | 11 | 100 | 0 | 2.36e-6 | 9.98e-1 |
| 8 | 12 | 100 | 0 | 1.65e-5 | 9.99e-1 |
| 9 | 12 | 100 | 0 | 0 | 3.02e-2 |
| 10 | 13 | 100 | 0 | 1.88e-6 | 1e0 |
| 11 | 13 | 100 | 0 | 0 | 7.85e-4 |
| 12 | 14 | 100 | 0 | 5e-6 | 9.78e-1 |
| 13 | 14 | 100 | 0 | 0 | 4.45e-3 |
| 14 | 15 | 100 | 0 | 4.68e-5 | 4.68e-5 |
| 15 | 15 | 100 | 0 | 0 | 0 |

Table 2-3: Simulation results for UE CQI reporting test (FDD, 1x4)

| SNR | Median CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} | BLER for Median CQI - 1 | BLER for Median CQI | BLER for Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| -13 | 1 | 100 | 5.4e-3 | 5.4e-3 | 2.5e-3 |
| -12 | 2 | 100 | 0 | 5.22e-3 | 8.14e-1 |
| -11 | 3 | 100 | 5.2e-6 | 8.65e-3 | 1.5e-2 |
| -10 | 4 | 100 | 0 | 8.54e-5 | 7.65e-1 |
| -9 | 4 | 100 | 0 | 2.3e-6 | 4.12e-3 |
| -8 | 5 | 100 | 0 | 1.52e-6 | 9.87e-1 |
| -7 | 5 | 100 | 0 | 0 | 1.1e-3 |
| -6 | 6 | 100 | 0 | 3.32e-6 | 1e0 |
| -5 | 6 | 100 | 0 | 0 | 2.3e-4 |
| -4 | 7 | 100 | 0 | 0 | 9.98e-1 |
| -3 | 8 | 100 | 0 | 4.32e-6 | 1e0 |
| -2 | 8 | 100 | 0 | 0 | 1.1e-4 |
| -1 | 9 | 100 | 0 | 8.3e-6 | 1e0 |
| 0 | 9 | 100 | 0 | 0 | 7.45e-1 |
| 1 | 10 | 100 | 0 | 8.55e-6 | 1e0 |
| 2 | 10 | 100 | 0 | 0 | 9.8e-2 |
| 3 | 11 | 100 | 0 | 1.52e-5 | 1e0 |
| 4 | 11 | 100 | 0 | 3.23e-6 | 9.12e-1 |
| 5 | 12 | 100 | 0 | 2.12e-5 | 1e0 |
| 6 | 12 | 100 | 0 | 0 | 3.4e-2 |
| 7 | 13 | 100 | 0 | 2.1e-6 | 1e0 |
| 8 | 13 | 100 | 0 | 0 | 8.1e-3 |
| 9 | 14 | 100 | 0 | 5.6e-6 | 9.99e-1 |
| 10 | 14 | 100 | 0 | 0 | 2.21e-1 |
| 11 | 15 | 100 | 0 | 5.23e-6 | 5.23e-6 |
| 12 | 15 | 100 | 0 | 0 | 0 |

- TDD

Table 2-4: Simulation results for UE CQI reporting test (TDD, 1x2)

| SNR | Median CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} | BLER for Median CQI - 1 | BLER for Median CQI | BLER for Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| -12 | 0 | 100 | 4.69e-3 | 4.69e-3 | 4.69e-3 |
| -11 | 0 | 100 | 5.45e-5 | 5.45e-5 | 5.45e-5 |
| -10 | 1 | 100 | 0 | 0 | 7.65e-4 |
| -9 | 2 | 100 | 0 | 1.2e-6 | 8.9e-2 |
| -8 | 3 | 100 | 0 | 2.56e-6 | 7.7e-2 |
| -7 | 4 | 100 | 0 | 3.23e-6 | 2.0e-1 |
| -6 | 4 | 100 | 0 | 0 | 1.2e-2 |
| -5 | 5 | 100 | 0 | 1.05e-6 | 2.42e-1 |
| -4 | 5 | 100 | 0 | 0 | 7.87e-2 |
| -3 | 6 | 100 | 0 | 1.3e-6 | 1e0 |
| -2 | 6 | 100 | 0 | 0 | 1.3e-5 |
| -1 | 7 | 100 | 0 | 0 | 8.7e-2 |
| 0 | 8 | 100 | 0 | 4.2e-6 | 1e0 |
| 1 | 8 | 100 | 0 | 0 | 8.6e-2 |
| 2 | 9 | 100 | 0 | 4.2e-6 | 1e0 |
| 3 | 9 | 100 | 0 | 0 | 1.2e-1 |
| 4 | 10 | 100 | 0 | 4.98e-6 | 1e0 |
| 5 | 10 | 100 | 0 | 0 | 9.89e-2 |
| 6 | 11 | 100 | 0 | 9.12e-6 | 1e0 |
| 7 | 11 | 100 | 0 | 0 | 8.71e-1 |
| 8 | 12 | 100 | 0 | 9.52e-6 | 1.29e-1 |
| 9 | 12 | 100 | 0 | 0 | 6.35e-3 |
| 10 | 13 | 100 | 0 | 1.27e-6 | 1e0 |
| 11 | 13 | 100 | 0 | 0 | 8.17e-5 |
| 12 | 14 | 100 | 0 | 1.64e-6 | 1e0 |
| 13 | 14 | 100 | 0 | 0 | 2.53e-4 |
| 14 | 15 | 100 | 0 | 7.46e-6 | 7.46e-6 |
| 15 | 15 | 100 | 0 | 0 | 0 |

Table 2-5: Simulation results for UE CQI reporting test (TDD, 1x4)

| SNR | Median CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} | BLER for Median CQI - 1 | BLER for Median CQI | BLER for Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| -13 | 1 | 100 | 4.5e-3 | 4.5e-3 | 1e-3 |
| -12 | 2 | 100 | 0 | 3.25e-3 | 4.97e-1 |
| -11 | 3 | 100 | 3.5e-6 | 6.49e-3 | 9.2e-3 |
| -10 | 4 | 100 | 0 | 2.3e-5 | 2.52e-1 |
| -9 | 4 | 100 | 0 | 1.3e-6 | 2.49e-3 |
| -8 | 5 | 100 | 0 | 1.2e-5 | 2.73e-1 |
| -7 | 5 | 100 | 0 | 0 | 9.84e-4 |
| -6 | 6 | 100 | 0 | 2.16e-6 | 1e0 |
| -5 | 6 | 100 | 0 | 0 | 8.6e-5 |
| -4 | 7 | 100 | 0 | 0 | 7.34e-1 |
| -3 | 8 | 100 | 0 | 3.23e-6 | 1e0 |
| -2 | 8 | 100 | 0 | 0 | 8.69e-5 |
| -1 | 9 | 100 | 0 | 2.82e-6 | 1e0 |
| 0 | 9 | 100 | 0 | 0 | 8.37e-2 |
| 1 | 10 | 100 | 0 | 5.24e-6 | 1e0 |
| 2 | 10 | 100 | 0 | 0 | 2.7e-2 |
| 3 | 11 | 100 | 0 | 8.42e-6 | 1e0 |
| 4 | 11 | 100 | 0 | 0 | 8.29e-2 |
| 5 | 12 | 100 | 0 | 1.9e-5 | 1e0 |
| 6 | 12 | 100 | 0 | 0 | 7.36e-3 |
| 7 | 13 | 100 | 0 | 1.5e-6 | 1e0 |
| 8 | 13 | 100 | 0 | 0 | 7.74e-3 |
| 9 | 14 | 100 | 0 | 4.95e-6 | 1e0 |
| 10 | 14 | 100 | 0 | 0 | 1.08e-1 |
| 11 | 15 | 100 | 0 | 4.15e-6 | 4.15e-6 |
| 12 | 15 | 100 | 0 | 0 | 0 |

Reference

- R4-2017674, “WF on ultra-low BLER requirements”, Ericsson.