3GPP TSG-RAN WG4 Meeting #98-e	R4-2101346

# Electronic Meeting, 2-13 Nov., 2020

Title: 	Simulation results on NR-U PUSCH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.1.7.4.2

Document for:	Discussion

Background

At last meeting, initial simulation assumptions and test scopes for NR-U PUSCH were discussed and related agreements and open issues are summarized in WF [1]. In this paper, we provide our simulation results.

Simulation results

The simulation results based on Table 2-2 derived from [1] are summarized in Table 2-1.

Table 2-1: Simulation results for NR-U PUSCH

| Bandwidth (MHz) |  | 20 |  | 40 |  | 60 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCS (kHz) |  | 15 | 30 | 15 | 30 | 30 | 30 |
| SNR@70% of maxTP(dB) | DMRS Type A | 10.4 | 10.4 | 10.6 | 10.6 | 10.6 | 10.5 |
|  | DMRS Type B | 10.4 | 10.4 | 10.6 | 10.6 | 10.6 | 10.5 |

Table 2-2: Simulation assumptions for NR-U PUSCH

| Parameters | Values |
| --- | --- |
| Waveform | CP-OFDM |
| Bandwidth | 20MHz,40MHz,60MHz,80MHz |
| Frequency domain resource allocation | First one interlace |
| Time domain resource allocation | 14 OFDM symbols |
| SCS | 15kHz and 30kHz |
| TDD pattern | 7D2S1U S=6D:4G:4U for 30kHz SCS 3D1S1U S=10D:2G:2U for 15kHz SCS |
| DMRS configuration | Type A and Type B Type 1 with single-symbol and dmrs-AdditionalPosition ‘pos1’ |
| Antenna configuration | 1x2 |
| MCS | 20 |
| Propagation condition | TDLA30-10 Low |
| Maximum number of HARQ transmission | 4 |
| RV sequence | {0,2,0,2} |
| Test metric | 70% of maxTP |

Conclusion

In this paper, we provide our simulation results for NR-U PUSCH.

Reference

[1]   R4-2017688 Way forward on NR-U BS demodulation requirements for general part and PUSCH