3GPP TSG-RAN WG4 Meeting #98-e	R4-2101353

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Simulation results on PSSCH requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.3.6.2.1

Document for:	Discussion

Background

At last meeting, simulation assumptions and test scope for V2X single-link were discussed. In this paper, we provide our further discussions based on WF [1]

Simulation results

Simulation results for PSSCH are shown in Figure 2-1 and Table 2-1 based on agreed simulation assumptions [2]. Corresponding simulation assumptions are shown in Table 5-1.

Figure 2-1: Simulation results for PSSCH

Table 2-1: Summary of simulation results

|  |  |  | PSSCH SNR(dB)@ 10% BLER | SCI stage 2 BLER when SNR at 10% BLER of PSSCH |
| --- | --- | --- | --- | --- |
| Test 1 | 10 RBs | PSFCH periodicity=1 | 13.24 | 0.0015 |
|  |  | PSFCH periodicity=4 | 12.95 | 0.0026 |
|  | 20 RBs | PSFCH periodicity=1 | 11.98 | 0.0001 |
|  |  | PSFCH periodicity=4 | 12.16 | 0.0001 |
| Test 2 | 10 RBs | DMRS{3,2} | 8.28 | 0 |
|  |  | DMRS{4,3} | 6.98 | 0.0003 |
|  | 20 RBs | DMRS{3,2} | 6.94 | 0.0004 |
|  |  | DMRS{4,3} | 6.37 | 0.0005 |
| Test 3 | TDLA-30-2700 | PSFCH periodicity=1 | 1.56 | 0.0014 |
|  |  | PSFCH periodicity=4 | 1.26 | 0.002 |
|  | TDLB-100-2700 | PSFCH periodicity=1 | 0.98 | 0 |
|  |  | PSFCH periodicity=4 | 0.74 | 0 |

Conclusion

In this contribution, we provide our simulation results for PSSCH test.

Reference

[1]   R4-2017687(WF on V2X single link demodulation). LG Electronics

[2]   R4-2017470 Simulation assumptions for NR V2X single link. Huawei, HiSilicon

Appendix

Table 5-1: Simulation assumptions for PSSCH

| Parameters |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Test |  |  | SCH_Test1 | SCH_Test2 | SCH_Test3 |
| Synchronization source |  |  | GNSS | GNSS | GNSS |
| Propagation condition |  |  | TDLA30-180 | TDLA30-1400 | Option 1:TDLA30-2700 baseline Option 2:TDLB100-2700 |
| Channel bandwidth |  | MHz | 20 | 20 | 20 |
| Allocated resource blocks |  | RB | Option 1: 10 Option 2: 20 | Option 1: 10 Option 2: 20 | 20 |
| PSCCH resource allocation |  |  | 10 RBs with 2 symbols | 10 RBs with 2 symbols | Option 1: 10 RBs with 2 symbols if Propagation condition is TDLA30-2700 Option 2: 10 RBs with 3 symbols if Propagation condition is TDLB100-2700 |
| Subcarrier spacing |  | kHz | 30 | 30 | 30 |
| Timing offset |  |  | CP/2-12Ts | CP/2-12Ts | CP/2-12Ts |
| Frequency offset |  | Hz | 600 | 600 | 600 |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 | 9 | 9 |
| CP-OFDM symbols for slot without PSFCH |  |  | 12 | 12 | 12 |
| DMRS symbols for slot with PSFCH |  |  | 2 | Option 1: 2 Option 2: 3 | 3 |
| DMRS symbols for slot without PSFCH |  |  | 2 | Option 1: 3 Option 2: 4 | 4(Note 2) |
| Modulation order |  |  | 6 | 4 | 2 |
| MCS index |  |  | 17 | 11 | 4 |
| 2nd stage SCI format 2-A configuration | Payloads | Bits | 35 | 35 | 35 |
|  | α |  | 1 | 1 | 1 |
|  | βoffset |  | [5] | [5] | [3.5] |
| Transport Block Size for slot with PSFCH |  | Bits |  |  |  |
| Transport Block Size for slot without PSFCH |  | Bits |  |  |  |
| Transport block CRC |  | Bits | 24 | 24 | 24 |
| Maximum number of HARQ transmissions |  |  | 1 | 1 | 1 |
| Binary Channel Bits for slots with PSFCH |  |  |  |  |  |
| Binary Channel Bits for slots without PSFCH |  | Bits |  |  |  |
| PSFCH resource period |  | Slot | Option 1: 1 Option 2: 4 | 4 | Option 1: 1 Option 2: 4 |
| MinTimeGapPSFCH |  | Slot | [3] | [3] | [3] |
| Antenna configuration |  |  | 1x2 | 1x2 | 1x2 |
| Performance metric |  |  | SNR@10% BLER of PSSCH |  |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and PSFCH symbols. Note 2: It is only applied to cases when PSFCH resource period =1 |  |  |  |  |  |