3GPP TSG-RAN WG4 meeting #98-e	R4-2101435

Electronic Meeting, 25 January – 5 February 2021

Agenda Item:	7.16.1.2

Source:	Ericsson

Title:	Simulation results for Rel-15 Type II PMI reporting test

Document for:	Information

# 1	Introduction

RAN4#97-e agreed with the way forward on Rel-15 Type-II PMI reporting requirements [1]. This contribution provides our simulation results according to the way forward and simulation assumption [2].

# 2	Simulation results

## 2.1	FDD with SU-MIMO scheduling

Figure 1 and Figure 2 show the PDSCH simulation results with the reported Rel-15 Type-II PMI and with the reported/random Rel-15 Type-I PMI in FDD. Table 1 shows the throughput ratio of followed PMI and random PMI with the SNR test points corresponding to the 70/90/95% of the maximum throughput with the followed PMI.

Figure 1	FDD PDSCH throughput with reported/random PMI with medium antenna correlation.

Figure 2	FDD PDSCH throughput with reported/random PMI with low antenna correlation.

Table 1	Summary of simulation results for FDD.

| Antenna configuration | correlation | Codebook Follow mode | SNR [dB 95% max TP] | Gain [95% max TP] | SNR [dB 90% max TP] | Gain [90% max TP] | SNR [dB 70% max TP] | Gain [70% max TP] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16Tx / 2Rx | Medium | Type II | 14.5 | 1.6 | 12.4 | 1.9 | 7.8 | 2.2 |
|  |  | Type I | 17.7 | 1.3 | 15.7 | 1.5 | 10.8 | 1.6 |
|  | Low | Type II | 12.8 | 1.7 | 10.9 | 1.8 | 7.2 | 2.1 |
|  |  | Type I | 13.8 | 1.5 | 12.0 | 1.6 | 8.5 | 1.7 |
| 16Tx / 4Rx | Medium | Type II | 7.6 | 2.1 | 6.8 | 2.1 | 4.4 | 2.2 |
|  |  | Type I | 10.2 | 1.6 | 9.1 | 1.7 | 6.4 | 1.7 |
|  | Low | Type II | 7.8 | 1.9 | 6.7 | 1.8 | 4.3 | 1.9 |
|  |  | Type I | 9.6 | 1.5 | 8.6 | 1.6 | 6.0 | 1.4 |

## 2.2	TDD with SU-MIMO scheduling

Figure 3 and Figure 4 show the PDSCH simulation results with the reported Rel-15 Type-II PMI and with the reported/random Rel-15 Type-I PMI in TDD. Table 2 shows the throughput ratio of followed PMI and random PMI with the SNR test points corresponding to the 70/90/95% of the maximum throughput with the followed PMI.

Figure 3	TDD PDSCH throughput with reported/random PMI with medium antenna correlation.

Figure 4	TDD PDSCH throughput with reported/random PMI with low antenna correlation.

Table 2	Summary of simulation results for TDD.

| Antenna configuration | correlation | Codebook | SNR [dB 95% max TP] | Gain [95% max TP] | SNR [dB 90% max TP] | Gain [90% max TP] | SNR [dB 70% max TP] | Gain [70% max TP] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16Tx / 2Rx | Medium | Type II | 10.3 | 2.2 | 8.8 | 2.7 | 5.7 | 3.1 |
|  |  | Type I | 11.5 | 2.0 | 10.4 | 2.1 | 8.1 | 2.3 |
|  | Low | Type II | 10.0 | 2.0 | 7.9 | 2.3 | 5.4 | 2.8 |
|  |  | Type I | 10.3 | 1.9 | 8.8 | 2.1 | 6.4 | 2.2 |
| 16Tx / 4Rx | Medium | Type II | 5.3 | 2.4 | 4.5 | 2.5 | 2.5 | 2.6 |
|  |  | Type I | 5.8 | 2.2 | 5.0 | 2.3 | 2.8 | 2.5 |
|  | Low | Type II | 5.4 | 1.9 | 4.6 | 2.0 | 2.5 | 1.9 |
|  |  | Type I | 6.7 | 1.6 | 5.7 | 1.7 | 3.4 | 1.8 |

# References

- R4-2017678, “Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32​​”, Ericsson, Samsung.

- R4-2017563, “Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports”, Ericsson.