3GPP TSG-RAN WG4 Meeting # 98-e 	    R4-2101761

Electronic Meeting 25th January – 05th February 2021

Agenda Item:		7.7			NR Positioning Support
7.7.2.3.3	gNB Rx-Tx time difference requirements

Source: 	Ericsson

Title:	gNB Rx-Tx measurement analysis

Document for:	Discussion

# Introduction

In last meetings WF it was agreed upon that measurement accuracy requirements for gNB Rx-Tx time difference shall be defined [1].

In this contribution, aspects are discussed that lead to the definition of such requirements and a structure for accuracy requirements definition is proposed.

# Discussion gNB Rx-Tx measurement analysis

While it was agreed, that measurement accuracy requirements shall only be defined for a subset of suitable subset of SRS configurations [1] and also that such measurement accuracy requirements are optional and only to be met if declared by manufacturer (uphold earlier agreement), this paper shall implement a thought process on what measurement accuracy requirement can be expected from such selected subset.

As it was proposed [2], timing measurement accuracy will be derived via link level simulation, implementing a defined set of Ês/Iot values, multiple subsets of SRS configurations and channel models. The link level simulation results as an outcome will give a ballpark figure about the timing measurement accuracy, while neglecting all other timing-related impairments posed by the network, which should be considered as a margin on top and are FFS.

Since in previous studies, simulations and agreements [2], timing measurement accuracy is highly dependent on SRS configuration, especially the number of allocated RBs per SRS, i.e. SRS bandwidth, different gNB Rx-Tx measurement accuracy requirements should be defined for different suitable subsets of SRS configurations. Therefore, it is proposed to include following tables concerning gNB Rx-Tx absolute measurement accuracy in TS 38.133.

Proposal 1: gNB Rx-Tx time difference measurement absolute accuracy in FR1

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SRS Ês/Iot | Minimum SRS bandwidth Note 3 | Io Note 4 range |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io Note 1 |  | Maximum Io |
| Tc Note 5 | dB | RB |  | dBm / SCSSRS |  | dBm/BWChannel |
|  |  |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz |  |
| TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5, excluding SDL bands. NOTE 3: SRS bandwidth is TBD. NOTE 4: The Io is defined in SRS slots. The same Io range applies to SRS and non-SRS symbols. Io levels are different in SRS and non-SRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |  |  |  |  |

Proposal 2: gNB Rx-Tx time difference measurement absolute accuracy in FR2

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SRS Ês/Iot | Minimum SRS bandwidth Note 3 | Io Note 4 range |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io Note 1 |  | Maximum Io |
| Tc Note 5 | dB | RB |  | dBm / SCSSRS |  | dBm/BWChannel |
|  |  |  |  | SCSSRS = 60 kHz | SCSSRS = 120 kHz |  |
| TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5, excluding SDL bands. NOTE 3: SRS bandwidth is TBD. NOTE 4: The Io is defined in SRS slots. The same Io range applies to SRS and non-SRS symbols. Io levels are different in SRS and non-SRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |  |  |  |  |

Link level simulation results presented in RAN4#97-e [3] also point out that the outcome of the link level simulation, a theoretical gNB Rx-Tx measurement accuracy displayed in units of Tc is strongly dependent on the Ês/Iot value. The range of TgNB-RX error values, even for same SRS bandwidths is very high, in some cases ranging from a very good accuracy result, i.e. few tens of Tc to a very bad one, i.e. a few thousands of Tc, considering applicability.

Since the number of permutations in configurations for such link level simulations is very high, one should agree on a set of Ês/Iot values first, to narrow the manifoldness in results and come to an agreement with gNB Rx-Tx measurement accuracy more easily.

Proposal 3: Fix and agree on set of Ês/Iot values as input for link level simulations in RAN4#98-e for better alignment of the accuracy results.

Furthermore, the preliminary results for UE Rx-Tx measurement accuracy can be studied as comparison. As an outcome of corresponding link level simulations and proposed in [4], the FR1 UE Rx-Tx accuracy in units of Tc for different PRS bandwidths for example ranges between [±20…±60]Tc for an Ês/Iot of -3dB, and between [±30…±90]Tc for an Ês/Iot of -13dB.

Further link level simulations, under given and agreed set of Ês/Iot values, shall comprise and lead to a conclusion if a similar value ranges can be expected for gNB Rx-Tx measurement accuracy.

# Conclusion

Proposal 1: gNB Rx-Tx time difference measurement absolute accuracy in FR1

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SRS Ês/Iot | Minimum SRS bandwidth Note 3 | Io Note 4 range |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io Note 1 |  | Maximum Io |
| Tc Note 5 | dB | RB |  | dBm / SCSSRS |  | dBm/BWChannel |
|  |  |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz |  |
| TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5, excluding SDL bands. NOTE 3: SRS bandwidth is TBD. NOTE 4: The Io is defined in SRS slots. The same Io range applies to SRS and non-SRS symbols. Io levels are different in SRS and non-SRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |  |  |  |  |

Proposal 2: gNB Rx-Tx time difference measurement absolute accuracy in FR2

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SRS Ês/Iot | Minimum SRS bandwidth Note 3 | Io Note 4 range |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io Note 1 |  | Maximum Io |
| Tc Note 5 | dB | RB |  | dBm / SCSSRS |  | dBm/BWChannel |
|  |  |  |  | SCSSRS = 60 kHz | SCSSRS = 120 kHz |  |
| TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5, excluding SDL bands. NOTE 3: SRS bandwidth is TBD. NOTE 4: The Io is defined in SRS slots. The same Io range applies to SRS and non-SRS symbols. Io levels are different in SRS and non-SRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. |  |  |  |  |  |  |

Proposal 3: Fix and agree on set of Ês/Iot values as input for link level simulations in RAN4#98-e for better alignment of the accuracy results.

# References

[1]		R4-2017159	WF on gNB requirements for NR positioning.

[2]		R4-2012140	WF on gNB requirements for NR positioning.

[3]		R4-2016154	gNB Positioning UL SRS Link Level Simulation Results

[4]		R4-2016406	On UE Rx-Tx measurement accuracy