3GPP TSG-RAN WG4 Meeting # 98-e	R4-2101792

Electronic Meeting, 25 January – 5 February 2021

Agenda Item:	10.1.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals of BS Parameters for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Document for:	Approval

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

-	6425-7025MHz and 7025-7125MHz.

-	10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE in these frequencies. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence study will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective. The WF on the simulation assumptions for the coexistence study was agreed in RAN4#95-e [7] and RAN4#96-e [8], and the WF on BS and UE parameters was agreed in RAN4#97-e [9] where some BS parameters are provisional or TBD.

This contribution provides the proposals of the open BS parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz according to the downlink and uplink coexistence simulation results provided in [10-13].

# 2.	Discussion

2.1	BS ACLR and ACS

The downlink coexistence simulation results provided in [12] have shown that the downlink throughput loss of the victim UE in the simulated indoor hotspot scenario can still be limited to 5% with a downlink ACIR offset of -5dB at both 7GHz and 10GHz (i.e. 5dB less stringent ACIR). Therefore, it is proposed to apply the BS ACLR agreed in the WF [9] to the Small cell indoor/Indoor urban scenario, i.e. 38dB and 37dB for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, respectively.

The uplink coexistence simulation results provided in [13] have shown that the uplink throughput loss of the victim UE in the simulated indoor hotspot scenario can still be limited to 5% with an uplink ACLR offset of -5dB at both 7GHz and 10GHz (i.e. 5dB less stringent ACLR). Therefore, it is proposed to apply the BS ACS agreed in the WF [9] to the Small cell indoor/Indoor urban scenario, i.e. 42dB and 40dB for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, respectively.

2.2	BS Spectral mask

It was agreed in the WF [9] that:

- Basic limits for bands above 1 GHz in clauses 6.6.4.2 and 9.7.4.2 of TS 38.104 with Frequency offset step size of [5..50] MHz for 6.425-7.125 GHz and 10.0-10.5 GHz

Considering the more relaxed BS ACLR agreed in the WF [9], and the expectation that the channel bandwidth for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz should probably be wider than 5MHz, e.g. the minimum channel bandwidth specified for operating band n96 is 20MHz [3,5], it is proposed to specify 20MHz as the Frequency offset step size of the BS Spectral mask.

2.3	BS (general) in-band blocking

It was agreed in the WF [9] that:

- Lower interfering signal mean power compared to those in clauses 7.4.2.2 and 10.5.2.2 of TS 38.104: [TBD]dB for 6.425-7.125 GHz and [TBD]dB for 10.0-10.5 GHz

The uplink coexistence simulation results provided in [11] have shown that the 99.99%-tile received blocking signal power levels at the TAB connector (i.e. including the antenna array gain) are around -40dBm and -39dBm, respectively, at 7GHz and 10GHz in the simulated urban macro scenario. Hence there is no room for in-band blocking relaxation at 7GHz and 10GHz comparing to the current FR1 BS (general) in-band blocking requirements in this scenario. Moreover, the current FR1 BS (general) in-band blocking requirements have also been agreed to apply to operating band n96 [5]. Therefore, it is proposed to keep the currently specified FR1 BS (general) in-band blocking requirements for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, i.e. [TBD] should be 0. Whether the currently specified FR1 BS in-band narrowband blocking requirements should be kept for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz should be decided according to the coexisting systems in each operating band.

2.4	BS out of band blocking

It was agreed in the WF [9] that:

- [-15] dBm CW interferer applies from 1MHz to FUL,low – [60..200]MHz and from FUL,high + [60..200]MHz up to 26000 MHz

For operating band n96, it has been agreed for BS type 1-H and 1-C that [5]:

•	-15 dBm CW interferer applies from 1MHz to FUL,low – 500MHz and from FUL,high + 500MHz up to 12750 MHz

•	-35 dBm CW interferer applies from FUL,low – 500MHz to FUL,low – 70MHz and from FUL,high + 70MHz up to FUL,high + 500MHz

For consistency, it is proposed to apply the above BS out of band blocking requirements for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

2.5	BS ΔfOBUE

It was agreed in the WF [9] that:

- [40-100] MHz for 6.425-7.125GHz and 10-10.5GHz

- Operating band characteristics should be considered

For operating band n96, it has been agreed to use ΔfOBUE of 50 MHz for BS type 1-H and 1-C [5]. For consistency, it is proposed to specify ΔfOBUE of 50 MHz for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

# 3.	Conclusion

This contribution provides the proposals of the open BS parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz according to the downlink and uplink coexistence simulation results provided in [10-13].

Proposals for approval:

1) To apply the BS ACLR agreed in the WF [9] to the Small cell indoor/Indoor urban scenario, i.e. 38dB and 37dB for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, respectively.

2) To apply the BS ACS agreed in the WF [9] to the Small cell indoor/Indoor urban scenario, i.e. 42dB and 40dB for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, respectively.

3) To specify 20MHz as the Frequency offset step size of the BS Spectral mask for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

4) To keep the currently specified FR1 BS (general) in-band blocking requirements for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

5) Whether the currently specified BS in-band narrowband blocking requirements below 6GHz should be kept for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz should be decided according to the coexisting systems in each operating band.

6) To apply the following BS out of band blocking requirements for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz:

•	-15 dBm CW interferer applies from 1MHz to FUL,low – 500MHz and from FUL,high + 500MHz up to 12750 MHz

•	-35 dBm CW interferer applies from FUL,low – 500MHz to FUL,low – 70MHz and from FUL,high + 70MHz up to FUL,high + 500MHz

7) To specify ΔfOBUE of 50 MHz for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v16.3.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	R4-2008928, “TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[8]	R4-2011827, “WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Nokia.

[9]	R4-2016905, “WF on BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz”, Nokia, CATT, Ericsson, ZTE, Huawei, Qualcomm.

[10]	R4-2014476, “Downlink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[11]	R4-2014477, “Uplink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[12]	R4-2101794, “Downlink Indoor Hotspot Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[13]	R4-2101795, “Uplink Indoor Hotspot Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.