3GPP TSG-RAN WG4 Meeting #98-e	R4-2101895

Electronic Meeting, 25 January – 5 February 2021

Source:	Ericsson

Title:	TP for TR 37.717-31-11: Including band combinations DC_3-20-40_n78

Agenda item:			9.5.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-31-11 to add DC_3A-20A-40A_n78A, 3A-20A-40C_n78A, 3A-20A-40A_n78(2A), 3A-20A-40C_n78(2A)

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_3-20-40_n78

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-20-40_n78 | CA_3-20-40 | n78 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-20A-40A_n78A DC_3A-20A-40C_n78A | DC_3A_n78A DC_20A_n78A DC_40A_n78A | CA_3A-20A-40A CA_3A-20A-40C | n78A |
| DC_3A-20A-40A_n78(2A) DC_3A-20A-40C_n78(2A) | DC_3A_n78A DC_20A_n78A DC_40A_n78A | CA_3A-20A-40A CA_3A-20A-40C | CA_n78(2A) |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_3-20-40_n78, the TIB,c and RIB,c values are given in the tables below based on values DC_3-28_n40-n78.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20-40_n78 | 3 | 0.6 |
|  | 20 | 0.5 |
|  | 40 | 0.36 |
|  | n78 | 0.86 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx |  |  |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20-40_n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.1.x.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

---End of changes---

# Reference

[1]		RP-202188, “Revised WID on Rel-17 Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)” Ericsson