3GPP TSG-RAN WG4 Meeting#98-e                                 R4-2101948

E-meeting,25th Jan – 5th Feb, 2021, 2021

Agenda item:	10.1.1

Source: 	ZTE Corporation

Title: 	TP to TR 38.921 UE transmitter requirements

Document for:	Approval

Introduction

In the RAN4#97e meeting, WF for BS and UE parameter for 6.425-7.125 and 10.0-10.5 GHz were approved, however there are still lots of open issues left for further study, therefore in this contribution, we want to share some analysis for those remaining issues from UE perspective.

Discussion

## 2.1. SEM requirements

In the last RAN4#97e meeting, ACLR/ACS requirements for UE RF has been approved in WF [1] based on coexistence evaluation and feasibility study, however as indicated in the WF, out of band emission requirements or SEM requirements for UE is not discussed yet until this meeting. Based on the approved ACLR requirements in Table 1 in last RAN4 meeting, then certain relaxation for UE SEM should also been allowed considering the relationship between SEM and ACLR even thought it’s one by one associated with each other.

Table 1.1. UE ACLR requirements for 6425-7125MHz and 10-10.5GHz

| Frequency range | ACLR [dB] | ACS [dB] |
| --- | --- | --- |
| 6425-7125MHz | 26 | 32 |
| 10-10.5GHz | 24 | 31 |

Proposal 1: certain relaxation for UE SEM requirement could be allowed based on the approved ACLR requirements.

## 2.2. ACS requirements

Adjacent channel selectivity (ACS) is agreed as 32dBc for 6425-7125MHz and 31dBc for 10-10.5GHz, therefore the corresponding ACS requirements could be defined as following Table 2.2-1/2/3.

Table 2.2-1: ACS for 6425-7125MHz and 10-10.5GHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| ACS for 7GHz | dB | 32 | 32 | 32 | 32 | 32 |
| ACS for 10GHz | dB | 31 | 31 | 31 | 31 | 31 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| ACS for 7GHz | dB | 32 | 32 | 32 | 32 |  |
| ACS for 10GHz | dB | 31 | 31 | 31 | 31 |  |

Table 2.2-2: Test parameters for 6425-7125MHz and 10-10.5GHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer for 7GHz | dBm | REFSENS + 44.5 dB |  |  |  |  |
| Pinterferer for 10GHz | dBm | REFSENS + 43.5 dB |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 40 | 50 |
| Finterferer (offset) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 40 / -40 | 50 / -50 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer for 7GHz | dBm | REFSENS + 44.5 dB | REFSENS + 44.5 dB | REFSENS + 44.5 dB | REFSENS + 44.5 dB |  |
| Pinterferer for 10GHz | dBm | REFSENS + 43.5 dB | REFSENS + 43.5 dB | REFSENS + 43.5 dB | REFSENS + 43.5 dB |  |
| BWinterferer | MHz | 60 | 80 | 90 | 100 |  |
| Finterferer (offset) | MHz | 60 / -60 | 80 / -80 | 90 / -90 | 100 / -100 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 in TS 38.101-1. |  |  |  |  |  |  |

Table 2.2-3: Test parameters for 6425-7125MHz and 10-10.5GHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Power in transmission bandwidth configuration for 7GHz | dBm | -55.5 |  |  |  |  |
| Power in transmission bandwidth configuration for 10GHz |  | -54.5 |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 40 | 50 |
| Finterferer (offset) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 40 / -40 | 50 / -50 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Power in transmission bandwidth configuration for 7GHz | dBm | -55.5 |  |  |  |  |
| Power in transmission bandwidth configuration for 10GHz |  | -54.5 |  |  |  |  |
| Pinterferer | dBm | -25 | -25 | -25 | -25 |  |
| BWinterferer | MHz | 60 | 80 | 90 | 100 |  |
| Finterferer (offset) | MHz | 60 / -60 | 80 / -80 | 90 / -90 | 100 / -100 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 in TS 38.101-1. |  |  |  |  |  |  |

Proposal 2: to define ACS requirements for 6425-7125MHz and 10-10.5GHz.

Conclusions

In this contribution, we shared updated simulation results for 6.425-7.125GHz and 10.0-10.5GHz for further discussion.

Proposal 1: certain relaxation for UE SEM requirement could be allowed based on the approved ACLR requirements.

Proposal 2: to define ACS requirements for 6425-7125MHz and 10-10.5GHz.

References

- R4-2016905, WF on BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz, Nokia, approved.

- TR 36.803

TP to TR 38.921

< START OF CHANGE>

## 7.1	Transmitter characteristics

### 7.1.2	Spectral mask

## Based on the approved ACLR requirements in Table 7.1.3, then certain relaxation for UE SEM should also been allowed considering the relationship between SEM and ACLR.

## 7.2	Receiver characteristics

### 7.2.3	Blocking response

### The blocking characteristic specified in clause 7.6 of TS 38.101-1 for frequency larger than 3300 MHz could be applied for 6425-7125MHz and 10-10.5GHz.

### 7.2.4	ACS

Adjacent channel selectivity (ACS) is agreed as 32dBc for 6425-7125MHz and 31dBc for 10-10.5GHz, therefore the corresponding ACS requirements could be defined as following Table 7.2.4-1/2/3.

Table 7.2.4-1: ACS for 6425-7125MHz and 10-10.5GHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| ACS for 7GHz | dB | 32 | 32 | 32 | 32 | 32 |
| ACS for 10GHz | dB | 31 | 31 | 31 | 31 | 31 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| ACS for 7GHz | dB | 32 | 32 | 32 | 32 |  |
| ACS for 10GHz | dB | 31 | 31 | 31 | 31 |  |

Table 7.2.4-2: Test parameters for 6425-7125MHz and 10-10.5GHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer for 7GHz | dBm | REFSENS + 44.5 dB |  |  |  |  |
| Pinterferer for 10GHz | dBm | REFSENS + 43.5 dB |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 40 | 50 |
| Finterferer (offset) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 40 / -40 | 50 / -50 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer for 7GHz | dBm | REFSENS + 44.5 dB | REFSENS + 44.5 dB | REFSENS + 44.5 dB | REFSENS + 44.5 dB |  |
| Pinterferer for 10GHz | dBm | REFSENS + 43.5 dB | REFSENS + 43.5 dB | REFSENS + 43.5 dB | REFSENS + 43.5 dB |  |
| BWinterferer | MHz | 60 | 80 | 90 | 100 |  |
| Finterferer (offset) | MHz | 60 / -60 | 80 / -80 | 90 / -90 | 100 / -100 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 in TS 38.101-1. |  |  |  |  |  |  |

Table 7.2.4-3: Test parameters for 6425-7125MHz and 10-10.5GHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Power in transmission bandwidth configuration for 7GHz | dBm | -55.5 |  |  |  |  |
| Power in transmission bandwidth configuration for 10GHz |  | -54.5 |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 40 | 50 |
| Finterferer (offset) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 40 / -40 | 50 / -50 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Power in transmission bandwidth configuration for 7GHz | dBm | -55.5 |  |  |  |  |
| Power in transmission bandwidth configuration for 10GHz |  | -54.5 |  |  |  |  |
| Pinterferer | dBm | -25 | -25 | -25 | -25 |  |
| BWinterferer | MHz | 60 | 80 | 90 | 100 |  |
| Finterferer (offset) | MHz | 60 / -60 | 80 / -80 | 90 / -90 | 100 / -100 |  |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 in TS 38.101-1. |  |  |  |  |  |  |

< End OF CHANGE>