3GPP TSG-RAN WG4 Meeting#98-e                                 R4-2101951

E-meeting,25th Jan – 5th Feb, 2021

Agenda item:	10.1.3.2

Source: 	ZTE Corporation

Title: 	DL simulation results for 6425-7125MHz and 10-10.5GHz

Document for:	Discussion

Introduction

In the RAN4#97e meeting, WF on simulation assumption [1] was approved for indoor coexistence evaluation work. Therefore in this contribution, we share some simulation results for further discussion.

Discussion

In the following sections, simulation results for both omni based and AAS based BS in indoor scenarios are provided based on the approved simulation assumption agreed on the reflector. In addition, baseline ACIR for demonstration based on legacy FR1 NR system is assuming that using ACLR=45dBc at BS and ACS=33dBc at UE) at 7GHz and 10GHz, relative offset to baseline ACIR is used for further comparison and check whether relative relaxed requirements could be achieved.

## 2.1. Downlink

The DL simulation results for 7GHz and 10GHz are summarized in the following Table 1 and ACIR results are shown in Figure 1/2/3/4.

Table 1: DL throughput loss of victim UE at 7GHz and 10GHz

| Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| Omni based BS | Average throughput loss in % (7GHz) | 0.1371 | 0.1724 | 0.2168 | 0.2726 | 0.3425 | 0.4302 |
|  | 5%-tile throughput loss in % (7GHz) | 0.1429 | 0.1798 | 0.2263 | 0.2847 | 0.3123 | 0.3427 |
| AAS based BS | Average throughput loss in %(7GHz) | 0.3043 | 0.3817 | 0.4775 | 0.5947 | 0.7378 | 0.9123 |
|  | 5%-tile throughput loss in % (7GHz) | 0.0054 | 0.0067 | 0.0085 | 0.0107 | 0.0135 | 0.1421 |
| Omni based BS | Average throughput loss in % (10GHz) | 0.1561 | 0.1956 | 0.2447 | 0.3058 | 0.3815 | 0.4750 |
|  | 5%-tile throughput loss in % (10GHz) | 0.1096 | 0.1380 | 0.1736 | 0.2369 | 0.3323 | 0.4823 |
| AAS based BS | Average throughput loss in % (10GHz) | 0.3188 | 0.3979 | 0.4950 | 0.6133 | 0.7587 | 0.9337 |
|  | 5%-tile throughput loss in % (10GHz) | 0.0481 | 0.0553 | 0.0645 | 0.0760 | 0.0969 | 0.1978 |

Figure 1.DL SINR in indoor scenario with omni based BS @7GHz

Figure 2. DL SINR in indoor scenario with AAS based BS @7GHz

Figure 3. indoor scenario with omni based BS @10GHz

Figure 4. indoor scenario with AAS based BS @10GHz

Based on the initial simulation results, agreed DL ACIR 31.0dB for 7GHz and 30.0dB for 10GHz in last RAN4 meeting could fulfill the coexistence requirements for indoor scenario, therefore we propose to reuse ACLR/ACS requirements of Urban macro scenario for that of indoor scenario.

Conclusions

In this contribution, we shared simulation results for 6.425-7.125GHz and 10.0-10.5GHz for further discussion and proposals are made as following:

Proposal 1: reuse the ACLR/ACS requirements of urban macro for that of indoor case.

References

[1] R4-2016904,WF on Coexistence Simulations results for 6.425-7.125 GHz and 10.0-10.5 GHz, approved.