3GPP TSG-RAN WG4 Meeting #98-e	R4-2102316

Electronic Meeting, 25 January – 5 February 2021

Source:	Ericsson, Bell Mobility

Title:	TP for TR 38.717-03-02: CA_n25-n41-n66

Agenda item:			9.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include CA_n25A-n41A-n66A.

# 2. Text Proposal

# ---Start of changes---

### 5.1.x	CA_n25-n41-n66

#### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n25-n41-n66 | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | n66 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.1.x.3	UE co-existence studies

UL n25-n41 have no IMD towards DL n66.

UL n25-n66 have no IMD towards DL n41.

UL n41-n66 have IMD4 towards DL n25.

#### 5.1.x.4	REFSENS requirements

IMD4 MSD values are derived from DC_2A-4A_n41A.

Below are the updates needed in Table 7.3A.5-2 of TS 38.101-1.

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25A-n41A-n66A | n25 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 | IMD4 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | N/A | N/A |

---End of changes---

# Reference

[1] 		RP-202201, “Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL”, ZTE