3GPP TSG-RAN WG4 Meeting #97-e                                          R4-2102392

Electronic Meeting, 2-13 Nov., 2020

Source: 	Huawei, Hisilicon

Title: 	System performance evaluation for FDD HPUE

Agenda Item:	12.5.5

Document for:	Approval

# Introduction

SI on high power UE (power class 2) for one NR FDD band was approved in RAN#90e meeting [1]. One objective related to simulation evaluation is:

Evaluate system performance gains on spectrum efficiency, and other metrics if needed could also be taken into account, to support NR FDD HPUE.

This contribution provides our consideration on simulation assumptions and the simulation results.

# Discussion

## Simulation assumptions

Initial system level simulation was performed in [2]. During the RAN discussion, example band n1 and n3 are considered in the SI.

The simulation assumptions used in [2] only considers band n1 and the antenna configuration at TRxP is 4Rx. Considering FDD massive MIMO is widely available nowadays, we think that 32Rx should be considered as well. Also the simulation should include both band n1 and n3.

Table 1 provides the updated simulation assumptions based on [2].

Table 1: Simulation assumptions

| Configuration parameters | Values |
| --- | --- |
| Scenario | Urban macro |
| ISD | 500 m |
| Duplexing | FDD |
| Carrier frequency | 1.8 GHz, 2.1GHz |
| Modulation | Up to 256QAM |
| Numerology | 15 kHz |
| Simulation bandwidth | 20 MHz |
| Transmission scheme | SU-MIMO |
| Codebook | For 2Tx, codebook [1 1]T is used for transmit diversity |
| SU dimension | 1 layer |
| Antenna configuration at TRxP | 4Rx, (M,N,P,Mg, Ng) = (1,2,10,1,1; 1,2) 32Rx, (M,N,P,Mg, Ng) = (8,8,2,1,1; 2,8) |
| Antenna configuration at UE | 1Tx, (M,N,P,Mg, Ng) = (1,1,1,1,1; 1,1), 2Tx, (M,N,P,Mg, Ng) = (1,1,2,1,1; 1,1), |
| UE maximal transmit power | For 1Tx, 23 dBm for each TXRU For 1Tx, 26 dBm for each TXRU (High power UE) For 2Tx, 23 dBm for each TXRU (High power UE) |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Ideal |
| Power control parameter | P0=-60, alpha = 0.6 |
| TRxP number per site | 3 |
| TRxP number | 21 |
| Channel model | UMa following TR 38.901 |
| Electronic tilt | 102° |
| Traffic model | FTP3, packet size: 100k / 10k Byte, arrival rate: 1 packet / s. |

Proposal 1: It is proposed to agree on the above simulation assumptions.

### Simulation results

Since the simulation for band n1 was already done, we only provide the simulation results for band n3 with assumption of 32Rx for Antenna configuration at TRxP.

Table 2 provides the simulation results.

Table 2: Simulation results

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/s | 156.2% | 102.7% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/s | 156.5% | 104.0% |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/s | 164.7% | 107.5% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/s | 165.2% | 109.5% |

Observation 1: From the simulation results, obvious performance gain on cell average throughput is observed.

# Conclusion

This contribution provides simulation assumptions and simulation results on system performance evaluation for FDD HPUE. Based on the results, we see that obvious performance gain.

Observation 1: From the simulation results, obvious performance gain on cell average throughput is observed.

Proposal 1: It is proposed to agree on the above simulation assumptions.

# References

[1] RP-202870, New SID Study on high power UE (power class 2) for one NR FDD band, China Unicom

[2] RP-202285 Motivation for Study on high power UE (power class 2) for one NR FDD band, China Unicom