3GPP TSG-RAN WG4 Meeting #98e                         	             R4-2102401

# Electronic meeting, 25th January – 5th February, 2021

Agenda item:	12.1.7

Source:	Huawei, HiSilicon

Title:	Analysis on reduce test time for FR2

Document for:	Approval

# Introduction

“Reduce test time” is one topic of FR2 test method enhancement. According to the discussion in RAN4#97-e meeting, the change of UE antenna array from 8x2 to 4x2 array was considered as a potential direction to reduce the number of required measurement grids. Especially for PC3 peak searching, the currently required number of measurement grids is 800 with constant density, and 1106 with constant step size, which is very time consuming. As suggested by the WF [1], cited as below for convenience, in this document we present our simulation results comparing measurement uncertainty of 8x2 and 4x2 array for different measurement grid configurations.

- RAN4 agrees to further study the following Testability enhancements approaches to reduce test time

- Option1: As part of the enhanced test methods for FR2 study item, RAN4 should discuss beam sweeping techniques further

- Option2: Adopt 4x2 array as the antenna assumption for deriving measurement grid for PC3, especially for smart phone UE.

- Option3: Develop two sets of measurement grids for PC3, one is the same as current grids based on 8x2 array assumption, the other is relatively sparse grids based on 4x2 assumption. Applicability depends on UE declaration

- …

- Next steps for testability enhancements to reduce test time

- Detailed parameters of 4x2 array antenna assumption for PC3 should be aligned next meeting

- Simulated new measurement grids with estimated testing time based on 4X2 antenna array are encouraged

- …

# Discussion

## 2.1 Simulation assumptions

The simulation assumptions are referred to [3], including

- Assume two antenna arrays integrated in the UE (front and back). The implementation loss for the antenna near the front is 5dB less than for the antenna near the back.

Figure 1: Illustration of the two antenna arrays integrated in the UE (from [3])

- Beam Steering Assumptions for both 8x2 and 4x2 antenna arrays are as below. In the simulation the optimal beam (with maximum EIRP) is selected according to the tested direction.

- In the xz plane, assume 45o beam steering granularity (from 45o to 135o)

- In the xy plane, assume 22.5o beam steering granularity (from -90o to 90o)

- Antenna Radiation Patterns: 260deg/130deg HPBW as in table 1 and table 2 of [3]. Because the large loss of the radio signal to penetrate the UE body i.e. battery, PCB etc., it is assumed that the front hemisphere is served by the front antenna array, and vice versa.

## 2.2 Simulation results and analysis

Similar as the simulations in TR38.810 section “G.2	Beam Peak Search Measurement Grids”, in order to estimate the measurement uncertainty of beam peak EIRP, the results are derived from a set of 10000 random relative orientations between the simulated UE and the measurement grid.

The simulation results for different UE antenna configurations (8x2 or 4x2) are list in below table 1, including the mean error, the standard deviation, and the offset at which the CDF is 5%. CDFs of Mean Error are further plotted in figure 1.

Table 1: Statistical Analyses of 10k simulations for the constant step size grids

|  | 8x2 |  |  | 4x2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number of unique grid points | Mean Error [dB] | STD [dB] | Offset5%CDF [dB] | Mean Error [dB] | STD [dB] | Offset5%CDF [dB] |
| 100 | 0.51 | 0.31 | 1.10 | 0.33 | 0.21 | 0.75 |
| 200 | 0.29 | 0.17 | 0.60 | 0.17 | 0.09 | 0.34 |
| 300 | 0.25 | 0.16 | 0.55 | 0.14 | 0.09 | 0.31 |
| 400 | 0.20 | 0.13 | 0.44 | 0.12 | 0.07 | 0.25 |
| 500 | 0.15 | 0.09 | 0.32 | 0.10 | 0.05 | 0.18 |
| 600 | 0.14 | 0.08 | 0.29 | 0.08 | 0.05 | 0.16 |
| 700 | 0.13 | 0.09 | 0.31 | 0.08 | 0.05 | 0.16 |
| 800 | 0.13 | 0.09 | 0.28 | 0.07 | 0.05 | 0.15 |

Figure 1: CDF of Mean Error with 10k simulations for the constant step size grids

Based on the criteria used in TR38.810 to determine the required grid number for ‘Beam Peak Search’, i.e. “the measurement grids leading to a systematic error of "Beam Peak Search" of 0.5 dB (offset from Beam Peak at which CDF is 5%”), we have following observations according to above simulation results,:

Observation 1: compared with 8x2, 4x2 UE antenna array assumption can significantly reduce the required grid number. For constant density grid, around 200 grids points seem to be able to meet the 0.5dB systematic error criteria.

Observation 2: in case of 8x2 UE antenna array assumption, the simulation assumptions used in this document significantly reduce the required grid number, i.e. around 400 compared with 800 in TR38.810 Table G.2.3-3 for constant density grid.

Based on above observations, we propose that

Proposal 1: in the WF [1], option2 and option3 with 4x2 array should be further studied considering the above results that 4x2 antenna array can significantly reduce the required number of measurement grids, compared with 8x2.

Proposal 2: beside the “Detailed parameters of 4x2 antenna array assumption for PC3 should be aligned in next meeting” [1], the simulation assumptions for 8x2 antenna array may also need to be discussed considering the above results that different assumptions can have very different required number of measurement grids, for the same 8x2 antenna array.

# 3	Conclusions

In this document, we present our simulation results and analysis on required number of measurement grids for FR2 UE beam peak searching

Observation 1: compared with 8x2, 4x2 UE antenna array assumption can significantly reduce the required grid number. For constant density grid, around 200 grids points seem to be able to meet the 0.5dB systematic error criteria.

Observation 2: in case of 8x2 UE antenna array assumption, the simulation assumptions used in this document significantly reduce the required grid number, i.e. around 400 compared with 800 in TR38.810 Table G.2.3-3 for constant density grid.

Proposal 1: in the WF [1], option2 and option3 with 4x2 array should be further studied considering the above results that 4x2 antenna array can significantly reduce the required number of measurement grids, compared with 8x2.

Proposal 2: beside the “Detailed parameters of 4x2 antenna array assumption for PC3 should be aligned in next meeting” [1], the simulation assumptions for 8x2 antenna array may also need to be discussed considering the above results that different assumptions can have very different required number of measurement grids, for the same 8x2 antenna array.

# References

- R4-2017597 WF on testability enhancements to reduce test time, vivo, Samsung, RAN4 #97-e

- 3GPP TR 38.810

- R4-1813581, Antenna and Beamforming Assumptions for Spherical Coverage Analyses, Keysight Technologies, RAN4#88bis