3GPP TSG-RAN WG4 Meeting #98-e	R4-2102485

Online, 25 Jan – 5 Feb, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7043 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for LTE FDD-TDD inter-band inter-frequency synchronous DAPS handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add test cases for LTE FDD-TDD inter-band inter-frequency synchronous DAPS handover |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test cases: 1. E-UTRAN FDD – TDD inter-band inter-F synchronous DAPS HO test cases 2. E-UTRAN TDD – FDD inter-band inter-F synchronous DAPS HO test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing test cases for FDD-TDD inter-band inter-frequency synchronous DAPS handover delay requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.1.x, A.5.1.x+1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR 36.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change>

### A.5.1.x	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover

#### A.5.1.x.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-TDD inter-band inter-frequency synchronous DAPS handover requirements specified in clause 5.7.2.2.

The test scenario comprises of one E-UTRA FDD cell and one E-UTRA TDD cell as given in tables A.5.1.x.1-1, A.5.1.x.1-2 and A.5.1.x.1-3. The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

At the start of time duration T1, the UE does not have any timing information of Cell 2. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

Starting T2, Cell 2 becomes detectable. During T2, the UE performs cell detection and measurements on Cell 2 and shall send event report to the network. After receiving the event report A3, the test system should send an RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing the RRC message implying DAPS handover command is sent to the UE. During T3, UE should be continuously scheduled on Cell 1 and shall perform random access to Cell 2. DL scheduling and UL feedback to Cell 1 should be avoided when UE is required to perform DL reception or UL transmission in Cell 2, except preamble transmission. After successful RACH procedure in Cell 2 is completed, the test system should send an RRC message implying Cell 1 release command.

The start of T4 is the instant when the last TTI containing the RRC message implying Cell 1 release command is sent to the UE. During T4, the UE shall accomplish the release actions within Dhandover2.

Starting T5, the UE shall stop to send CSI report to Cell 1.

Table A.5.1.x.1-1: General test parameters for E-UTRAN FDD-TDD inter-band inter-frequency synchronous DAPS handover test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Cell 1 PDSCH parameters |  |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| Cell 1 PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Cell 2 PDSCH parameters |  |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| Cell 2 PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbour cell |  | Cell 2 |  |
| Final conditions | Active cell |  | Cell 2 |  |
| Cell 1 E-UTRA RF channel number |  |  | 1 | One FDD carrier is used |
| Cell 2 E-UTRA RF channel number |  |  | 2 | One TDD carrier is used |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| A3-Offset |  | dB | -4 |  |
| Hysteresis |  | dB | 0 |  |
| Time to Trigger |  | ms | 0 |  |
| Filter coefficient |  |  | 0 |  |
| DRX |  |  | OFF |  |
| CP length |  |  | Normal |  |
| E-UTRA TDD Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. Applicable to cell 2. |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. Applicable to cell 2 |
| E-UTRA TDD PRACH configuration |  |  | 53 | As specified in table 5.7.1-3 in TS 36.211 |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts |
| Time offset between cells |  |  | 33µs | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | £5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | 16 |  |
| T5 |  | ms | 100 |  |

Table A.5.1.x.1-2: Cell specific test parameters for E-UTRAN FDD (Cell 1) in E-UTRAN FDD-TDD inter-band inter-frequency synchronous DAPS handover test case

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD | OP.1 FDD | OP.1 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA | dB | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | 4 |
| Note 2 | dBm/15 kHz | -98 |  |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | 4 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -94 | -94 |
| Propagation Condition | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameter themselves. |  |  |  |  |  |  |

Table A.5.1.x.1-3: Cell specific test parameters for E-UTRAN TDD (Cell 2) in E-UTRAN FDD-TDD inter-band inter-frequency synchronous DAPS handover test case

| Parameter | Unit | Cell 2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |  | T4 |  | T5 |  |
| E-UTRA RF Channel number |  | 2 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.2 TDD |  | OP.2 TDD |  | OP.2 TDD |  | OP.2 TDD |  | OP.1 TDD |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
|  | dB | -Infinity |  | 7 |  | 7 |  | 7 |  | 7 |
| Note 2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |
|  | dB | -Infinity |  | 7 |  | 7 |  | 7 |  | 7 |
| RSRP Note 3 | dBm/15 KHz | -Infinity |  | -91 |  | -91 |  | -91 |  | -91 |
| Propagation Condition | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameter themselves. |  |  |  |  |  |  |  |  |  |  |

#### A.5.1.x.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Dhandover1 = 65 ms from the start of T3 and interruption on Cell 1 during T3 shall not exceed Tinterrupt1 = 1ms.

The UE shall complete to release Cell 1 less than Dhandover2 = 16 ms from the start of T4 and interruption on Cell 2 during T4 shall not exceed Tinterrupt2 = 1ms.

The rate of correct DAPS handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover1 can be expressed as: TRRC_procedure + Tsearch + TIU + 20ms, and Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

TRRC_procedure = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tsearch = 0 ms in the test as it is assumed as known cell; Tsearch is defined in clause 5.7.2.1.1.

TIU = 30 ms in the test; TCHO_execution is defined in clause 5.7.2.1.1.

Tinterrupt1 = 1 ms in the test; Tinterrupt is defined in clause 5.7.2.1.2.

Tinterrupt2 = 1 ms in the test; Tinterrupt is defined in clause 5.7.2.1.2.

This gives a total of 65 ms for Dhandover1 and 16 ms for Dhandover2.

The UE shall not transmit CSI reports on Cell 1 after T5.

### A.5.1.x+1	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover

#### A.5.1.x+1.1	Test Purpose and Environment

This test is to verify the requirement for the TDD-FDD inter-band inter-frequency synchronous DAPS handover requirements specified in clause 5.7.2.3.

The test scenario comprises of one E-UTRA TDD cell and one E-UTRA FDD cell as given in tables A.5.1.x+1.1-1, A.5.1.x+1.1-2 and A.5.1.x+1.1-3. The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

At the start of time duration T1, the UE does not have any timing information of Cell 2. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

Starting T2, Cell 2 becomes detectable. During T2, the UE performs cell detection and measurements on Cell 2 and shall send event report to the network. After receiving the event report A3, the test system should send an RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing the RRC message implying DAPS handover command is sent to the UE. During T3, UE should be continuously scheduled on Cell 1 and should be able to perform random access to Cell 2. DL scheduling and UL feedback to Cell 1 should be avoided when UE is required to perform DL reception or UL transmission in Cell 2, except preamble transmission. After successful RACH procedure of Cell 2 is completed, the test system should send an RRC message implying Cell 1 release command.

The start of T3 is the instant when the last TTI containing the RRC message implying DAPS handover command is sent to the UE. During T3, UE should be continuously scheduled on Cell 1 and shall perform random access to Cell 2. DL scheduling and UL feedback to Cell 1 should be avoided when UE is required to perform DL reception or UL transmission in Cell 2, except preamble transmission. After successful RACH procedure in Cell 2 is completed, the test system should send an RRC message implying Cell 1 release command.

The start of T4 is the instant when the last TTI containing the RRC message implying Cell 1 release command is sent to the UE. During T4, the UE shall accomplish the release actions within Dhandover2.

Starting T5, the UE should stop to send CSI report to Cell 1.

Table A.5.1.x+1.1-1: General test parameters for E-UTRAN TDD-FDD inter-band inter-frequency synchronous DAPS handover test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Cell 1 PDSCH parameters |  |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| Cell 1 PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Cell 2 PDSCH parameters |  |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| Cell 2 PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbour cell |  | Cell 2 |  |
| Final conditions | Active cell |  | Cell 2 |  |
| Cell 1 E-UTRA RF channel number |  |  | 1 | One TDD carrier is used |
| Cell 2 E-UTRA RF channel number |  |  | 2 | One FDD carrier is used |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| A3-Offset |  | dB | -4 |  |
| Hysteresis |  | dB | 0 |  |
| Time to Trigger |  | ms | 0 |  |
| Filter coefficient |  |  | 0 |  |
| DRX |  |  | OFF |  |
| CP length |  |  | Normal |  |
| E-UTRA FDD PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in TS 36.211 |
| E-UTRA FDD Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts |
| Time offset between cells |  |  | 33µs | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | £5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | 16 |  |
| T5 |  | ms | 100 |  |

Table A.5.1.x+1.1-2: Cell specific test parameters for E-UTRAN TDD (Cell 1) in E-UTRAN TDD-FDD inter-band inter-frequency synchronous DAPS handover test case

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD | OP.1 TDD | OP.1 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | 4 |
| Note 2 | dBm/15 kHz | -98 |  |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | 4 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 |  |  |  |
| Propagation Condition | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameter themselves. |  |  |  |  |  |  |

Table A.5.1.x+1.1-3: Cell specific test parameters for E-UTRAN FDD (Cell 2) in E-UTRAN TDD-FDD inter-band inter-frequency synchronous DAPS handover test case

| Parameter | Unit | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel number |  | 2 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.2 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD | OP.1 FDD |
| PBCH_RA | dB | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dB | -Infinity | 7 | 7 | 7 | 7 |
| Note 2 | dBm/15 kHz | -98 |  |  |  |  |
|  | dB | -Infinity | 7 | 7 | 7 | 7 |
| RSRP Note 3 | dBm/15 KHz | -Infinity | -91 | -91 | -91 | -91 |
| Propagation Condition | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameter themselves. |  |  |  |  |  |  |

#### A.5.1.x+1.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Dhandover1 = 65 ms from the start of T3 and interruption on Cell 1 during T3 shall not exceed Tinterrupt1 = 1ms.

The UE shall complete to release Cell 1 less than Dhandover2 = 16 ms from the start of T4 and interruption on Cell 2 during T4 shall not exceed Tinterrupt2 = 1ms.

The rate of correct DAPS handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover1 can be expressed as: TRRC_procedure + Tsearch + TIU + 20ms, and Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

TRRC_procedure = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tsearch = 0 ms in the test as it is assumed as known cell; Tsearch is defined in clause 5.7.2.1.1.

TIU = 30 ms in the test; TCHO_execution is defined in clause 5.7.2.1.1.

Tinterrupt1 = 1 ms in the test; Tinterrupt is defined in clause 5.7.2.1.2.

Tinterrupt2 = 1 ms in the test; Tinterrupt is defined in clause 5.7.2.1.2.

This gives a total of 65 ms for Dhandover1 and 16 ms for Dhandover2.

The UE shall not transmit CSI reports on Cell 1 after T5.

# < End of change>