# 3GPP TSG RAN WG4 Meeting #98-e	R4-2102493

# Electronic Meeting, 25th January – 5th February, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR | draft | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.941: Relative calibration approach |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | OTA_BS_testing-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | An alternate calibration method can be defined that improves MU |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clause 8.7 to describe the relative calibration approach, including dedicated sub-clauses to describe the step-by-step procedure for Tx and Rx requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Further improvement of MU cannot be applied |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion paper in R4-2102492 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

< Unchanged Text Deleted >

< Beginning of Changes >

## 8.7	Relative calibration approach

Range path loss relative approach based on a reference receiver and has a simple foundation: same measurement equipment is used in both stages (calibration and measurement) while a reference receiver, with good absolute accuracy, is used during calibration stage. Following this approach:

- The final DUT measurement becomes relative, and thus the MU source for absolute accuracy of the measurement receiver / signal generator is cancelled out.

- The absolute accuracy of the reference receiver replaces the one for the network analyzer on Stage 1 – Calibration measurement.

- Mismatch sources are also minimized since there is no change in the measurement path between calibration and BS measurements.

A high-level description of both Tx and Rx path loss measurements is provided in the following sections.

This approach is applicable to all OTA measurement systems described in clause 7.

### 8.7.1 Relative calibration approach, Tx requirements

In case of transmitter measurements, the relative approach step-by-step procedure is as follows:

- Connect the reference receiver to the RF output port of the signal generator, as shown in figure 8.7.1-1, using a reference cable and a matching attenuator.

Figure 8.7.1-1: Absolute Power Calibration with Reference Receiver

- Set the frequency fn and the output power Pout of CW signal at the output D of the signal generator.

- Measure PRX'fn at the interface E which is the input of the reference receiver.

- Replace the reference receiver with the reference antenna at the interface E and connect the measurement receiver to the RF output port B of the Measurement Antenna as shown in figure 8.7.1-2. The phase center of the reference antenna A shall be aligned with the coordinate system. The interface A shall coincide with the origin of the DUT positioner coordinate system. Realized gain of the reference antenna GREFfn in the direction towards Measurement Antenna is known.

Figure 8.7.1-2: Tx range path loss Calibration with Reference Receiver

- Measure PRXfn at the interface C which is the input of the measurement receiver.

- Calculate path loss value PLEIRPfn between the interfaces A and D for EIRP measurement. Note that PLEIRPfn includes absolute level uncertainty of the measurement receiver which is cancelled out during transmitter measurement.
PLEIRPfn=10logPRX'fnGREFfnPRXfn [dB]

### 8.7.2 Relative calibration approach, Rx requirements

In case of receiver measurements, the relative approach step-by-step procedure is as follows:

- Connect the signal generator to the input B of Measurement Antenna and the input of the reference receiver to the RF output port D of the reference antenna, as shown in 3.2-1. The phase center of the reference antenna A shall be aligned with the coordinate system A. Realized gain of the reference antenna GREFfn in the direction towards Measurement Antenna is known.

Figure 3.2-1: Rx range path loss Calibration with Reference Receiver

- Set the frequency fn and the output power Pout of CW signal at the output C of the signal generator.

- Measure PRXfn at the interface D which is the input of the reference receiver.

- Calculate path loss value PLEISfn between the interfaces A and C for EIS measurement. Note that PLEISfn includes absolute level uncertainty of the signal generator which will be cancelled out during EIS measurement.
PLEISfn=10logPoutfnGREFfnPRXfn [dB]

< End of Changes >