3GPP TSG-RAN WG4 Meeting #98-e 	R4-2102528

# Electronic Meeting, January 25  February 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CCA model in NR-U test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | No CCA model in the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a CCA model for NR-U test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No CCA model in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: A.3.20, A.3.20.1, A.3.20.2, A.3.20.2.1, A.3.20.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## A.3.20	CCA model

### A.3.20.1	Introduction

The CCA model is used in some RRM test cases with at least one cell on a carrier frequency with CCA. The intention with the CCA model is to emulate in the test equipment the behaviour of a gNB or UE which performs channel measurement to check that the channel is clear prior to performing one or more downlink transmissions.

### A.3.20.2	CCA model for operation on a carrier frequency with CCA in FR1

#### A.3.20.2.1	DL CCA

Prior to each discovery burst transmission window configured within time interval Ti (i≥1) during a test, the test equipment shall determine whether to transmit based on probability PCCA_DL,i specified in the corresponding test case. The set of possible values for PCCA_DL,i is the range [0, 1].

If the test equipment determines a successful CCA attempt (i.e., the corresponding signals have to be transmitted), then the candidate SSB location within the discovery burst transmission window is randomly selected from the set of possible candidate SSB locations. The SSB shift within the discovery burst transmission window is not necessary to model for non-congested channels.

One probability value (per transmitter) applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_DL,i per time interval Ti where i≥1 and the multiple time intervals (when i>1) do not overlap.

Probability PCCA_DL, i is defined on case-by-case basis for different test cases. For the same test casespecified for both semi-static channel occupancy and dynamic channel occupancy:

- probability PCCA_DL, i is higher for the semi-static channel occupancy compared to that for dynamic channel occupancy, when Es/Iot<[-6] dB.

- probability PCCA_DL, i can be the same for semi-static channel occupancy and dynamic channel occupancy, when Es/Iot≥[-6] dB.

Prior to each discovery burst transmission window within a time interval i of the test:

- Generate a uniform random variable p from the range [0, 1].

- If the SSB shift is modelled, determine the candidate SSB location x to be used for the transmission by randomly selecting a value from the set XSSB of possible candidate SSB locations. Otherwise, x is the first candidate SSB location index.

- Transmit the discovery burst based on p and x: if p<PCCA_DL,i, the discovery burst is transmitted at candidate SSB location x; otherwise the discovery burst is muted.

The above steps are repeated for each discovery burst transmission window in each time interval i of the test.

In many cases, the requirement under a test depends on the number of configured signal occasions which are not available during the test, so the test equipment shall track how many such signal occasions are not transmitted during the test period.

#### A.3.20.2.2	UL CCA

For UL CCA, the modelling approach is based on a probability PCCA_UL,i of successful access during the corresponding time Ti of the time interval i.

Prior to each UL transmission burst within a time interval i of the test:

- Generate a uniform random variable p from the range [0, 1].

- If p<PCCA_UL,i, then the energy generated by the test system in the corresponding portion of UL slot is equal to or below the energy detection threshold [TBD]; otherwise the energy generated by the test system in the portion of UL slot is above the energy detection threshold [TBD].