3GPP TSG-RAN4 Meeting #98-e	R4-2102570

# Online, , 25th Jan 2020 – 5th	Feb 2021

Source: 	Huawei

Title: 	TP to TR 38.808: BS aspects

Agenda Item:	12.2.2

Document for:	Approval

# Introduction

This contribution provides TP to TR 38.808 on selected BS aspects for 52.6 – 71 GHz range, including BS architecture and BS classes and additional examples of BS antenna array.

# Discussion

During previous meeting, TP to TR 38.808 on BS RF for NR beyond 52.6 GHz was approved in [2] with example of the 16x8 BS antenna array for 70GHz, as extracted below:

Table 4.2.5.1-1: Example antenna arrays Carrier frequency; 70.0GHz. Array size; 16x8. System Gain & Coverage Analysis. Total Conducted Power; dBm; 28.0. Tx Arrayed Antenna Gain; dBi; 24.8. EIRP per polarization; dBm; 52.8. Number of antenna polarizations; 2. Number of streams / polarizations; 1. Transmitter Array Information (per polarization). # elements in X; 16. # elements in Y; 8. Number of antenna elements; 128. Element spacing; lambda; 0.5. Array size, X dimension; mm; 34.3. Array size, Y dimension; mm; 17.1. Element directivity; dB; 6.0. Array factor; dB; 21.0. Antenna directive gain; dB; 27.0. Antenna coupling efficiency; %; 60 %. Antenna coupling efficiency; dB; -2.2. Tx Array Antenna Gain; dB; 24.8. Transmitter Power Amplifier Info (per polarization). PA OP1dB; dBm; 16. Backoff, i.e. modulated signal PAPR; dB; 9.0.

In this contribution we provide further examples of the BS antenna arrays for 52,6 – 71GHz NR operation.

Furthermore, during previous meeting, TP to TR 38.808 on BS architecture and BS classes was provided in [3]. An updated version of that text proposal is also provided below.

# Conclusions

The attached TP to TR 38.808 is proposed for approval.

# 4	References

[1]		TS 38.808, v1.0.0 		Study on supporting NR from 52.6 GHz to 71 GHz

[2]		R4-2016995				TP to TR 38.808: BS RF for NR beyond 52.6 GHz, RAN4#97e

[3]		R4-2015947				TP to TR 38.808: BS architecture and BS classes for 52-71 GHz range, RAN4#97e

# 5			TP to TR 38.808: Study on supporting NR from 52.6 GHz to 71 GHz

Based on the discussion initiated in section 2, below we provide a combined TP indicating BS-related topic areas.

------------------------------ Modified section ------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies"

[2]	3GPP TR 38.807: "Study on requirements for NR beyond 52.6 GHz".

[3]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[4]	ETSI EN 302 567 v2.1.20: "Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU".

[5]	R1-2007549 "Further discussion on B52 numerology" FUTUREWEI.

[6]	R1-2007558 "Discussion on physical layer impacts for NR beyond 52.6 GHz" Lenovo, Motorola Mobility.

[7]	R1-2007604 "PHY design in 52.6-71 GHz using NR waveform" Huawei, HiSilicon.

[8]	R1-2007642 "Physical layer design for NR 52.6-71GHz" Beijing Xiaomi Software Tech.

[9]	R1-2007652 "Discussion on required changes to NR using existing DL/UL NR waveform" vivo.

[10]	R1-2007785 "Consideration on required changes to NR using existing NR waveform" Fujitsu.

[11]	R1-2007790 "Consideration on supporting above 52.6GHz in NR" InterDigital, Inc.

[12]	R1-2007847 "System Analysis of NR opration in 52.6 to 71 GHz" CATT.

[13]	R1-2007883 "Required changes to NR using existing DL/UL NR waveform" TCL Communication Ltd.

[14]	R1-2007926 "Required changes to NR using existing DL/UL NR waveform" Nokia, Nokia Shanghai Bell.

[15]	R1-2007929 "On phase noise compensation for NR from 52.6GHz to 71GHz" Mitsubishi Electric RCE.

[16]	R1-2009379 "Discussion on Required Changes to NR in 52.6 – 71 GHz" Intel Corporation.

[17]	R1-2007965 "On the required changes to NR for above 52.6GHz" ZTE, Sanechips.

[18]	R1-2007982 "On NR operations in 52.6 to 71 GHz" Ericsson.

[19]	R1-2009653 "Consideration on required physical layer changes to support NR above 52.6 GH"	LG Electronics.

[20]	R1-2008076 "Discussion on required changes to NR using existing DL/UL NR waveform in 52.6GHz ~ 71GHz" CMCC.

[21]	R1-2008082 "Study on the numerology to support 52.6 GHz to 71GHz" NEC.

[22]	R1-2008872 "Design aspects for extending NR to up to 71 GHz" Samsung.

[23]	R1-2008250 "Discusson on required changes to NR using DL/UL NR waveform" OPPO.

[24]	R1-2008353 "Considerations on required changes to NR from 52.6 GHz to 71 GHz" Sony.

[25]	R1-2008457 "A Discussion on Physical Layer Design for NR above 52.6GHz" Apple.

[26]	R1-2008493 "Discussions on required changes on supporting NR from 52.6GHz to 71 GHz" CAICT.

[27]	R1-2008501 "On required changes to NR using existing DL/UL NR waveform for operation in 60GHz band" MediaTek Inc.

[28]	R1-2008516 "On NR operation between 52.6 GHz and 71 GHz" Convida Wireless.

[29]	R1-2009062 "Evaluation Methodology and Required Changes on NR from 52.6 to 71 GHz" NTT DOCOMO, INC.

[30]	R1-2008615 "NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz" Qualcomm Incorporated.

[31]	R1-2008726 "Discussion on physical layer aspects for NR beyond 52.6GHz" WILUS Inc.

[32]	R1-2008769 "Waveform considerations for NR above 52.6 GHz" Charter Communications.

[33]	R1-2007550 "On channel access modes in 60GHz" FUTUREWEI.

[34]	R1-2007559 "Discussion on channel access for NR beyond 52.6 GHz" Lenovo, Motorola Mobility.

[35]	R1-2008976 "Channel access mechanism for 60 GHz unlicensed operation" Huawei, HiSilicon.

[36]	R1-2007643 "Channel access mechanism for NR on 52.6-71 GHz" Beijing Xiaomi Software Tech.

[37]	R1-2007653 "Discussion on channel access mechanism" vivo.

[38]	R1-2007791 "On Channel access mechanisms" InterDigital, Inc.

[39]	R1-2007848 "Channel Access Mechanism in support of NR operation in 52.6 to 71 GHz" CATT.

[40]	R1-2007884 "Channel access mechanism" TCL Communication Ltd.

[41]	R1-2007918 "Channel access mechanisms for NR from 52.6-71GHz" AT&T.

[42]	R1-2009312 "Design of NR channel access mechanisms for 60 GHz unlicensed band" Nokia, Nokia Shanghai Bell.

[43]	R1-2009380 "Channel Access Procedure for NR in 52.6 - 71 GHz" Intel Corporation.

[44]	R1-2007966 "On the channel access mechanism for above 52.6GHz" ZTE, Sanechips.

[45]	R1-2007983 "Channel Access Mechanism" Ericsson.

[46]	R1-2008046 "Considerations on channel access mechanism to support NR above 52.6 GHz" LG Electronics.

[47]	R1-2008091 "Discussion on channel access mechanism for above 52.6GHz" Spreadtrum Communications.

[48]	R1-2008157 "Channel access mechanism for 60 GHz unlicensed spectrum" Samsung.

[49]	R1-2008251 "Discussion on channel access" OPPO.

[50]	R1-2008354 "Channel access mechanism for 60 GHz unlicensed spectrum" Sony.

[51]	R1-2008458 "Views on Channel Access Mechanisms for Unlicensed Access above 52.6 GHz" Apple.

[52]	R1-2008494 "Discussions on channel access mechanism on supporting NR from 52.6GHz to 71 GHz" CAICT.

[53]	R1-2008517 "On Channel Access Mechanism and Interference Handling for Supporting NR from 52.6 GHz to 71 GHz" Convida Wireless.

[54]	R1-2008548 "Channel Access Mechanism for NR in 60 GHz unlicensed spectrum" NTT DOCOMO, INC.

[55]	R1-2008563 "Discussion on channel access mechanism" ITRI.

[56]	R1-2009362 "Channel access mechanism for NR in 52p6 to 71GHz band" Qualcomm Incorporated.

[57]	R1-2008717 "Discussion on channel access mechanism for 52.6 to 71GHz unlicensed ban"	Potevio

[58]	R1-2008770 "Further aspects of channel access mechanisms" Charter Communications.

[59]	R1-2007560 "Additional evaluations for NR beyond 52.6GHz" Lenovo, Motorola Mobility.

[60]	R1-2007654 "Evaluation on different numerologies for NR using existing DL/UL NR waveform" vivo.

[61]	R1-2007792 "Evaluation results for above 52.6 GHz" InterDigital, Inc.

[62]	R1-2007928 "Simulation Results for NR from 52.6 GHz to 71 GHz" Nokia, Nokia Shanghai Bell.

[63]	R1-2007943 "Considerations on performance evaluation for NR in 52.6-71GHz" Intel Corporation.

[64]	R1-2009450 "Simulation results for NR above 52.6GHz" ZTE, Sanechips.

[65]	R1-2007984 "Evaluation results for NR in 52.6 - 71 GHz" Ericsson.

[66]	R1-2008047 "Considerations on phase noise compensation to support NR above 52.6 GHz" LG Electronics.

[67]	R1-2008873 "Evaluation results for extending NR to up to 71 GHz" Samsung.

[68]	R1-2009615 "Discussion on other aspects" OPPO.

[69]	R1-2008459 "Evaluation results for Physical Layer Design for NR above 52.6GHz" Apple.

[70]	R1-2008549 "Potential Enhancements for NR on 52.6 to 71 GHz" NTT DOCOMO, INC.

[71]	R1-2009157 "Performance evaluations for NR above 52.6 GHz" Charter Communications.

[72]	R1-2009610 "Link level and System level evaluation for NR system operating in 52.6GHz to 71GHz" Huawei, HiSilicon.

[73]	3GPP TR 38.803: "Study on new radio access technology; Radio Frequency (RF) and co-existence aspects".

[74]	Hua Wang, Fei Wang, Sensen Li, Tzu-Yuan Huang, Amr S. Ahmed, Naga Sasikanth Mannem, Jeongseok Lee, Edgar Garay, David Munzer, Christopher Snyder, Sanghoon Lee, Huy Thong Nguyen, and Michael Edward Duffy Smith, "Power Amplifiers Performance Survey 2000-Present," [Online]. Available: https://gems.ece.gatech.edu/PA_survey.html

[75]	ETSI TR 101 854: "Fixed Radio Systems; Point-to-point equipment; Derivation of receiver interference parameters useful for planning fixed service point-to-point systems operating different equipment classes and/or capacities"

[76]	3GPP TR 38.817-02: "General aspects for Base Station (BS) Radio Frequency (RF) for NR"

[77]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[78]		3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

------------------------------ Next modified section ------------------------------

### 4.2.5	BS aspects

#### 4.2.5.1	BS antenna arrays

More detailed antenna array modelling may be further considered in work item if needed.

Antenna array configurations were evaluated with respect to target output power. It is assumed that each antenna array element has its own power stage and antenna elements have 60% aperture coupling efficiency. The EIRP can be calculated as shown in Table 4.2.4.2-3. While the example in table 4.2.5.1-1 is provided for 16x8 element array, the same principle can be used to evaluate the array sizes required to reach 40 dBm and 60 dBm EIRP, which we set for targets for system evaluations according to Annex A.2 The result is that reaching 40 dBm EIRP requires an 4x8 element array whereas 60 dBm EIRP requires 16x16 element array.

It should be noted that the estimates are very conservative, given that the power amplifier study [74] as captured in clause 4.2.4 outlines the saturated power for CMOS technology between 22 and 23 dBm for 52.6 – 71 GHz frequency range. Therefore, plenty of design margin for possible front-end losses have been included.

Table 4.2.5.1-1: Example antenna arrays

|  |  | Antenna array examples |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency | GHz | 70.0 | 70 | 70 | 70 |
| Array size |  | 16x8 | 16x16 | 16x32 | 32x32 |
| System Gain & Coverage Analysis |  |  |  |  |  |
| Total Conducted Power | dBm | 28.0 | 31 | 34 | 37 |
| Tx Arrayed Antenna Gain | dBi | 24.8 | 27,8 | 30,8 | 33,8 |
| EIRP per polarization | dBm | 52.8 | 58,8 | 64,8 | 70,8 |
| Number of antenna polarizations |  | 2 | 2 | 2 | 2 |
| Number of streams / polarizations |  | 1 | 1 | 1 | 1 |
| Transmitter Array Information (per polarization) |  |  |  |  |  |
| # elements in X |  | 16 | 16 | 16 | 32 |
| # elements in Y |  | 8 | 16 | 32 | 32 |
| Number of antenna elements |  | 128 | 256 | 512 | 1024 |
| Element spacing | lambda | 0.5 | 0,5 | 0,5 | 0,5 |
| Array size, X dimension | mm | 34.3 | 34,3 | 34,3 | 68,5 |
| Array size, Y dimension | mm | 17.1 | 34,3 | 68,5 | 68,5 |
| Element directivity | dB | 6.0 | 6 | 6 | 6 |
| Array factor | dB | 21.0 | 24 | 27 | 30 |
| Antenna directive gain | dB | 27.0 | 30 | 33 | 36 |
| Antenna coupling efficiency | % | 60 % | 60% | 60% | 60% |
| Antenna coupling efficiency | dB | -2.2 | -2.2 | -2.2 | -2.2 |
| Tx Array Antenna Gain | dB | 24.8 | 27.8 | 30.8 | 33.8 |
| Transmitter Power Amplifier Info (per polarization) |  |  |  |  |  |
| PA OP1dB | dBm | 16 | 16 | 16 | 16 |
| Backoff, i.e. modulated signal PAPR | dB | 9.0 | 9 | 9 | 9 |
| Average power output per PA | dBm | 7 | 7 | 7 | 7 |
| Power into each sub-array or antenna element | dBm | 7 | 7 | 7 | 7 |
| Number of Tx sub-array PAs or antenna elements |  | 128 | 256 | 512 | 1024 |
| Multi element or sub-array power gain | dB | 21 | 24 | 27 | 30 |
| Total Conducted Power | dBm | 28.0 | 31 | 34 | 37 |

NOTE:	The above examples of the BS antenna arrays are not limited by the BS antenna array configurations and antenna array patterns in annex A.2 (i.e. System level simulations assumptions), which were defined for the purpose of performance evaluations in RAN1.

The array sizes are similar to FR2 array in 24.25 – 52.6 GHz frequency range, and therefore also the beamwidths are similar.

#### 4.2.5.2	Noise figure

Referring to ETSI technical report TR 101 854 [75] on fixed radio point-to-point systems, one can find information on typical NF and associated industrial margins (IMF) values across wide range of frequencies. The IMF considers various performance variations of system elements over e.g. temperature extremes, voltage variations, or aging by capturing production spread of RF circuits.

For the 52.6 – 71 GHz range, the above NF values are in range of 10 – 13 dB.

Table 4.2.5.2-1: Typical Noise Figures (NF) and associated Industrial Margins (IMF)

| Frequency band (GHz) | Typical Noise Figure (NF) (dB) | Industrial margin (IMF) (dB) |
| --- | --- | --- |
| 48 - 50 | ~9 | +3 |
| 52 - 55 | ~10 | +3 |
| 71 - 76 | ~13 | +4 |
| 81 - 86 | ~13 | +4 |

NOTE:	The above ETSI originated Industrial Margins (IMF) shall not be confused with the Implementation Margins (IM) used in process of RAN4 conformance requirements derivation.

Referring to the system level evaluation assumptions in annex A, the BS NF was assumed as 7 dB, while UE NF was assumed as 10 dB (13 dB optionally).

Link level and system level simulation assumptions in annex A are assuming UE mobility (i.e. 3km/h), while the above referred ETRI TR treats about the fixed point-to-point applications. Still one can observe good alignment of NF values among the ETSI TR and those in this 52.6-71 GHz performance evaluation. This can be interpreted that the RF technology to be used for both static and mobile applications in 52.6-71 GHz frequency range will have similar and comparable capabilities, making the ETSI TR 101 854 [75] a valid reference for this technical report.

#### 4.2.5.3	BS architecture

Referring to the NR BS operating in FR2 (i.e. 24250 – 52600 MHz), the BS type 2-O was defined based on the AAS architecture, only. The non-AAS architecture was out-ruled for FR2 due to practical reasons of its implementation at high frequencies, related complexity of connectorized antenna arrays, etc. For more background information on the BS architecture for the NR operation in FR2, refer to TR 38.817-02 [76].

Considering NR operation in 52.6 – 71 GHz range, it can be assumed that the BS type 2-O architecture (i.e. the AAS BS architecture) will be considered as the baseline, at least for LA BS, MR BS and WA BS classes (see clause 4.2.5.4).

Referring to BS classes in clause 4.2.5.4, one can notice that there may be need to further extend the range of BS classes for NR operation in 52.6 – 71 GHz range with products specifically targeting indoor scenarios (e.g. for potential BS products optimized for low cost indoor deployments). For such scenarios consideration of non-AAS BS architecture may be beneficial and shall not be precluded. Such decision is beyond the scope of this SI and shall be further studied during the follow-up WI, considering inputs from interested operators.

Therefore the conclusion for the NR BS architecture for 52.6 – 71 GHz range operation shall assume the AAS BS architecture to be the baseline for the already defined NR BS classes, while not restricting non-AAS architecture pending future studies during WI.

#### 4.2.5.4	BS classes

Referring to the NR BS specification for FR2, the following BS classes were defined in TS 38.104 [77]:

-	Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios with a BS to UE minimum distance along the ground equal to 35 m.

-	Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios with a BS to UE minimum distance along the ground equal to 5 m.

-	Local Area Base Stations are characterised by requirements derived from Pico Cell scenarios with a BS to UE minimum distance along the ground equal to 2 m.

Referring to the deployment scenarios considered in annex A, both indoor and outdoor scenarios were considered. For the outdoor scenarios (i.e. single and dual layer Dense Urban scenarios) the above listed BS classes defined for NR operation in FR2 are considered to be sufficiently flexible to accommodate any of the considered outdoor scenarios. Therefore for outdoor scenarios, BS classes defined for BS type 2-O are considered as the baseline for NR operation in 52.6 – 71 GHz range.

For indoor scenarios considered in this SI (i.e. open office with ISD of 20m, indoor factory hall with ISD of 50m) there may be need to further study whether additional BS class would be needed to address potential BS products intended for indoor only use, e.g. NR products operating in 52.6 – 71 GHz range, defined similar to the Home Base Station for E-UTRA (characterised by requirements derived from Femto Cell scenarios), as defined in TS 36.104 [78]. Such extension of the NR BS classes is considered as FFS for the followup WI.

It shall be clarified, that deployment scenarios considered in annex A for the performance evaluation shall not be considered as a limiting factor for future RAN4 discussion on BS classes for 52.5 – 71 GHz range. It shall be emphasized that the follow-up WI may revisit the range of considered deployment scenarios depending on the performance evaluation outcomes and operator’s interest, which may require RAN4 to revisit the above observations on BS classes.

----------------------------- End of modified section ------------------------------