3GPP TSG-RAN4 Meeting #98-e	R4-2102615

# Online, , 25th Jan 2021 - 5th Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.827 | CR | 0010 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Channel Model Topics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_MIMO_OTA_test |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Channel model rotations, especially relative orientations between UE and BS and effect on DUT orientations, have not been clarified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications on the channel model rotations are provided |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Channel models could be implemented differently and thus lead to different results |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Some changes of R4-2016211 from last meeting were not implemented properly. Discussion Paper in R4-2102614 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

<<< START OF CHANGE >>>

### 6.2.3	3D Multi-Probe Anechoic Chamber (MPAC) for FR2

The 3D MPAC test method is the reference methodology for FR2 NR MIMO OTA testing. By arranging an array of antennas around the Equipment Under Test (EUT), a spatial distribution of angles of arrival in the 3D MPAC system may be simulated to expose the EUT to a near field environment that appears to have originated from a complex multipath far field environment.

As illustrated schematically in Figure 6.2.3-1, signals propagate from the base station/communication tester to the EUT through a simulated multipath environment known as a spatial channel model, where appropriate channel impairments such as Doppler and fading are applied to each path prior to injecting all of the directional signals into the chamber simultaneously through the probe array. The resulting field distribution in the test zone is then integrated by the EUT antenna(s) and processed by the receiver(s) just as it would do so in any non-simulated multipath environment. The 3D MPAC system with 6 dual-polarized probes (illustrated with black dots in Figure 6.2.3-1) placed on a sector with minimum radius of 0.75m from the centre of the test zone is permitted for NR FR2 MIMO OTA testing.

Figure 6.2.3-1: 3D MPAC system layout for NR FR2 MIMO OTA testing

The exact probe locations with respect to the OTA test system coordinate system are tabulated in Table 6.2.3-1.

Table 6.2.3-1. FR2 3D MPAC Probe Locations in OTA test system coordinate system

| Probe Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 0.0 | 0.0 |
| 2 | 11.2 | 116.7 |
| 3 | 20.6 | -104.3 |
| 4 | 20.6 | 104.3 |
| 5 | 20.6 | 75.7 |
| 6 | 30.0 | 90.0 |

The 3D MPAC probes in Table 6.2.3-1 can be implemented using conventional millimetre-wave probes as well as IFF-based probes as long as the same probe configuration and same number of probes is used.

The channel model parameters and probe locations for channel model implementation are defined in a channel model coordinate system, which is illustrated in figure 6.2.3-2. The channel model coordinate axes xCM, yCM, and zCM correspond to the OTA test system coordinate axes z, y, and -x, respectively.

Figure 6.2.3-2: Channel Model Coordinate Axes

The probe locations with respect to channel model coordinate axes are tabulated in table 6.2.3-2.

Table 6.2.3-2. FR2 3D MPAC Probe Locations in Channel Model Coordinate System

| Probe Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 90 | 0 |
| 2 | 85 | 10 |
| 3 | 85 | -20 |
| 4 | 85 | 20 |
| 5 | 95 | 20 |
| 6 | 90 | 30 |

The channel model rotations assumed for this probe configuration are tabulated in Table 6.2.3-3.

Table 6.2.3-3. Channel Model Rotations

| InO CDL-A |  | UMi CDL-C |  |
| --- | --- | --- | --- |
| Phi [deg] | Theta [deg] | Phi [deg] | Theta [deg] |
| -5 | -2.0 | 32 | 15.0 |

These channel model rotations assume the relative orientations of BS and UE antennas displayed in Figure 6.2.3-3, i.e., the DUT antenna is pointed towards the BS in channel model coordinate system.

Figure 6.2.3-3: Relative orientations of BS and UE antennas.

Since the test points are uniformly spaced in 3D already, Table 6.2.3.2-1, there is no need to adjust/rotate the DUT rotations by the channel model rotations.

<<< Skip unchanged sections >>>

<<< END OF CHANGES >>>