3GPP TSG RAN WG4 Meeting #98-e	R4-2102638

Electronic Meeting, Jan 25-Feb 05, 2021

Agenda Item:	7.4.5.2

Source: 	Ericsson

Title:	Analysis of RRC re-establishment tests for LA IAB-MT

Document for:	Discussion

- Introduction

In the last RAN4 meeting the following was agreed and captured in the approved WF regarding RRM tests for IAB-MT in general and for RRC re-establishment test cases in particular [1]:

- Take UE test cases in TS 38.133 Annex as baseline when defining test cases for IAB-MTs.

- For RRC Re-establishment, only unknown target cell should be considered in the testing and only local-area IAB-MT to be tested.

- RRM tests are defined in FR1 for IAB type 1-H and FR2 for IAB type 2-O to verify all IAB-MT requirements defined in TS 38.174 that are also feasible to test.

In this paper we analyze the test cases to verify requirements on RRC re-establishment to target cell.

# Test setup and scenarios for RRC re-establishment tests

The IAB-MT core requirements for RRC re-establishment are specified for both intra-frequency and inter-frequency target cells in clause 12.2.1.1, TS 38.174. Furthermore, requirements are also defined for the case when the IAB-MT does not have serving cell timing e.g. when serving cell SCH Es/Iot < -8 dB.

In the annex of TS 38.133 the following RRC re-establishment test cases are defined for the UE in standalone scenario in FR1 and FR2:

- A.6.3.2.1.2	Intra-frequency RRC Re-establishment in FR1

- A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1

- A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing

- A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2

- A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2

- A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing

The UE tests on intra-frequency RRC re-establishment (tests in clauses A.6.3.2.1.2 and A.7.3.2.1.1) when serving cell timing is known are done when the target cell is known.

For IAB-MT as per agreement the target cell should be unknown. Therefore, all the UE tests (except tests in clauses A.6.3.2.1.2 and A.7.3.2.1.1) can be reused to develeop corresponding IAB-MT tests to verify different types of RRC re-establishment requirements for IAB-MT. The tests are limited to local area (LA) IAB-MT as also agreed in the WF [1]. Furthermore, FR1 and FR2 tests will be applicable to IAB type 1-H and IAB type 2-O respectively. In summary following 4 tests can be defined covering both intra-frequency and inter-frequency scenarios:

- TC1: Inter-frequency RRC Re-establishment in FR1 for LA IAB-MT and IAB type 1-H

- TC2: Intra-frequency RRC Re-establishment in FR1 without serving cell timing for LA IAB-MT and IAB type 1-H

- TC3: Inter-frequency RRC Re-establishment in FR2 for LA IAB-MT and IAB type 1-O

- TC4: Intra-frequency RRC Re-establishment in FR2 without serving cell timing for LA IAB-MT and IAB type 1-O

The parameters used in the UE tests can be reused in the IAB-MT tests. However, as discussed in another contribution, the test can be done by any set of parameters [2]. The parameters and corresponding test requirements in these tests are considered only one example.

- Summary

In this paper we have analyzed the scenarios and tests for verifying RRC re-establishment requirements. Based on the analysis following is the main proposal:

- Proposal 1: Following test cases are defined to verify RRC re-establishment requirements in clause 12.1.1.1, TS 38.174:

  - TC1: Inter-frequency RRC Re-establishment in FR1 for LA IAB-MT and IAB type 1-H

  - TC2: Intra-frequency RRC Re-establishment in FR1 without serving cell timing for LA IAB-MT and IAB type 1-H

  - TC3: Inter-frequency RRC Re-establishment in FR2 for LA IAB-MT and IAB type 1-O

  - TC4: Intra-frequency RRC Re-establishment in FR2 without serving cell timing for LA IAB-MT and IAB type 1-O

The corresponding draft CR to TS 38.174 on RRC re-establishment test cases for IAB-MT is provided in [3].

- References

- R4-2017383, WF on test cases for IAB-MTs, ZTE

- R4-2102640, “General principles for IAB RRM test cases, Ericsson

- R4-2102639, “RRC re-establishment tests for LA IAB-MT, Ericsson