3GPP TSG-RAN WG4 Meeting #98-e	R4-2102774

Electronic Meeting, 25 January – 5 February, 2021

Title: 	Further simulation results for PRS-RSRP

Source: 	Huawei, HiSilicon

Agenda item:	7.7.2.2.1.2

Document for:	Discussion

Introduction

In RAN4#97, the PRS-RSRP measurement accuracy requirements are discussed, and the outcomes are captured in [1]. One agreement is that

- Define both absolute and relative accuracy requirements for PRS-RSRP

In our paper [2] we have provided simulation results for the relative accuracy, which is re-produced in section 2.2 for completeness. In this paper we will provide further simulation results for absolute PRS-RSRP measurement accuracy.

Discussion

Simulation assumptions

A subset of cases have been simulated from the simulation assumptions in [3].

- Only Synchronous case with time shifts <0, 0, 3 us> is simulated

- PRS periodicity is 160ms

- 100% RE utilization in non-PRS symbols

- All listed SCS and BW are simulated in all listed channels

- All comb sizes are simulated

- Repetition factor 1,2,4 are simulated with PRS-ResourceTimeGap = 1

Simulation results for relative PRS-RSRP accuracy [2]

Table 1-3 show the simulation results for relative PRS-RSRP accuracy. The relative PRS-RSRP accuracy is calculated as the gap between 95%-tile and 5%-tile values in the CDF for each cell.

Table 1: Relative PRS-RSRP error for 15kHz

| RB num | Comb | Repetition | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  | TDL-C |  |  |
|  |  |  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |
| 52 | 2 | 1 | 1.8 | 6.9 | 5.9 | 2.3 | 9.4 | 9.1 | 2.1 | 7.6 | 7.6 | 2.0 | 7.0 | 7.7 |
|  |  | 2 | 1.1 | 3.4 | 3.5 | 1.5 | 4.9 | 4.6 | 2.4 | 5.1 | 5.0 | 1.3 | 4.4 | 4.3 |
|  |  | 4 | 0.7 | 2.0 | 2.1 | 1.0 | 2.8 | 2.8 | 4.8 | 6.1 | 6.9 | 2.0 | 3.3 | 3.4 |
|  | 4 | 1 | 1.7 | 6.0 | 6.1 | 2.4 | 8.7 | 9.9 | 2.2 | 8.1 | 7.3 | 2.1 | 8.7 | 7.4 |
|  |  | 2 | 1.1 | 3.4 | 3.5 | 1.4 | 4.9 | 5.1 | 2.3 | 5.0 | 5.0 | 1.3 | 4.3 | 4.3 |
|  |  | 4 | 0.8 | 2.1 | 2.1 | 0.9 | 2.9 | 2.7 | 5.3 | 6.4 | 6.2 | 2.2 | 3.3 | 3.4 |
|  | 6 | 1 | 1.8 | 6.7 | 5.9 | 2.4 | 9.2 | 8.3 | 2.1 | 7.7 | 8.6 | 2.0 | 8.4 | 9.2 |
|  |  | 2 | 1.2 | 3.5 | 3.5 | 1.5 | 5.2 | 4.7 | 2.3 | 5.0 | 5.4 | 1.4 | 4.4 | 4.4 |
|  |  | 4 | 0.7 | 2.1 | 2.1 | 0.9 | 2.9 | 3.2 | 4.7 | 6.3 | 6.4 | 2.1 | 3.3 | 3.4 |
|  | 12 | 1 | 1.7 | 6.3 | 6.1 | 2.3 | 8.7 | 12.0 | 2.3 | 8.5 | 8.4 | 2.2 | 9.9 | 8.5 |
|  |  | 2 | 1.1 | 3.4 | 3.5 | 1.4 | 5.1 | 4.6 | 2.2 | 4.9 | 5.3 | 1.5 | 4.4 | 4.9 |
|  |  | 4 | 0.8 | 2.1 | 2.1 | 0.9 | 2.9 | 2.9 | 4.7 | 6.2 | 6.2 | 2.3 | 3.4 | 3.5 |
| 104 | 2 | 1 | 1.3 | 4.1 | 4.4 | 1.5 | 5.7 | 5.5 | 1.4 | 4.9 | 5.3 | 1.5 | 4.9 | 5.3 |
|  |  | 2 | 0.8 | 2.5 | 2.4 | 0.9 | 3.2 | 2.9 | 1.8 | 3.5 | 3.8 | 1.0 | 3.0 | 3.0 |
|  |  | 4 | 0.5 | 1.5 | 1.5 | 0.6 | 2.0 | 1.8 | 4.5 | 4.9 | 4.8 | 1.8 | 2.6 | 2.9 |
|  | 4 | 1 | 1.3 | 4.8 | 4.4 | 1.6 | 5.8 | 6.6 | 1.5 | 5.2 | 5.5 | 1.5 | 5.0 | 5.2 |
|  |  | 2 | 0.8 | 2.6 | 2.5 | 1.0 | 3.1 | 3.4 | 1.8 | 3.8 | 3.5 | 1.1 | 3.1 | 3.1 |
|  |  | 4 | 0.5 | 1.4 | 1.5 | 0.6 | 1.8 | 1.8 | 4.6 | 5.1 | 5.4 | 1.7 | 2.6 | 2.6 |
|  | 6 | 1 | 1.3 | 4.6 | 4.7 | 1.6 | 5.4 | 5.8 | 1.4 | 5.3 | 5.4 | 1.5 | 5.7 | 4.8 |
|  |  | 2 | 0.8 | 2.5 | 2.5 | 1.0 | 3.0 | 3.1 | 1.8 | 3.6 | 3.6 | 1.1 | 3.0 | 3.1 |
|  |  | 4 | 0.5 | 1.5 | 1.5 | 0.6 | 1.8 | 1.9 | 4.4 | 5.0 | 5.0 | 1.9 | 2.7 | 2.6 |
|  | 12 | 1 | 1.3 | 4.4 | 4.5 | 1.6 | 5.4 | 6.1 | 1.4 | 5.4 | 5.6 | 1.6 | 5.3 | 6.1 |
|  |  | 2 | 0.8 | 2.6 | 2.7 | 0.9 | 3.0 | 3.2 | 1.8 | 3.8 | 3.6 | 1.2 | 3.2 | 3.1 |
|  |  | 4 | 0.5 | 1.5 | 1.5 | 0.6 | 1.9 | 2.0 | 4.1 | 5.2 | 5.1 | 2.2 | 2.9 | 2.8 |
| 268 | 2 | 1 | 0.8 | 2.9 | 2.9 | 1.0 | 4.2 | 3.8 | 0.9 | 3.3 | 3.3 | 0.9 | 3.5 | 3.3 |
|  |  | 2 | 0.5 | 1.6 | 1.7 | 0.7 | 2.2 | 2.0 | 1.3 | 2.3 | 2.3 | 0.7 | 1.9 | 1.9 |
|  |  | 4 | 0.3 | 1.0 | 0.9 | 0.4 | 1.2 | 1.2 | 3.5 | 4.0 | 3.7 | 1.7 | 2.1 | 2.0 |
|  | 4 | 1 | 0.8 | 2.9 | 2.9 | 1.1 | 3.8 | 4.0 | 0.9 | 3.6 | 3.2 | 0.9 | 3.3 | 3.5 |
|  |  | 2 | 0.5 | 1.7 | 1.6 | 0.6 | 2.2 | 2.2 | 1.3 | 2.4 | 2.4 | 0.7 | 2.0 | 2.0 |
|  |  | 4 | 0.3 | 0.9 | 0.9 | 0.4 | 1.3 | 1.3 | 3.2 | 3.8 | 3.6 | 1.8 | 2.2 | 2.2 |
|  | 6 | 1 | 0.8 | 2.9 | 3.0 | 1.0 | 4.1 | 4.1 | 0.9 | 3.2 | 3.3 | 1.0 | 3.4 | 3.5 |
|  |  | 2 | 0.5 | 1.6 | 1.7 | 0.6 | 2.1 | 2.2 | 1.3 | 2.2 | 2.3 | 0.7 | 2.0 | 2.1 |
|  |  | 4 | 0.3 | 1.0 | 1.0 | 0.4 | 1.3 | 1.3 | 3.2 | 3.6 | 3.6 | 1.8 | 2.0 | 2.1 |
|  | 12 | 1 | 0.8 | 2.9 | 2.8 | 1.1 | 3.9 | 3.7 | 0.9 | 3.2 | 3.3 | 1.0 | 3.5 | 3.4 |
|  |  | 2 | 0.5 | 1.6 | 1.6 | 0.7 | 2.2 | 2.1 | 1.3 | 2.6 | 2.2 | 0.9 | 2.1 | 2.0 |
|  |  | 4 | 0.3 | 0.9 | 0.9 | 0.4 | 1.2 | 1.1 | 3.2 | 3.5 | 3.8 | 1.7 | 2.2 | 2.3 |

Table 2: Relative PRS-RSRP error for 30kHz

| RB num | Comb | Repetition | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  | TDL-C |  |  |
|  |  |  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |
| 48 | 2 | 1 | 1.8 | 6.7 | 6.8 | 2.4 | 8.7 | 7.4 | 2.3 | 8.0 | 7.3 | 2.1 | 7.5 | 7.2 |
|  |  | 2 | 1.1 | 3.8 | 3.8 | 1.4 | 4.7 | 4.6 | 1.4 | 4.1 | 4.2 | 1.4 | 4.3 | 4.2 |
|  |  | 4 | 0.7 | 2.1 | 2.1 | 0.9 | 2.8 | 2.8 | 2.1 | 3.2 | 3.2 | 1.0 | 2.7 | 2.7 |
|  | 4 | 1 | 1.7 | 6.1 | 7.0 | 2.2 | 8.2 | 9.2 | 2.2 | 8.9 | 8.4 | 2.0 | 8.3 | 8.0 |
|  |  | 2 | 1.2 | 3.6 | 3.8 | 1.4 | 4.6 | 4.8 | 1.5 | 4.5 | 4.7 | 1.3 | 4.8 | 4.4 |
|  |  | 4 | 0.8 | 2.1 | 2.2 | 0.9 | 2.8 | 2.8 | 2.0 | 3.3 | 3.4 | 1.1 | 2.7 | 2.9 |
|  | 6 | 1 | 1.9 | 6.5 | 6.7 | 2.3 | 9.7 | 7.7 | 2.2 | 9.4 | 9.8 | 2.3 | 9.5 | 11.2 |
|  |  | 2 | 1.2 | 3.7 | 3.6 | 1.3 | 4.5 | 4.5 | 1.6 | 4.6 | 4.2 | 1.4 | 4.7 | 4.5 |
|  |  | 4 | 0.8 | 2.1 | 2.2 | 0.9 | 2.6 | 2.8 | 2.2 | 3.2 | 3.6 | 1.2 | 2.9 | 2.8 |
|  | 12 | 1 | 1.9 | 6.6 | 6.6 | 2.4 | 8.1 | 8.9 | 2.3 | 10.9 | 10.6 | 2.4 | 10.5 | 11.2 |
|  |  | 2 | 1.2 | 3.5 | 3.9 | 1.4 | 4.6 | 4.4 | 1.6 | 4.6 | 4.7 | 1.7 | 4.8 | 4.8 |
|  |  | 4 | 0.8 | 2.2 | 2.1 | 0.9 | 2.7 | 2.6 | 2.2 | 3.6 | 3.5 | 1.4 | 3.1 | 3.1 |
| 132 | 2 | 1 | 1.1 | 4.1 | 4.0 | 1.5 | 5.1 | 5.5 | 1.3 | 4.6 | 4.5 | 1.3 | 4.5 | 4.9 |
|  |  | 2 | 0.7 | 2.3 | 2.3 | 0.9 | 3.3 | 3.0 | 0.9 | 2.6 | 2.8 | 0.9 | 2.7 | 2.6 |
|  |  | 4 | 0.5 | 1.3 | 1.4 | 0.6 | 1.8 | 1.7 | 1.4 | 2.2 | 2.2 | 0.8 | 1.6 | 1.7 |
|  | 4 | 1 | 1.1 | 4.0 | 4.0 | 1.5 | 5.1 | 5.3 | 1.3 | 4.7 | 4.3 | 1.4 | 4.8 | 5.0 |
|  |  | 2 | 0.7 | 2.1 | 2.2 | 0.9 | 3.3 | 3.1 | 0.9 | 2.6 | 2.5 | 0.8 | 2.9 | 2.8 |
|  |  | 4 | 0.5 | 1.3 | 1.3 | 0.6 | 1.8 | 1.8 | 1.5 | 2.2 | 2.3 | 0.8 | 1.7 | 1.8 |
|  | 6 | 1 | 1.2 | 3.9 | 4.1 | 1.5 | 5.8 | 5.4 | 1.3 | 4.5 | 4.7 | 1.4 | 4.5 | 4.7 |
|  |  | 2 | 0.7 | 2.2 | 2.3 | 0.9 | 3.2 | 2.9 | 0.9 | 2.6 | 2.7 | 1.0 | 2.8 | 2.7 |
|  |  | 4 | 0.5 | 1.3 | 1.3 | 0.6 | 1.7 | 1.8 | 1.6 | 2.2 | 2.1 | 0.9 | 1.9 | 1.8 |
|  | 12 | 1 | 1.1 | 3.9 | 3.6 | 1.5 | 5.3 | 5.7 | 1.3 | 4.5 | 4.9 | 1.7 | 5.2 | 5.3 |
|  |  | 2 | 0.7 | 2.4 | 2.2 | 0.9 | 3.3 | 3.1 | 0.9 | 2.9 | 2.8 | 1.4 | 3.0 | 3.2 |
|  |  | 4 | 0.5 | 1.3 | 1.4 | 0.5 | 1.9 | 1.8 | 1.6 | 2.3 | 2.4 | 1.3 | 2.0 | 2.1 |
| 272 | 2 | 1 | 0.8 | 2.8 | 2.8 | 1.0 | 3.1 | 3.3 | 0.9 | 3.1 | 3.5 | 0.9 | 3.3 | 3.3 |
|  |  | 2 | 0.5 | 1.6 | 1.6 | 0.6 | 1.9 | 2.0 | 0.6 | 1.8 | 1.9 | 0.6 | 1.9 | 1.8 |
|  |  | 4 | 0.3 | 0.9 | 0.9 | 0.4 | 1.1 | 1.1 | 1.1 | 1.6 | 1.6 | 0.5 | 1.3 | 1.2 |
|  | 4 | 1 | 0.8 | 2.8 | 2.8 | 0.9 | 3.4 | 3.2 | 0.9 | 3.1 | 3.1 | 1.0 | 3.2 | 3.3 |
|  |  | 2 | 0.5 | 1.5 | 1.6 | 0.6 | 1.9 | 1.9 | 0.6 | 1.7 | 1.8 | 0.6 | 1.9 | 1.8 |
|  |  | 4 | 0.3 | 0.9 | 0.9 | 0.4 | 1.1 | 1.1 | 1.1 | 1.7 | 1.6 | 0.6 | 1.3 | 1.2 |
|  | 6 | 1 | 0.8 | 2.9 | 2.9 | 0.9 | 3.6 | 3.4 | 0.9 | 3.1 | 3.0 | 1.0 | 3.5 | 3.2 |
|  |  | 2 | 0.5 | 1.6 | 1.6 | 0.6 | 1.9 | 1.9 | 0.7 | 1.9 | 1.9 | 0.7 | 2.0 | 2.0 |
|  |  | 4 | 0.3 | 0.9 | 0.9 | 0.4 | 1.1 | 1.0 | 1.2 | 1.6 | 1.6 | 0.7 | 1.3 | 1.3 |
|  | 12 | 1 | 0.8 | 2.8 | 2.8 | 0.9 | 3.4 | 3.2 | 0.9 | 3.5 | 3.3 | 1.1 | 3.7 | 3.5 |
|  |  | 2 | 0.5 | 1.5 | 1.6 | 0.6 | 1.9 | 2.0 | 0.7 | 2.0 | 2.0 | 0.9 | 2.1 | 2.0 |
|  |  | 4 | 0.3 | 0.9 | 1.0 | 0.4 | 1.1 | 1.2 | 1.2 | 1.7 | 1.7 | 1.1 | 1.5 | 1.5 |

Table 3: Relative PRS-RSRP error for 120kHz

| RB num | Comb | Repetition | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SNR=(-6,-13) |  |  |  |  |  | SNR=(-3,-10) |  |  |  |  |  |
|  |  |  | AWGN |  |  | TDL-C |  |  | AWGN |  |  | TDL-C |  |  |
|  |  |  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |
| 32 | 2 | 1 | 2.3 | 7.5 | 9.3 | 2.6 | 11.3 | 8.8 | 1.4 | 4.3 | 4.3 | 1.6 | 5.2 | 5.1 |
|  |  | 2 | 1.4 | 4.6 | 4.5 | 1.6 | 4.7 | 5.3 | 0.9 | 2.6 | 2.7 | 1.1 | 3.1 | 3.3 |
|  |  | 4 | 0.9 | 2.7 | 2.7 | 1.1 | 3.0 | 3.2 | 0.6 | 1.6 | 1.7 | 0.8 | 2.0 | 2.0 |
|  | 4 | 1 | 2.2 | 8.4 | 7.7 | 2.7 | 9.2 | 11.9 | 1.4 | 4.2 | 4.3 | 1.8 | 5.3 | 5.3 |
|  |  | 2 | 1.4 | 4.4 | 4.2 | 1.7 | 5.2 | 5.1 | 0.9 | 2.7 | 2.7 | 1.1 | 3.2 | 3.1 |
|  |  | 4 | 0.9 | 2.5 | 2.6 | 1.2 | 3.1 | 3.0 | 0.6 | 1.6 | 1.6 | 0.8 | 2.0 | 2.1 |
|  | 6 | 1 | 2.2 | 8.3 | 8.9 | 2.6 | 11.4 | 11.9 | 1.4 | 4.7 | 4.5 | 1.7 | 5.4 | 5.6 |
|  |  | 2 | 1.4 | 4.3 | 4.3 | 1.6 | 5.2 | 5.2 | 0.9 | 2.6 | 2.6 | 1.1 | 3.1 | 3.3 |
|  |  | 4 | 1.0 | 2.7 | 2.7 | 1.2 | 3.3 | 3.1 | 0.6 | 1.5 | 1.7 | 0.9 | 2.0 | 2.1 |
|  | 12 | 1 | 2.1 | 8.8 | 7.6 | 3.0 | 15.5 | 15.4 | 1.5 | 4.9 | 4.3 | 2.0 | 6.1 | 6.2 |
|  |  | 2 | 1.5 | 4.7 | 4.6 | 1.8 | 6.6 | 5.6 | 1.0 | 2.6 | 2.6 | 1.3 | 3.6 | 3.3 |
|  |  | 4 | 0.9 | 2.7 | 2.6 | 1.6 | 3.2 | 3.6 | 0.6 | 1.7 | 1.7 | 1.2 | 2.2 | 2.2 |
| 64 | 2 | 1 | 1.6 | 5.7 | 6.2 | 1.9 | 6.6 | 6.5 | 1.0 | 3.1 | 3.1 | 1.2 | 3.9 | 4.0 |
|  |  | 2 | 1.0 | 3.0 | 3.1 | 1.1 | 3.5 | 3.7 | 0.7 | 1.9 | 1.9 | 0.8 | 2.2 | 2.2 |
|  |  | 4 | 0.6 | 2.0 | 1.8 | 0.8 | 2.2 | 2.3 | 0.4 | 1.1 | 1.2 | 0.6 | 1.4 | 1.4 |
|  | 4 | 1 | 1.6 | 5.2 | 5.9 | 2.0 | 7.3 | 7.2 | 1.0 | 3.2 | 3.1 | 1.2 | 3.7 | 3.8 |
|  |  | 2 | 1.0 | 3.2 | 3.4 | 1.2 | 3.6 | 3.7 | 0.6 | 1.8 | 2.1 | 0.8 | 2.2 | 2.2 |
|  |  | 4 | 0.7 | 1.9 | 2.0 | 0.8 | 2.3 | 2.3 | 0.4 | 1.1 | 1.1 | 0.6 | 1.4 | 1.4 |
|  | 6 | 1 | 1.6 | 6.2 | 5.3 | 1.9 | 6.6 | 6.8 | 1.0 | 3.2 | 3.2 | 1.2 | 3.8 | 3.9 |
|  |  | 2 | 1.0 | 3.2 | 3.1 | 1.2 | 3.7 | 3.9 | 0.6 | 1.9 | 1.9 | 0.8 | 2.1 | 2.2 |
|  |  | 4 | 0.7 | 1.8 | 2.0 | 0.9 | 2.3 | 2.3 | 0.4 | 1.2 | 1.2 | 0.7 | 1.5 | 1.6 |
|  | 12 | 1 | 1.6 | 5.3 | 5.2 | 2.1 | 7.4 | 8.6 | 1.0 | 3.1 | 3.1 | 1.4 | 4.0 | 4.3 |
|  |  | 2 | 1.0 | 3.3 | 3.0 | 1.4 | 4.2 | 3.7 | 0.7 | 1.9 | 1.8 | 1.0 | 2.4 | 2.7 |
|  |  | 4 | 0.6 | 1.9 | 1.9 | 1.2 | 2.5 | 2.5 | 0.4 | 1.1 | 1.2 | 1.0 | 1.7 | 1.6 |
| 128 | 2 | 1 | 1.2 | 3.9 | 3.9 | 1.3 | 4.4 | 4.5 | 0.7 | 2.3 | 2.3 | 0.8 | 2.7 | 2.8 |
|  |  | 2 | 0.7 | 2.3 | 2.1 | 0.9 | 2.6 | 2.6 | 0.5 | 1.4 | 1.4 | 0.6 | 1.6 | 1.6 |
|  |  | 4 | 0.5 | 1.3 | 1.4 | 0.6 | 1.7 | 1.7 | 0.3 | 0.8 | 0.9 | 0.5 | 1.0 | 1.0 |
|  | 4 | 1 | 1.1 | 3.9 | 4.1 | 1.3 | 4.6 | 4.6 | 0.7 | 2.2 | 2.4 | 0.9 | 2.5 | 2.8 |
|  |  | 2 | 0.7 | 2.3 | 2.3 | 0.8 | 2.6 | 2.7 | 0.5 | 1.4 | 1.4 | 0.6 | 1.6 | 1.6 |
|  |  | 4 | 0.5 | 1.3 | 1.4 | 0.6 | 1.6 | 1.7 | 0.3 | 0.8 | 0.8 | 0.5 | 1.1 | 1.0 |
|  | 6 | 1 | 1.1 | 3.9 | 4.1 | 1.3 | 4.8 | 5.0 | 0.7 | 2.3 | 2.3 | 0.9 | 2.9 | 2.7 |
|  |  | 2 | 0.7 | 2.3 | 2.3 | 0.9 | 2.5 | 2.7 | 0.5 | 1.3 | 1.4 | 0.6 | 1.7 | 1.7 |
|  |  | 4 | 0.5 | 1.3 | 1.4 | 0.6 | 1.6 | 1.6 | 0.3 | 0.9 | 0.9 | 0.5 | 1.1 | 1.1 |
|  | 12 | 1 | 1.2 | 4.2 | 3.8 | 1.5 | 5.1 | 5.6 | 0.7 | 2.4 | 2.4 | 1.0 | 2.8 | 2.9 |
|  |  | 2 | 0.7 | 2.2 | 2.2 | 1.1 | 3.0 | 2.9 | 0.5 | 1.4 | 1.4 | 0.9 | 1.7 | 1.8 |
|  |  | 4 | 0.5 | 1.3 | 1.4 | 0.9 | 1.9 | 1.8 | 0.3 | 0.8 | 0.8 | 0.8 | 1.3 | 1.3 |

Simulation results for absolute PRS-RSRP accuracy (new)

Table 4-6 show the simulation results for absolute PRS-RSRP accuracy. The absolute PRS-RSRP accuracy is calculated as the max between 95%-tile and 5%-tile values in the CDF for each cell.

Table 4: absolute PRS-RSRP error for 15kHz

| RB num | Comb | Repetition | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  | TDL-C |  |  |
|  |  |  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |
| 52 | 2 | 1 | 1.20 | 3.44 | 3.45 | 1.17 | 5.12 | 5.35 | 1.10 | 4.20 | 4.37 | 1.00 | 3.78 | 4.47 |
|  |  | 2 | 0.72 | 2.16 | 2.26 | 0.79 | 2.58 | 2.34 | 2.27 | 3.60 | 3.47 | 0.91 | 2.54 | 2.40 |
|  |  | 4 | 0.51 | 1.33 | 1.41 | 0.48 | 1.48 | 1.43 | 6.33 | 7.06 | 7.82 | 2.21 | 2.77 | 2.87 |
|  | 4 | 1 | 1.18 | 3.46 | 3.49 | 1.29 | 4.89 | 6.12 | 1.15 | 4.62 | 3.96 | 1.09 | 5.33 | 4.27 |
|  |  | 2 | 0.79 | 2.20 | 2.20 | 0.68 | 2.64 | 2.88 | 2.21 | 3.50 | 3.61 | 0.90 | 2.38 | 2.42 |
|  |  | 4 | 0.50 | 1.35 | 1.45 | 0.46 | 1.49 | 1.47 | 6.69 | 7.27 | 7.17 | 2.33 | 2.78 | 2.91 |
|  | 6 | 1 | 1.21 | 3.48 | 3.46 | 1.27 | 5.52 | 4.20 | 1.13 | 4.44 | 5.44 | 1.07 | 5.31 | 6.08 |
|  |  | 2 | 0.80 | 2.20 | 2.24 | 0.75 | 2.82 | 2.49 | 2.17 | 3.75 | 3.88 | 1.01 | 2.64 | 2.68 |
|  |  | 4 | 0.51 | 1.37 | 1.44 | 0.44 | 1.47 | 1.68 | 6.22 | 7.06 | 7.24 | 2.29 | 2.77 | 2.92 |
|  | 12 | 1 | 1.18 | 3.47 | 3.44 | 1.15 | 4.82 | 8.11 | 1.24 | 5.01 | 5.24 | 1.47 | 6.74 | 5.39 |
|  |  | 2 | 0.74 | 2.28 | 2.22 | 0.75 | 2.89 | 2.35 | 2.16 | 3.66 | 3.61 | 1.25 | 2.73 | 3.26 |
|  |  | 4 | 0.53 | 1.43 | 1.44 | 0.48 | 1.56 | 1.50 | 6.16 | 6.92 | 7.18 | 2.63 | 3.10 | 3.13 |
| 104 | 2 | 1 | 0.86 | 2.53 | 2.62 | 0.79 | 3.14 | 2.86 | 0.71 | 2.47 | 2.89 | 0.81 | 2.54 | 2.73 |
|  |  | 2 | 0.55 | 1.68 | 1.59 | 0.48 | 1.69 | 1.52 | 1.82 | 2.66 | 2.91 | 0.75 | 1.82 | 1.56 |
|  |  | 4 | 0.36 | 1.03 | 1.01 | 0.33 | 1.04 | 0.93 | 6.21 | 6.19 | 6.15 | 2.07 | 2.43 | 2.63 |
|  | 4 | 1 | 0.85 | 2.88 | 2.68 | 0.80 | 3.09 | 4.12 | 0.81 | 2.68 | 2.90 | 0.81 | 2.51 | 2.65 |
|  |  | 2 | 0.55 | 1.67 | 1.74 | 0.52 | 1.64 | 1.83 | 1.90 | 2.98 | 2.72 | 0.81 | 1.77 | 1.87 |
|  |  | 4 | 0.37 | 1.00 | 1.05 | 0.32 | 0.94 | 0.93 | 6.24 | 6.42 | 6.62 | 1.98 | 2.40 | 2.53 |
|  | 6 | 1 | 0.86 | 2.64 | 2.74 | 0.79 | 2.82 | 3.17 | 0.72 | 2.76 | 2.99 | 0.86 | 3.21 | 2.47 |
|  |  | 2 | 0.55 | 1.61 | 1.72 | 0.51 | 1.56 | 1.55 | 1.92 | 2.75 | 2.69 | 0.87 | 1.82 | 1.89 |
|  |  | 4 | 0.38 | 1.02 | 1.01 | 0.33 | 0.91 | 0.95 | 6.08 | 6.38 | 6.39 | 2.19 | 2.58 | 2.49 |
|  | 12 | 1 | 0.86 | 2.74 | 2.68 | 0.89 | 2.80 | 3.33 | 0.75 | 2.86 | 3.11 | 1.07 | 2.95 | 3.72 |
|  |  | 2 | 0.57 | 1.72 | 1.73 | 0.47 | 1.60 | 1.63 | 1.92 | 2.96 | 2.77 | 1.10 | 2.04 | 2.11 |
|  |  | 4 | 0.35 | 1.00 | 1.03 | 0.33 | 1.00 | 1.05 | 5.77 | 6.63 | 6.35 | 2.61 | 2.92 | 2.74 |
| 268 | 2 | 1 | 0.55 | 1.87 | 1.86 | 0.52 | 2.18 | 1.97 | 0.45 | 1.69 | 1.71 | 0.49 | 1.82 | 1.65 |
|  |  | 2 | 0.34 | 1.13 | 1.12 | 0.34 | 1.13 | 0.99 | 1.54 | 1.99 | 1.98 | 0.61 | 1.16 | 1.23 |
|  |  | 4 | 0.23 | 0.66 | 0.65 | 0.21 | 0.59 | 0.61 | 5.53 | 5.84 | 5.52 | 2.04 | 2.26 | 2.10 |
|  | 4 | 1 | 0.59 | 1.91 | 1.81 | 0.59 | 2.01 | 2.07 | 0.47 | 1.95 | 1.63 | 0.50 | 1.66 | 1.83 |
|  |  | 2 | 0.33 | 1.16 | 1.11 | 0.33 | 1.17 | 1.13 | 1.57 | 2.07 | 2.07 | 0.62 | 1.28 | 1.29 |
|  |  | 4 | 0.22 | 0.64 | 0.63 | 0.21 | 0.66 | 0.73 | 5.28 | 5.58 | 5.48 | 2.08 | 2.29 | 2.38 |
|  | 6 | 1 | 0.58 | 1.86 | 1.95 | 0.52 | 2.26 | 2.12 | 0.49 | 1.63 | 1.75 | 0.55 | 1.71 | 1.94 |
|  |  | 2 | 0.35 | 1.08 | 1.12 | 0.32 | 1.07 | 1.18 | 1.54 | 1.90 | 2.04 | 0.67 | 1.29 | 1.41 |
|  |  | 4 | 0.23 | 0.69 | 0.65 | 0.21 | 0.68 | 0.70 | 5.28 | 5.39 | 5.45 | 2.12 | 2.20 | 2.27 |
|  | 12 | 1 | 0.57 | 1.86 | 1.85 | 0.56 | 1.98 | 1.88 | 0.46 | 1.64 | 1.68 | 0.74 | 1.93 | 2.08 |
|  |  | 2 | 0.35 | 1.09 | 1.13 | 0.34 | 1.17 | 1.08 | 1.58 | 2.22 | 1.96 | 0.95 | 1.50 | 1.49 |
|  |  | 4 | 0.23 | 0.67 | 0.65 | 0.20 | 0.64 | 0.62 | 5.29 | 5.41 | 5.65 | 2.19 | 2.51 | 2.53 |

Table 5: absolute PRS-RSRP error for 30kHz

| RB num | Comb | Repetition | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  | TDL-C |  |  |
|  |  |  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |
| 48 | 2 | 1 | 1.25 | 3.60 | 3.67 | 1.21 | 5.10 | 4.02 | 1.18 | 4.69 | 4.00 | 1.08 | 4.44 | 4.00 |
|  |  | 2 | 0.83 | 2.27 | 2.26 | 0.70 | 2.39 | 2.43 | 0.99 | 2.38 | 2.36 | 0.78 | 2.39 | 2.22 |
|  |  | 4 | 0.51 | 1.45 | 1.47 | 0.48 | 1.52 | 1.49 | 2.22 | 2.67 | 2.75 | 0.85 | 1.68 | 1.71 |
|  | 4 | 1 | 1.24 | 3.47 | 3.53 | 1.11 | 4.73 | 5.61 | 1.11 | 5.35 | 4.98 | 1.17 | 5.22 | 4.98 |
|  |  | 2 | 0.80 | 2.23 | 2.27 | 0.71 | 2.38 | 2.66 | 1.04 | 2.62 | 2.81 | 0.83 | 2.92 | 2.54 |
|  |  | 4 | 0.52 | 1.36 | 1.46 | 0.47 | 1.41 | 1.45 | 2.20 | 2.68 | 2.91 | 0.98 | 1.81 | 1.96 |
|  | 6 | 1 | 1.22 | 3.58 | 3.52 | 1.28 | 6.06 | 4.13 | 1.19 | 6.03 | 6.24 | 1.37 | 6.40 | 8.18 |
|  |  | 2 | 0.87 | 2.31 | 2.31 | 0.71 | 2.44 | 2.36 | 1.09 | 2.79 | 2.40 | 1.00 | 2.78 | 2.65 |
|  |  | 4 | 0.56 | 1.44 | 1.41 | 0.46 | 1.32 | 1.44 | 2.37 | 2.75 | 3.00 | 1.13 | 2.03 | 1.93 |
|  | 12 | 1 | 1.27 | 3.35 | 3.61 | 1.24 | 4.54 | 5.32 | 1.34 | 7.68 | 7.45 | 1.76 | 7.74 | 8.13 |
|  |  | 2 | 0.80 | 2.32 | 2.42 | 0.74 | 2.53 | 2.36 | 1.23 | 2.80 | 2.79 | 1.51 | 3.14 | 3.19 |
|  |  | 4 | 0.54 | 1.39 | 1.48 | 0.44 | 1.43 | 1.35 | 2.48 | 3.08 | 3.19 | 1.58 | 2.49 | 2.41 |
| 132 | 2 | 1 | 0.76 | 2.41 | 2.46 | 0.78 | 2.56 | 2.96 | 0.65 | 2.37 | 2.28 | 0.68 | 2.28 | 2.67 |
|  |  | 2 | 0.47 | 1.50 | 1.48 | 0.45 | 1.67 | 1.56 | 0.68 | 1.53 | 1.62 | 0.54 | 1.48 | 1.35 |
|  |  | 4 | 0.33 | 0.90 | 0.97 | 0.30 | 0.93 | 0.85 | 1.74 | 2.25 | 2.07 | 0.71 | 1.09 | 1.16 |
|  | 4 | 1 | 0.74 | 2.44 | 2.49 | 0.77 | 2.61 | 2.90 | 0.66 | 2.34 | 2.29 | 0.82 | 2.57 | 2.75 |
|  |  | 2 | 0.47 | 1.44 | 1.54 | 0.45 | 1.64 | 1.61 | 0.72 | 1.54 | 1.47 | 0.60 | 1.67 | 1.72 |
|  |  | 4 | 0.31 | 0.89 | 0.97 | 0.29 | 0.94 | 0.93 | 1.87 | 2.14 | 2.28 | 0.78 | 1.22 | 1.36 |
|  | 6 | 1 | 0.79 | 2.45 | 2.43 | 0.80 | 3.19 | 2.71 | 0.65 | 2.36 | 2.44 | 0.97 | 2.56 | 2.52 |
|  |  | 2 | 0.50 | 1.45 | 1.57 | 0.47 | 1.63 | 1.48 | 0.71 | 1.47 | 1.64 | 0.83 | 1.68 | 1.74 |
|  |  | 4 | 0.32 | 0.88 | 0.88 | 0.30 | 0.87 | 0.89 | 1.89 | 2.23 | 2.12 | 1.05 | 1.57 | 1.48 |
|  | 12 | 1 | 0.76 | 2.46 | 2.33 | 0.75 | 2.68 | 3.06 | 0.82 | 2.34 | 2.57 | 1.47 | 3.30 | 3.38 |
|  |  | 2 | 0.51 | 1.51 | 1.47 | 0.52 | 1.66 | 1.54 | 0.83 | 1.81 | 1.76 | 1.40 | 2.18 | 2.32 |
|  |  | 4 | 0.32 | 0.96 | 0.93 | 0.29 | 1.04 | 0.93 | 2.08 | 2.40 | 2.45 | 1.51 | 1.85 | 1.98 |
| 272 | 2 | 1 | 0.57 | 1.76 | 1.86 | 0.53 | 1.63 | 1.74 | 0.47 | 1.58 | 1.75 | 0.48 | 1.74 | 1.66 |
|  |  | 2 | 0.36 | 1.07 | 1.13 | 0.29 | 0.95 | 1.03 | 0.53 | 1.06 | 1.19 | 0.39 | 1.09 | 0.95 |
|  |  | 4 | 0.23 | 0.64 | 0.64 | 0.19 | 0.57 | 0.58 | 1.60 | 1.79 | 1.80 | 0.55 | 1.00 | 0.88 |
|  | 4 | 1 | 0.56 | 1.83 | 1.76 | 0.45 | 1.75 | 1.63 | 0.47 | 1.57 | 1.67 | 0.59 | 1.78 | 1.74 |
|  |  | 2 | 0.34 | 1.08 | 1.06 | 0.29 | 1.02 | 0.99 | 0.52 | 1.03 | 1.16 | 0.48 | 1.12 | 1.12 |
|  |  | 4 | 0.23 | 0.63 | 0.60 | 0.20 | 0.59 | 0.62 | 1.63 | 1.88 | 1.79 | 0.68 | 1.04 | 0.97 |
|  | 6 | 1 | 0.56 | 1.81 | 1.84 | 0.49 | 1.93 | 1.80 | 0.53 | 1.62 | 1.62 | 0.73 | 1.96 | 1.77 |
|  |  | 2 | 0.36 | 1.11 | 1.08 | 0.31 | 1.02 | 0.99 | 0.57 | 1.15 | 1.21 | 0.65 | 1.32 | 1.27 |
|  |  | 4 | 0.23 | 0.62 | 0.65 | 0.20 | 0.56 | 0.53 | 1.65 | 1.84 | 1.87 | 0.82 | 1.18 | 1.22 |
|  | 12 | 1 | 0.56 | 1.83 | 1.80 | 0.47 | 1.81 | 1.65 | 0.62 | 1.84 | 1.85 | 1.15 | 2.44 | 2.30 |
|  |  | 2 | 0.35 | 1.07 | 1.09 | 0.31 | 0.94 | 1.09 | 0.72 | 1.33 | 1.41 | 1.08 | 1.66 | 1.63 |
|  |  | 4 | 0.22 | 0.64 | 0.71 | 0.22 | 0.59 | 0.59 | 1.80 | 2.04 | 1.97 | 1.40 | 1.66 | 1.61 |

Table 6: absolute PRS-RSRP error for 120kHz

| RB num | Comb | Repetition | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SNR=(-6,-13) |  |  |  |  |  | SNR=(-3,-10) |  |  |  |  |  |
|  |  |  | AWGN |  |  | TDL-C |  |  | AWGN |  |  | TDL-C |  |  |
|  |  |  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |
| 32 | 2 | 1 | 1.51 | 4.11 | 5.37 | 1.32 | 7.50 | 4.96 | 0.98 | 2.66 | 2.69 | 0.80 | 2.66 | 2.68 |
|  |  | 2 | 0.93 | 2.62 | 2.63 | 0.89 | 2.38 | 2.81 | 0.64 | 1.75 | 1.71 | 0.62 | 1.65 | 1.74 |
|  |  | 4 | 0.66 | 1.72 | 1.70 | 0.76 | 1.73 | 1.82 | 0.44 | 1.15 | 1.10 | 0.60 | 1.22 | 1.17 |
|  | 4 | 1 | 1.51 | 4.37 | 4.31 | 1.48 | 5.32 | 7.97 | 0.96 | 2.64 | 2.80 | 0.97 | 2.69 | 2.75 |
|  |  | 2 | 0.94 | 2.69 | 2.75 | 0.98 | 2.95 | 2.80 | 0.64 | 1.72 | 1.81 | 0.72 | 1.75 | 1.76 |
|  |  | 4 | 0.62 | 1.72 | 1.77 | 0.82 | 1.70 | 1.70 | 0.44 | 1.11 | 1.25 | 0.66 | 1.26 | 1.40 |
|  | 6 | 1 | 1.50 | 4.18 | 4.87 | 1.45 | 7.52 | 7.92 | 0.99 | 2.69 | 2.58 | 1.02 | 3.00 | 3.26 |
|  |  | 2 | 0.96 | 2.80 | 2.47 | 1.05 | 2.92 | 2.97 | 0.62 | 1.75 | 1.64 | 0.76 | 1.82 | 1.99 |
|  |  | 4 | 0.66 | 1.71 | 1.69 | 0.91 | 2.01 | 1.92 | 0.47 | 1.06 | 0.97 | 0.77 | 1.30 | 1.37 |
|  | 12 | 1 | 1.45 | 4.54 | 4.23 | 2.10 | 11.97 | 11.79 | 1.01 | 2.69 | 2.78 | 1.55 | 3.87 | 4.01 |
|  |  | 2 | 0.98 | 2.59 | 2.70 | 1.46 | 4.26 | 3.57 | 0.68 | 1.69 | 1.78 | 1.19 | 2.47 | 2.09 |
|  |  | 4 | 0.61 | 1.75 | 1.77 | 1.48 | 2.26 | 2.61 | 0.45 | 1.15 | 1.20 | 1.22 | 1.73 | 1.82 |
| 64 | 2 | 1 | 1.04 | 3.23 | 3.22 | 0.93 | 3.63 | 3.52 | 0.71 | 1.96 | 2.00 | 0.64 | 2.08 | 2.25 |
|  |  | 2 | 0.72 | 2.04 | 2.03 | 0.56 | 1.82 | 1.94 | 0.46 | 1.32 | 1.25 | 0.43 | 1.20 | 1.13 |
|  |  | 4 | 0.45 | 1.29 | 1.23 | 0.58 | 1.30 | 1.31 | 0.31 | 0.82 | 0.79 | 0.50 | 0.95 | 0.88 |
|  | 4 | 1 | 1.13 | 3.19 | 3.31 | 1.15 | 4.35 | 4.28 | 0.68 | 2.09 | 1.94 | 0.69 | 1.88 | 2.03 |
|  |  | 2 | 0.71 | 2.04 | 1.93 | 0.70 | 1.98 | 1.92 | 0.46 | 1.24 | 1.25 | 0.51 | 1.18 | 1.22 |
|  |  | 4 | 0.46 | 1.26 | 1.25 | 0.65 | 1.44 | 1.35 | 0.31 | 0.78 | 0.73 | 0.56 | 1.00 | 0.96 |
|  | 6 | 1 | 1.05 | 3.28 | 3.25 | 1.22 | 3.60 | 3.85 | 0.67 | 2.04 | 2.19 | 0.76 | 2.16 | 2.11 |
|  |  | 2 | 0.73 | 2.04 | 2.14 | 0.81 | 2.17 | 2.18 | 0.47 | 1.29 | 1.42 | 0.59 | 1.25 | 1.33 |
|  |  | 4 | 0.46 | 1.22 | 1.38 | 0.80 | 1.39 | 1.46 | 0.31 | 0.81 | 0.95 | 0.65 | 1.03 | 1.20 |
|  | 12 | 1 | 1.07 | 3.15 | 3.32 | 1.58 | 4.61 | 5.79 | 0.70 | 2.03 | 2.11 | 1.17 | 2.53 | 2.74 |
|  |  | 2 | 0.69 | 2.05 | 2.12 | 1.24 | 2.79 | 2.21 | 0.46 | 1.31 | 1.33 | 1.02 | 1.70 | 1.89 |
|  |  | 4 | 0.46 | 1.27 | 1.39 | 1.25 | 1.95 | 1.91 | 0.33 | 0.77 | 0.96 | 1.13 | 1.51 | 1.40 |
| 128 | 2 | 1 | 0.84 | 2.50 | 2.40 | 0.64 | 2.25 | 2.27 | 0.52 | 1.58 | 1.53 | 0.42 | 1.42 | 1.48 |
|  |  | 2 | 0.49 | 1.54 | 1.48 | 0.48 | 1.40 | 1.33 | 0.32 | 0.95 | 0.92 | 0.33 | 0.83 | 0.87 |
|  |  | 4 | 0.33 | 0.88 | 0.96 | 0.46 | 1.02 | 1.01 | 0.23 | 0.57 | 0.60 | 0.41 | 0.69 | 0.63 |
|  | 4 | 1 | 0.75 | 2.46 | 2.48 | 0.74 | 2.51 | 2.33 | 0.51 | 1.45 | 1.57 | 0.55 | 1.35 | 1.50 |
|  |  | 2 | 0.50 | 1.46 | 1.59 | 0.53 | 1.41 | 1.56 | 0.34 | 0.95 | 1.02 | 0.41 | 0.91 | 0.86 |
|  |  | 4 | 0.34 | 0.88 | 0.98 | 0.56 | 1.01 | 1.10 | 0.22 | 0.60 | 0.64 | 0.48 | 0.77 | 0.75 |
|  | 6 | 1 | 0.78 | 2.47 | 2.39 | 0.82 | 2.65 | 2.77 | 0.50 | 1.51 | 1.45 | 0.62 | 1.57 | 1.56 |
|  |  | 2 | 0.48 | 1.53 | 1.43 | 0.68 | 1.44 | 1.59 | 0.34 | 0.91 | 0.80 | 0.50 | 1.08 | 1.04 |
|  |  | 4 | 0.34 | 0.89 | 0.84 | 0.65 | 1.18 | 1.10 | 0.23 | 0.61 | 0.55 | 0.59 | 0.86 | 0.87 |
|  | 12 | 1 | 0.80 | 2.53 | 2.41 | 1.24 | 3.12 | 3.52 | 0.48 | 1.54 | 1.52 | 0.99 | 1.87 | 2.03 |
|  |  | 2 | 0.51 | 1.47 | 1.45 | 1.04 | 2.04 | 1.92 | 0.35 | 0.95 | 0.90 | 0.93 | 1.39 | 1.46 |
|  |  | 4 | 0.32 | 0.91 | 0.90 | 1.08 | 1.58 | 1.58 | 0.22 | 0.59 | 0.55 | 1.06 | 1.28 | 1.31 |

From the tables, we can observe that

Observation 1: The performance for all comb size are similar.

Observation 2: Repetition up to 4 improves the performance for all RB number in all channels, while the gain is more significant for small RB number at low SNR.

Observation 3: The performance are dependent on the channel profile, but the difference is not much.

Observation 4: The accuracy improves in proportion with BW in RB due to more REs for estimation.

Observation 5: The performance is quite dependent on SNR condition.

Conclusions

In this paper we provided further simulation results for absolute PRS-RSRP measurement accuracy. The performance trends are almost same for absolute and relative accuracy.

Observation 1: The performance for all comb size are similar.

Observation 2: Repetition up to 4 improves the performance for all RB number in all channels, while the gain is more significant for small RB number at low SNR.

Observation 3: The performance are dependent on the channel profile, but the difference is not much.

Observation 4: The accuracy improves in proportion with BW in RB due to more REs for estimation.

Observation 5: The performance is quite dependent on SNR condition.

Reference

- RP-2017371, WF on NR Positioning Performance Requirements, Intel Corporation

- R4-2004362, Link level simulation results for PRS-RSRP measurement, Huawei, HiSilicon

- R4-2002287, Updated link level simulation assumption for RSTD and RSRP, Huawei, HiSilicon