3GPP TSG-RAN WG4 Meeting #98-e		                               R4-2102932

Electronic Meeting, 25 January – 5 February, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	n48 30MHz A-MPR Measurements

Agenda Item:		9.21.2.2 MPR/A-MPR/NS signaling [NR_bands_R17_BWs -Core]

Document for:	Approval

# Introduction

At the last RAN plenary meeting, the addition of 30MHz channel bandwidth (CBW) was added to band n48. This contribution presents Power Amplifier (PA) back-off measurement data to propose an A-MPR table for NS_27 30MHz CBW. Some minor corrections are also proposed for 40MHz CBW operation.

# Discussion

## Power Amplifier Calibration

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR;

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc; and

- IQ Image rejection: -28dB.

## NS_27 30MHz Measurement Results

PA raw back-off required to pass NS_27 requirements are presented in Table 1 for 30MHz CBW, and carrier frequency “Fc” equal to 3565 MHz using DFT-s-OFDM QPSK, SCS15kHz waveforms.

Table 1: NS-27 30MHz raw PA Back-off for DFT-S-OFDM Power Class 3 – Fc=3565MHz.

| LCRB | RBstart | Back-off |
| --- | --- | --- |
| 1 | 0 | 9.5 |
| 1 | 10 | 9.5 |
| 1 | 20 | 2.3 |
| 1 | 30 | 2.4 |
| 1 | 149 | 4.0 |
| 1 | 159 | 3.9 |
| 2 | 0 | 9.4 |
| 2 | 10 | 9.4 |
| 2 | 148 | 3.7 |
| 2 | 158 | 3.8 |
| 5 | 0 | 8.0 |
| 5 | 10 | 8.0 |
| 5 | 145 | 7.2 |
| 5 | 155 | 6.9 |
| 10 | 0 | 5.7 |
| 10 | 10 | 5.5 |
| 10 | 130 | 2.5 |
| 10 | 150 | 2.3 |
| 20 | 0 | 4.3 |
| 50 | 0 | 5.5 |
| 100 | 0 | 6.5 |
| 160 | 0 | 9.9 |

## NS_27 A-MPR proposal for 30MHz and corrections for 40MHz

The measured data is consistent with data obtained for 40MHz CBW operation. For the low A-MPR region, in Table 2 below, we specify the maximum allocation size at which the fifth order intermodulation distortion product (IMD5) no longer falls in the -40dBm/MHz region using the equation proposed in [1]. Our calculation indicates that for 40MHz CBW operation, the maximum allocation should be corrected from 40MHz to 25MHz. Applying the 2,3,5 factor rule, we find that the actual allocation should be corrected from 20MHz to 24.3MHz. Corrections for 40MHz and values obtained for 30MHz operation are summarized in Table 2 below.

Table 2: Maximum Allocation Size for IM5 to miss -40dBm/MHz region – Example of Carrier at Lower Edge of n48.

| Channel Bandwidth, MHz | -40dBm/MHz Edge, MHz | Channel Edge, MHz | Fc, MHz for low AMPR start | Maximum Allocation, MHz | Actual Allocation, MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 3530 | 3555 | 3562.5 | 12.5 | 11.52 |
| 20 |  | 3560 | 3570 | 15 | 14.4 |
| 30 |  | 3570 | 3585 | 20 | 19.44 |
| 40 |  | 3580 | 3600 | 25 | 24.3 |

We also note that, in 38.101-1 Table 6.2.3.16-1 “A-MPR for NS_27”, the majority of Lcrb/RBstart/RBend region boundaries expressed in MHz are multiple integers of a resource block (RB) except for 40MHz CBW, Region A, where for 3570 ≤ FC < 3600, one border is set to 31.0 MHz, a value which is not an integer multiple of a RB. We propose to replace 31.0MHz with 30.96MHz.

The combined changes to the A-MPR table for NS_27 are presented in Proposal 1.

Proposal 1: A-MPR table for NS_27

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS | A-MPR | LCRB*12*SCS | A-MPR |
| 15 MHz | 3557.5 ≤ FC < 3562.5 | <1.8 MHz |  |  | A3 | ≥10.8 MHz | A3 |
|  | 3687.5 < FC ≤ 3692.5 | >11.52 MHz |  |  |  |  |  |
| 15 MHz | 3562.5 ≤ FC < 3567.5 | ≤1.08 MHz |  | <1.44 MHz | A4 | ≥11.52 MHz | [2] |
|  | 3682.5 < FC ≤ 3687.5 |  | ≥13.22 MHz |  |  |  |  |
| 20 MHz | 3560 ≤ FC < 3570 | <3.6 MHz |  |  | A5 | ≥10.8 MHz | A5 |
|  | 3680 < FC ≤ 3690 | >12.96 MHz |  |  |  |  |  |
| 20 MHz | 3570 ≤ FC < 3580 | ≤2.16 MHz |  | <1.44 MHz | A6 | ≥14.4 MHz | [2] |
|  | 3670 < FC ≤ 3680 |  | ≥16.92 |  |  |  |  |
| 30 MHz | 3565 ≤ FC < 3585 | <[7.2] MHz |  |  | A7 |  |  |
|  |  | ≥[7.2] MHz, ≤[24.3] MHz |  | ≥[15.3]MHz | A2 |  |  |
|  |  |  |  | <[15.3] MHz | A1 |  |  |
|  |  | >[24.3] MHz |  | <[2.7] MHz | A7 |  |  |
|  | 3665 < FC ≤ 3685 |  | >[21.6] MHz |  | A7 |  |  |
|  |  |  | ≤[21.6] MHz, ≥[4.32] MHz | ≥[15.3] MHz | A2 |  |  |
|  |  |  |  | <[15.3] MHz | A1 |  |  |
|  |  |  | <[4.32] MHz | <[2.7] MHz | A7 |  |  |
|  | 3585 ≤ FC ≤ 3665 | ≤[3.96] MHz |  | <[1.44] MHz | A8 | >[19.44] MHz | [3.5] |
|  |  |  | ≥ [24.66] |  |  |  |  |
| 40 MHz | 3570 ≤ FC < 3600 | <11.34 MHz |  |  | A7 |  |  |
|  |  | ≥11.34 MHz, ≤30.96MHz |  | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  | >30.96 MHz |  | <3.6 MHz | A7 |  |  |
|  | 3650 < FC ≤ 3680 |  | >24.48 MHz |  | A7 |  |  |
|  |  |  | ≤24.48 MHz, ≥6.48 MHz | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  |  | <6.48 MHz | <3.6 MHz | A7 |  |  |
| 40 MHz | 3600 ≤ FC ≤ 3650 | ≤6.12 MHz |  | <1.44 MHz | A8 | >[24.3] MHz | [4.5] |
|  |  |  | ≥ 32.76 |  |  |  |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution we present raw PA back-off measurements for n48 30MHz CBW operation with carrier located at the edge of n48. We make the following A-MPR table proposal for NS_27:

Proposal 1:

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS | A-MPR | LCRB*12*SCS | A-MPR |
| 15 MHz | 3557.5 ≤ FC < 3562.5 | <1.8 MHz |  |  | A3 | ≥10.8 MHz | A3 |
|  | 3687.5 < FC ≤ 3692.5 | >11.52 MHz |  |  |  |  |  |
| 15 MHz | 3562.5 ≤ FC < 3567.5 | ≤1.08 MHz |  | <1.44 MHz | A4 | ≥11.52 MHz | [2] |
|  | 3682.5 < FC ≤ 3687.5 |  | ≥13.22 MHz |  |  |  |  |
| 20 MHz | 3560 ≤ FC < 3570 | <3.6 MHz |  |  | A5 | ≥10.8 MHz | A5 |
|  | 3680 < FC ≤ 3690 | >12.96 MHz |  |  |  |  |  |
| 20 MHz | 3570 ≤ FC < 3580 | ≤2.16 MHz |  | <1.44 MHz | A6 | ≥14.4 MHz | [2] |
|  | 3670 < FC ≤ 3680 |  | ≥16.92 |  |  |  |  |
| 30 MHz | 3565 ≤ FC < 3585 | <[7.2] MHz |  |  | A7 |  |  |
|  |  | ≥[7.2] MHz, ≤[24.3] MHz |  | ≥[15.3]MHz | A2 |  |  |
|  |  |  |  | <[15.3] MHz | A1 |  |  |
|  |  | >[24.3] MHz |  | <[2.7] MHz | A7 |  |  |
|  | 3665 < FC ≤ 3685 |  | >[21.6] MHz |  | A7 |  |  |
|  |  |  | ≤[21.6] MHz, ≥[4.32] MHz | ≥[15.3] MHz | A2 |  |  |
|  |  |  |  | <[15.3] MHz | A1 |  |  |
|  |  |  | <[4.32] MHz | <[2.7] MHz | A7 |  |  |
|  | 3585 ≤ FC ≤ 3665 | ≤[3.96] MHz |  | <[1.44] MHz |  | >[19.44] MHz | [3.5] |
|  |  |  | ≥ [24.66] |  |  |  |  |
| 40 MHz | 3570 ≤ FC < 3600 | <11.34 MHz |  |  | A7 |  |  |
|  |  | ≥11.34 MHz, ≤30.96MHz |  | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  | >30.96 MHz |  | <3.6 MHz | A7 |  |  |
|  | 3650 < FC ≤ 3680 |  | >24.48 MHz |  | A7 |  |  |
|  |  |  | ≤24.48 MHz, ≥6.48 MHz | ≥18 MHz | A2 |  |  |
|  |  |  |  | <18 MHz | A1 |  |  |
|  |  |  | <6.48 MHz | <3.6 MHz | A7 |  |  |
| 40 MHz | 3600 ≤ FC ≤ 3650 | ≤6.12 MHz |  | <1.44 MHz | A8 | >[24.3] MHz | [4.5] |
|  |  |  | ≥ 32.76 |  |  |  |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

# References

[1]		R4-1909607, n48 A-MPR Simplification and add missing AMPR, Qualcomm Inc., 3GPP TSG-RAN WG4 Meeting #92.